3GPP TSG-SA WG2 Meeting #132	S2-1904669

# Xi’an (China), 8 April – 12 April 2019												rev of S2-1903210

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1073 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 – Support of emergency services in public network integrated NPNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, (?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 11/04/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An Editor's Note exist on whether emergency services is supported in a CAG cell Editor's note: It is FFS whether Emergency services are to be supported in a CAG cell. This CR aims to resolve that Editor's note by proposing what the UE supporting CAG and camped on a CAG cell need to consider when attempting to obtain eergency services |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is indicated the cases that emergency services is possible for a CAG capable UE in a CAG cell of a public network integrated NPNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency services will remain unsupported in CAG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.1, 5.30.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

### 5.30.3	Public network integrated NPN

#### 5.30.3.1	General

Public network integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. The existing network slicing functionalities apply as described in clause 5.15. When an NPN is made available via a PLMN, then the UE has a subscription for the PLMN.

NOTE:	Annex D provides additional consideration to consider when supporting Non-Public Network as a Network Slice of a PLMN.

As network slicing does not enable the possibility to prevent UEs from trying to access the network in areas which the UE is not allowed to use the Network Slice allocated for the NPN, Closed Access Groups may optionally be used in addition to network slicing to apply access control.

A Closed Access Group identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG.

CAG is used for the Public network integrated NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated cell(s).

The following clauses describes the functionality needed for supporting CAGs.

*** Next change ***

#### 5.30.3.X	Support of emergency services in CAG cells

Option 1

For UEs supporting CAG that is camping on a CAG cell, the following requirements are considered to obtain emergency services:

-	If the UE is RM-REGISTERED, and had been informed at Registration procedure that emergency services are supported, then UE initiates a PDU Session for Emergency services as specified in 3GPP TS 23.502 [3] for normal emergency services;

-	If the UE is RM-REGISTERED, and had been informed at Registration procedure that emergency services are not supported, but the CAG cell the UE is on indicates that emergency services in limited service is supported – see 3GPP TS 38.331 [28], then the UE moves to CM-IDLE and initiates an emergency registration to obtain emergency services without indicating any CAG Identifier; or

NOTE:	In this release of specification, NW sharing is not considered.

-	If the UE is RM-REGISTERED, and had been informed at Registration procedure that emergency services arenot supported, and the CAG cell the UE is on indicates that emergency services in limited service is not supported – see 3GPP TS 38.331 [28], then if the UE needs to obtain emergency services, the UE moves to CM-IDLE and search for a cell to obtain emergency services as specified in 3GPP TS 38.304 [50].

Option 2

For UEs not RM-REGISTERED but able to access the CAG cell as an acceptable cell, as defined in TS 38.304 [50], needing emergency service, the UE performs emergency calls in limited service state based on procedures defined in clause 5.16.4.

During handover to CAG cell, if the UE is not authorized to access the target CAG cell and has emergency services, the target NG-RAN node only accepts the emergency PDU sessions and the target AMF releases the non-emergency PDU connections that were not accepted by the NG-RAN node as specified in subclause 5.16.4.3 and 5.16.4.9a. UEs considers itself as emergency registered.

*** End of change ***