3GPP TSG-SA2 Meeting #132	S2-1904672

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1271 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service-mesh-based SCP deployment option |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG, DoCoMo, InterDigital, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019.04.02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Provides a deployment option of the SCP based on service mesh for clarification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative Annex with describing a SCP deployment option based on a service mesh Rev 1: - Initial mention that this section is a deployment example - Clarified and simplified Figure X.1-1 - Made Service Agent capitalization consistent - Clarified and simplified Figure X.1-2 and its related description (numbering made it less clear) - Clarified and simplified Figure X.2- and its related description (numbering made it less clear) - Added clarification to Figure X.3-1 so that it is immediately clear to the reader that the division left is implementation-specific |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 spec may be misinterpreted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************** Change (all new) **************

Annex X (informative):
SCP Deployment Example

## X.1	Distributed deployment based on Service Mesh

This section describes a deployment example for the SCP based on a distributed model in which SCP endpoints are co-located with 5GC functionality (e.g. an NF, an NF Service, a subset thereof such as a microservice implementing part of an NF/NF service or a superset thereof such as a group of NFs, NF Services or microservices).

In this deployment example, when so required, Service Agent(s) implementing necessary peripheral tasks (e.g. an SCP endpoint) can be co-located with 5GC functionality, as depicted in Figure X.1-1.

Figure X.1-1: Deployment unit: 5GC functionality and co-located Service Agent(s) implementing peripheral tasks

If deployed as a service mesh, an SCP Service Agent, i.e. a service communication proxy, is co-located with 5GC functionality and provides each deployed unit with indirect communication and delegated discovery.

Figure X.1-2 shows an overview of the deployment scenario where an SCP is deployed as a service mesh. For SBI-based interactions with other 5GC functionalites, a consumer (5GC functionality A) communicates through its Service Agent via SBI. Its Service Agent selects a target producer based on the request and routes the request to the producer’s (5GC functionality B) Service Agent. What routing and selection policies a Service Agent applies for a given request is determined by routing and selection policies pushed by the service mesh controller. Information required by the service mesh controller may be pushed by the Service Agents to the service mesh controller.

Deployments can choose whether the SCP manages registration and discovery for communication within the service mesh and/or whether it interacts with an external NRF for service exposure and communication across service mesh boundaries. Operator-defined policies may be additionally employed to generate the routing and selection policies to be used by the Service Agents.

Figure X.1-2: SCP Service mesh co-location with 5GC functionality

## X.2	Communication across service mesh boundaries

It is a deployment aspect whether a single service mesh should cover all functionality within a given deployment or not. Furthermore, in cases of communication across the boundaries of a service mesh, the service mesh routing the outbound message may neither know whether the selected producer is in a service mesh nor the internal topology of the potential service mesh where the producer resides.

In such a deployment, as shown in Figure X.1-3, after producer selection is performed, routing policies on the ougoing service mesh need only be aware of the next hop.

Given a request sent by A, A’s Service Agent will perform producer selection based on the received request. If the selected producer endpoint is determined to be outside of Service Mesh 1, A’s Service Agent will route the request to the Egress Proxy. For a successful routing, the Egress Proxy must be able to determine the next hop of the request. In this case, this need be the Ingress Proxy of Service Mesh 2. The Ingress Proxy of Service Mesh 2 is, based on the information in the received request and its routing policies, able to determine the route for the request. Subsequently, D receives the request. No topology information need be exchanged between Service Mesh 1 and Service Mesh 2 besides a general routing rule towards Service Mesh 2 (e.g. a FQDN prefix) and an Ingress Proxy destination for requests targeting endpoints in Service Mesh 2.

Figure X.2-1: Message routing across service mesh boundaries

## X.3	SCP Functionality

Depending on the set of SCP functionality deployed by the operator, the Service Agent may provide indirect communication with or without delegated discovery (Figure X.1-4). Discovery/selection and routing can be provided by one Service Agent or by separate Service Agents each respectively applying discovery/selection policies and routing policies.

Figure X.3-1: Service Agent deployment examples: with and without Delegated Discovery.

************** End changes **************