3GPP TSG-SA2 Meeting #132	S2-1903885

# Xi'an, P.R. China, 08-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1303 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying RAI handling and Inter-UE QoS diffentiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.502 clause contains the following Editor's note: Editor's note: How to handle the case of RAI with single UL/single DL packet only is FFS. FS_CIoT_5G study item concluded with the agreement to take Solution 52 for NB-IoT inter-UE QoS differentiation of NB-IoT as the basis for normative work. Which parameter to use was left for normative work. This CR proposes to use the newly defined NB-IoT UE Priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove Note 1 from clause 4.24.2, which appears to have mistakenly been copied from TR 23.724 and simplifies clause 4.24.2 by referring to clause 4.24.1 for further uplink data transmission. Clarify that for MT Data Transfer, the RAN may, between RRC message 3 and RRC message 4, retrieve the NB-IoT UE priority previously stored in the AMF and use it to prioritise NB-IoT UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.24.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes >>>

### 4.24.2	UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation

This clause describes the procedures for Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation where the PDU Session is terminated at a UPF.

Figure 4.24.2-1: Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation

1.	The UPF forwards a downlink data packet to the SMF. In the home-routed roaming case, the H-SMF forwards the data to the V-SMF.

Editor's note:	What service operation to use for data forwarding is pending CT WG4 discussion.

2.	The (V-)SMF compresses the header if header compression applies to the PDU session and encapsulates the downlink data as payload in a NAS message. The (V-)SMF forwards the NAS message and the PDU session ID to the AMF using the Namf_Communication_N1N2MessageTransfer service operation.

Editor's note:	Header compression and buffering for home routed roaming scenario is FFS.

3.	[Conditional] If the UE is in CM Idle, the AMF sends a paging message to NG-RAN.

4.	[Conditional] If NG-RAN received a paging message from AMF, NG-RAN perform paging.

5.	[Conditional] If the UE receives paging message, it responds with a NAS message sent over RRCConnectionRequest.

5a.	[Conditional] In the NB-IoT case, at the same time of Step 5, the NG-RAN, based on configuration, may retrieve the NB-IoT UE Priority and the Expected UE Behaviour Parameters from the AMF, if not previously retrieved. Based on such parameters, the NG-RAN may apply prioritisation between requests from different UEs before triggering step 6 and throughout the RRC connection. The NG-RAN may retrieve additional parameters (e.g., UE Radio Capabilities).

6.	[Conditional] The NAS message is forwarded to the AMF.

7.	The AMF creates a DL NAS transport message with the PDU session ID and the NAS message received from the SMF. The AMF ciphers and integrity protects the NAS transport message.

8.	The AMF sends the DL NAS transport message to NG-RAN.

9.	NG-RAN delivers the NAS payload over RRC to the UE.

10.	While the RRC connection is established, further uplink and downlink data can be exchanged. In order to send uplink data, the procedure continues as per steps 1-10 of the UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation procedure (clause 4.24.1).

11.	[Conditional] If no further activity is detected by NG-RAN, then NG-RAN triggers the AN release procedure.

12.	[Conditional] The UE's logical NG-AP signaling connection and RRC signaling connection are released according to clause 4.2.6.

NOTE 1:	The details of the NGAP messages to be used for this procedure are specified in TS 38.413 [10]..

## <<< End of changes >>>