3GPP TSG-SA WG2 Meeting #133	S2-1904988

Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1129 | rev | 4 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF Delegated Discovery and Selection for HR scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Oracle Corporation, ZTE, Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-03-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) The following EN in 23.501 clause 6.3.2 needs to be replaced by normative text. The normative text shall reflect the conclusions of the eSBA study for KI #3: “Editor's note: It is FFS how the SCP handles the selection of the vSMF and hSMF in the HR roaming case and it is also FFS how the vSMF is informed of the selected hSMF in the case of delegated discovery and HR case.” 2) The I-SMF selection does not cover yet the delegated discovery. 3) The following EN needs to be resolved: Editor's note: It is FFS how to capture that required discovery and selection information should be conveyed to the SCP with as minimal impact on the end-to-end signalling performance latency between AMF and SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) The EN in 23.501 clause 6.3.2 is resolved by proposing that the SCP discovers and selects the V-SMF upon receipt of the PDU session establishment request from the AMF, and then discovers and selects the H-SMF upon receipt of the PDU session establishment request from the V-SMF. 2) Clause 5.34.3 “I-SMF selection” is updated to cover the delegated I-SMF discovery, using a similar procedure as V-SMF discovery in 6.3.2 for HR roaming. 3) Remove the EN related to latency; the possible impacts to latency should be taken into account in Stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs in 23.501 clause 6.3.2 remains and the next version of 23.501 v16.0 will be missing procedures to handle V-SMF and I-SMF delegated discovery and selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 5.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 6.3.2	SMF discovery and selection

The SMF selection functionality is supported by the AMF and SCP and is used to allocate an SMF that shall manage the PDU Session. The SMF selection procedures are described in clause 4.3.2.2.3 of TS 23.502 [3].

The SMF discovery and selection functionality follows the principles stated in clause 6.3.1.

If the AMF does discovery, the AMF shall utilize the NRF to discover SMF instance(s) unless SMF information is available by other means, e.g. locally configured on AMF. The AMF provides UE location information to the NRF when trying to discover SMF instance(s). The NRF provides NF profile(s) of SMF instance(s) to the AMF. In addition, the NRF also provides the SMF service area of SMF instance(s) to the AMF. The SMF selection functionality in the AMF selects an SMF instance and an SMF service instance based on the available SMF instances obtained from NRF or on the configured SMF information in the AMF.

NOTE 1:	Protocol aspects of the access to NRF are specified in TS 29.510 [58].

The SMF selection functionality is applicable to both 3GPP access and non-3GPP access.

The SMF selection for Emergency services is described in clause 5.16.4.5.

The following factors may be considered during the SMF selection:

a)	Selected Data Network Name (DNN).

b)	S-NSSAI.

c)	NSI-ID.

NOTE 2:	The use of NSI -ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI.

d)	Access technology being used by the UE.

e)	Support for Control Plane CIoT 5GS Optimisation.

f)	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed.

-	per S-NSSAI: the subscribed DNN(s).

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed.

g)	per (S-NSSAI, subscribed DNN): whether EPC interworking is supported.

h)	Local operator policies.

i)	Load conditions of the candidate SMFs.

j)	Analytics (i.e. statistics or predictions) for candidate SMFs' load as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

k)	UE location (i.e. TA).

l)	Service Area of the candidate SMFs.

m)	MA-PDU Capability of the SMF.

n) Whether the SMF supports delegated discovery

In the case of delegated discovery, the AMF, shall send all the available factors a)-d) and k) to the SCP and shall also indicate if interworking with EPS is required and if the PDU session shall be home routed, i.e. the target PLMN ID of the H-SMF.

In addition, the AMF may indicate to the SCP which NRF to use (in case of NRF dedicated to the target slice). The AMF the receives the Service Area of the SMF instance in the response from the selected SMF.

If there is an existing PDU Session and the UE requests to establish another PDU Session to the same DNN and S-NSSAI and the UE subscription data indicates the support for interworking with EPS for this DNN and S-NSSAI, the same SMF shall be selected. In any such a case where the AMF can determine which SMF should be selected, if delegated discovery is used, the AMF shall indicate a desired NF Instance ID so that the SCP is able to route the message to the relevant SMF. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different SMF may be selected. For example, to support a SMF load balancing or to support a graceful SMF shutdown (e.g., a SMF starts to no more take new PDU Sessions).

In the home-routed roaming case, the SMF selection functionality selects an SMF in VPLMN as well as an SMF in HPLMN. In the context of Network Slicing this is specified in clause 4.3.2.2.3.3 of TS 23.502 [3].

When the UE requests to establish a PDU Session to a DNN and an S-NSSAI, if the UE MM Core Network Capability indicates the UE supports EPC NAS and optionally, if the UE subscription indicates the support for interworking with EPS for this DNN and S-NSSAI, the selection functionality (in AMF or SCP) selects a combined SMF+PGW-C. Otherwise, a standalone SMF may be selected.

If the UDM provides a subscription context that allows for handling the PDU Session in the visited PLMN (i.e. using LBO) for this DNN and S-NSSAI and, optionally, the AMF is configured to know that the visited VPLMN has a suitable roaming agreement with the HPLMN of the UE, the following applies:

-	If the AMF does discovery, the SMF selection functionality in AMF selects an SMF from the visited PLMN.

-	If delegated discovery is used, the AMF indicate to the SCP that home routing is not needed, the delegated discovery and selection functionality in SCP shall discover and select an SMF from the visited PLMN.

If an SMF in VPLMN cannot be derived for the DNN and S-NSSAI, or if the subscription does not allow for handling the PDU Session in visited PLMN using LBO, then the following applies:

-	If the AMF does discovery, both an SMF in VPLMN and an SMF in HPLMN are selected, and the DNN and S-NSSAI is used to derive an SMF identifier from the HPLMN.

In the HR roaming case, when delegated discovery is used, the AMF shall provide in the Nsmf_PDUSession_CreateSMContext Request it sends to the SCP the HPLMN ID, i.e. target PLMN ID for the SMF. The AMF includes the S-NSSAI, DNN and UE location as discovery and selection parameters.

Upon receipt of the request from the AMF, the SCP shall perform discovery and selection of the SMF in the serving PLMN, optionally via the V-NRF. The SCP shall then forward the request to the V-SMF. The response from V-SMF to AMF includes the Service Area of the V-SMF. The AMF uses the SMF Service Area of the V-SMF to decide V-SMF relocation for subsequent UE mobility. Based on the indication of target PLMN ID of the H-SMF, the V-SMF sends an Nsmf_PDUSession_Create Request towards the HPLMN. When the V-SMF sends the request towards the HPLMN via the SCP, it shall include the HPLMN ID as a discovery and selection parameter. The SCP shall then perform discovery and selection of the H-SMF via the V-NRF. In this case, the SCP shall provide the HPLMN ID to the V-NRF.

NOTE 1: In the above HR roaming procedure, the H-SMF address need not be supplied by the AMF, thus delegated discovery is used for V-SMF and H-SMF discovery and selection by the concerned AMFs and V-SMFs. To enforce selection of a delegated discovery-capable V-SMF by the SCP, factor n) can be used.

If the initially selected SMF in VPLMN (for roaming with LBO) detects it does not understand information in the UE request, it may reject the N11 message (related with a PDU Session Establishment Request message) with a proper N11 cause triggering the AMF to select both a new SMF in the VPLMN and a SMF in the HPLMN (for home routed roaming).

The AMF selects SMF(s) considering support for CIoT 5GS optimisations (e.g. Control Plane CIoT 5GS Optimisation).

Additional details of AMF selection of an I-SMF are described in the clause 5.34.

In the case of home routed scenario, the AMF selects a new V-SMF if it determines that the current V-SMF cannot serve the UE location. The selection/relocation is same as an I-SMF selection/relocation as described in the clause 5.34.

* * * * 2nd Change * * * *

### 5.34.3	I-SMF selection

The AMF is responsible of detecting when to add or to remove an I-SMF for a PDU Session. For this purpose, when delegated discovery is not used, the AMF gets from NRF information about the Service Area of SMF(s).

When delegated discovery is used, the AMF gets the information about the Service Area of the selected I-SMF in a response to the Nsmf_PDUSession_CreateSMContext Request, and the Service Area of the selected SMF in a PDU Session status change notification from the I-SMF.

During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI. Conversely if the AMF detects that an I-SMF is no more needed it removes the I-SMF and interfaces directly with the SMF of the PDU Session. If the AMF detects that the SMF cannot serve the UE location (e.g. due to mobility), then the AMF initiates an insertion of a new I-SMF serving the UE location. In case of existing I-SMF cannot serve the UE location, then the AMF initiates a I-SMF relocation.

At PDU Session Establishment in non-roaming and roaming with LBO scenarios, if the AMF or SCP cannot select an SMF with a Service Area supporting the current UE location for the selected DNN, S-NSSAI etc, the AMF or SCP selects an I-SMF serving the UE location. If delegated discovery is not used, the AMF in addition selects an SMF for the selected DNN and S-NSSAI.

When Delegated Discovery is used, the SCP performs I-SMF discovery and selection based on the discovery and selection parameters provided by AMF. When PDU Session in non-roaming and roaming with LBO scenarios is established:

-	The AMF shall send Nsmf_PDUSessionCreateSMContext Request to SCP and include the parameters as defined in clause 6.3.2 (e.g. the S-NSSAI, DNN, UE location) as discovery and selection parameters. If the SCP successfully selects an SMF matching all discovery and selection parameters, the SCP forwards the Nsmf_PDUSessionCreateSMContext Request to the selected SMF.

-	If the SCP cannot select an SMF matching all discovery and selection parameters, the SCP may select an I-SMF matching with the the selection parameters except the DNN. The SCP shall then forward the request to the I-SMF. The I-SMF returns its SMF Service Area in the Nsmf_PDUSessionCreateSMContext Response message to AMF via SCP. The AMF uses the SMF Service Area of the I-SMF to decide I-SMF relocation for subsequent UE mobility.

-	The I-SMF decides it cannot serve the requested S-NSSAI and DNN then it decides to select an SMF. The I-SMF sends the Nsmf_PDUSession_Create Request towards the SMF via the SCP, and shall include include the parameters as defned in clause 6.3.2 except UE location as discovery and selection parameters.  The SCP shall then perform discovery and selection of the SMF.

-	If the SCP cannot select an SMF, the SCP reject the request with cause value indicting the delegated discovery fails. Based on the reject cause value the AMF may send Nsmf_PDUSessionCreateSMContext Request message to SCP, with the discovery and selection parameters as defined for I-SMF selection in no Delegated Discovery case.

NOTE 1: In the above procedure, the SMF address need not be supplied by the AMF, thus delegated discovery is used for I-SMF and SMF discovery and selection by the concerned AMFs and I-SMFs.

During the lifetime of a PDU Session, the AMF initiates an insertion or relocation of a new I-SMF if it determines that neither the current SMF nor the current I-SMF (if any) can serve the UE location.

Comparing to the SMF selection function defined in clause 6.3.2, following parameter are not applicable:

-	Data Network Name (DNN).

-	Subscription information from UDM parameters.

* * * * End of Changes * * * *