3GPP TSG-SA WG2 Meeting #133	S2-1905053

Reno, NV, USA, May 13 - 17, 2019	(revision of S2-1904319)

Source:	Ericsson

Title:	UDM information storage

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This pCR propose to define Privacy profile and MO-LR subscription data stored in UDM.

.

# 1. Discussion

In clause 5.4 the handling of “privacy profiles” is being defined. The privacy data is defined to be stored in UDM/UDR. From 23.271 it is found beneficial to have a normative definition of privacy data and structure that is stored in UDM/UDR.

Further the MO-LR procedure includes checking of whether the subscription profile allows the specific MO-LR service.

Thus, it proposed to include in clause 7.1 the data stored in UDM related to privacy profile and MO-LR.

This pCR is based on S2-1904314 and email discussions prior to SA2 #133.

# 2. Proposal

It is proposed to include below text in TS 23.273

***** BEGIN 1st CHANGE *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	3GPP TS 23.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	OMA RLP TS: "Roaming Location Protocol", [http://www.openmobilealliance.org].

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	ETSI ES 282 004: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture; Network Attachment Sub-System (NASS)".

[23]	ITU Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[r2]	RFC 2396: "Uniform Resource Identifiers".

[r3]	RFC 3261: "SIP: Session Initiation Protocol".

[r4]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[r5]	3GPP TS 23.003: “Numbering, addressing and identification”.

***** NEXT CHANGE *****

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy class and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Override Class | M O O | Indication of one of the following mutually exclusive global settings: - Location is overridden - Immediate location is not overridden - Immediate and deferred location are not overridden (default) Indication for LCS clients not identified in the Location Override Class: - Positioning not allowed - Start time and duration of positioning not allowed A list of zero or more LCS clients, AFs, LCS client groups with the following indication for each entry: - Positioning not allowed - Start time and duration of positioning not allowed |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [4]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE |

Editor's Note: Table 7.1-1 is based on definitions in clause 5.4 and will need to be updated as clause 5.4 evolves.

The Mobile Originating data is defined in table 7.1-1 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location) - Autonomous Self Location (UE can receive location assistance data) - Transfer to Third Party |

***** NEXT CHANGE *****

## 7.2	GMLC

### 7.2.1	Information for an LCS Client

The GMLC holds information for external LCS clients which are permitted to request location information for UE subscribers. Table 7.2.1-1 shows the information which may be stored in the GMLC for an external LCS Client.

Table 7.2.1-1: GMLC Information for an External LCS Client

| LCS Client Information | Status | Description |
| --- | --- | --- |
| LCS Client Type | M | Identifies the type of LCS client from among the following: - Emergency Services - Value Added Services - PLMN Operator Services - Lawful Intercept Services |
| External identity | O | A list of one or more identifiers used to identify an external LCS client. The identity may be used for a 5GC-MT-LR and/or 5GC-MO-LR. The format of the identity is an international E.164 address [23]. |
| Authentication data | M | Data employed to authenticate the identity of an external LCS client – details are outside the scope of the present document |
| Internal identity | O | Identifies the sub-type of a PLMN operator services LCS Client from among the following: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE |
| Client name | O | An address string which is associated with the LCS client's external identity (i.e., E.164 address). |
| Client name type | O | Indication of the type of the LCS client name. The type of the LCS client name can be one of the following: - Logical name - MSISDN - E-mail address (RFC 2396 [r2]) - URL (RFC 2396 [r2]) - SIP URL (RFC 3261 [r3]) - IMS public identity (TS 23.228 [r4]) - GPSI |
| Override capability | O | Indication of whether the LCS client possesses the POI capability (only applicable to lawful intercept and emergency services clients) |
| Authorized UE List | O | A list of SUPIs and/or groups of SUPI for which the LCS client may issue a request for a 5GC-MT-LR for immediate or deferred location. |
| Priority | O | The priority of the LCS client |
| QoS parameters | M | The default QoS requirements for the LCS client, comprising: - Accuracy - Response time - LCS QoS Class |
| Service Coverage | O | A list of E.164 country codes for geographic areas [23] where the LCS client is permitted to request and receive UE location information. |
| Allowed LCS Request Types | M | Indicates which of the following are allowed: - Request of current immediate location - Request of current or last known immediate location - Request of deferred location for the UE available event - Request of deferred location for UE periodic events - Request of deferred location for the Area Event - Request of deferred location for the Motion Event |
| Local Co-ordinate System | O | Definition of the co-ordinate system(s) in which a location estimate shall be provided – details are outside the scope of the present document |
| Access Barring List(s) | O | List(s) of SUPIs or groups of SUPI for which a location request is barred |
| Service Identities | O | List of service identities allowed for the LCS client. |
| Maximum Target UE Number | O | The maximum number of the Target UEs in one LCS request. For a specific LCS Client, this parameter may have different values for different service identities. |

***** NEXT CHANGE *****

Annex X (informative):
Differences with TS 23.271

X.a		Differences in Information Storage in the UDR/UDM versus HSS/GMLC for EPS

Table X.a-1 shows a list of LCS information storage items for a UE in the UDR and UDM defined in this TS and for the HSS and/or GMLC defined in TS 23.271 [4], differences if any between this TS and TS 23.271, and comments on these including any significant consequences.

Table X.a-1: Differences in Information Storage for a UE in the UDR/UDM versus HSS/GMLC for EPS

| Information Storage Item(s) | Difference | Comments |
| --- | --- | --- |
| UE Privacy Universal Class | Not supported in this TS | This privacy class should not be supported for 5GC capable UEs on the EPS side to avoid differences in LCS privacy support between EPS access and 5GS access for the same UE. (Note 1) |
| UE Privacy Location Override Class | Not supported in TS 23.271 | This privacy class can create differences between LCS client access to a UE which has 5GS access versus EPS access. (Note 1) |
| UE Privacy Call/Session related Class | Not supported in this TS | This privacy class should not be supported for 5GC capable UEs on the EPS side to avoid differences in LCS privacy support between EPS access and 5GS access for the same UE. (Note 1) |
| UE Privacy Call/Session unrelated Class | Supported as defined in TS 23.271 with the following differences: - Information related to an R-GMLC is not included - An optional time period is not supported in TS 23.271 - An optional geographic area is not supported in TS 23.271 | This privacy class can create differences between LCS client access to a UE which has 5GS access versus EPS access. (Note 1) |
| UE Privacy PLMN Operator Class | None |  |
| UE LCS Mobile Originating Data | None |  |

Note 1:	A PLMN operator can avoid any difference in UE privacy support for EPS access versus 5GS access by only storing information for UE privacy (in the UE LCS privacy profile) in a UDR (and UDM) and not in an HSS or GMLC.

X.b	Differences in Information Storage in the GMLC

Table X.b-1 shows a list of information storage items in the GMLC for an LCS Client defined in this TS and in TS 23.271 [4], differences if any between this TS and TS 23.271, and comments on these including any significant consequences.

Table X.b-1: Differences in Information Storage for an LCS Client in the GMLC

| Information Storage Item(s) | Difference | Comments |
| --- | --- | --- |
| LCS Client Type | None |  |
| LCS Client Type | None |  |
| External identity | None |  |
| Authentication data | None |  |
| Call/session related identity | Not supported in this TS | This information item can create differences between LCS client access to a UE which has 5GS access versus EPS access. (Note 1) |
| Internal identity | None |  |
| Client name | None |  |
| Client name type | No difference except that this TS adds a Client Name type in the form of a GPSI. | A GPSI can be an MSISDN or External Identifier in the form of “username@realm” as defined in TS 23.003 [r5]. TS 23.271 allows an MSISDN but the External Identifier is not included. |
| Override capability | None |  |
| Authorized UE List | None |  |
| Priority | None |  |
| QoS parameters | None |  |
| Service Coverage | None |  |
| Allowed LCS + Request Types | TS 23.271 includes the following items which are not in this TS: - Non-call related CS-MT-LR/PS-MT-LR/EPC-MT-LR - Call/session related CS-MT-LR/PS-MT-LR/EPC-MT-LR - Specification or negotiation of priority - Specification or negotiation of QoS parameters - Specification or negotiation of Service Coverage parameter This TS includes the following items which are not in TS 23.271: - Request of deferred location for the UE available event - Request of deferred location for UE periodic events - Request of deferred location for the Area Event - Request of deferred location for the Motion Event | This TS should take precedence (Note 1) |
| Local Co-ordinate System | None |  |
| Access Barring List(s) | TS 23.271 includes a list of MSISDNs. This TS includes a list of SUPIs | This TS can take precedence (Note 1) |
| Service Identities | None |  |
| Maximum Target UE Number | None |  |

Note 1:	A PLMN operator can avoid any difference in LCS Client support for a target UE with EPS access versus 5GS access by only storing information for an LCS Client in a GMLC as defined in this TS.

***** End of CHANGE *****