3GPP TSG SA WG2 Meeting #133	S2-1905065

13th – 17th May 2019, Reno, Nevada, USA

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1328 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Serving 4G only UEs by SMF+PGW-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-04-30 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF+PGW-C is allowed to be selected for a 4G only UE whose subscription has been updated to enalbe access to 5GS (SMF+PGW-C, PCF, CHF) via MME/SGW/HSS. This was the motivation for the following note in 23.502: #### 4.11.0a.4 PGW Selection When the UE requests to establish a PDN connection to an APN, the MME may use the UE's support for 5GC NAS indication included in the UE Network Capability and/or UE's subscription from HSS that includes UE's mobility restriction paramters related to 5GS and/or indication of support for interworking with 5GS for this APN to determine if PGW-C+SMF or a standalone PGW-C should be selected. NOTE: If restriction for Core Network Type indicates that the UE can access to 5GC, it implies that the UE has 5G subscription data. The note is cryptic but the idea was that if the UE's subscription has been updated, the MME may select SMF+PGW-C even for a UE that is 4G-only. This is also useful both in the early stages of 5GC deployment when there may not be too many 5G Stand-alone UEs available and also when the operator wants to migrate all UEs (4G-only and 4G+5G) to the new 5GS core. PGW/SMF+PGW-C Selection procedure is updated to clearly state that SMF+PGW-C maybe selected for a 4G only UE. Other aspects of serving 4G only UE by SMF+PGW-C are captured in CR to 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing that MME may select SMF+PGW-C for a 4G only UE whose subscription has been updated to support 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMF+PGW-C not able to support 4G only UEs. This will hinder migration plans of operators and/or require support of Gx and diameter charging interface from SMF+PGW-C which is not specified in 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.1, 4.11.0a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.501 ... CR 1387, 1388 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.11.0a	Impacts to EPS Procedures

#### 4.11.0a.1	General

This clause captures changes to procedures in TS 23.401 [13] that are common to interworking based on N26 and interworking without N26.

In section 4.11, UEs are assumed to support both 5GC and EPC NAS unless explicitly stated otherwise.

* * * Next Change * * * *

#### 4.11.0a.4	PGW Selection

When the UE requests to establish a PDN connection to an APN, the MME may use the UE's support for 5GC NAS indication included in the UE Network Capability and/or UE's subscription from HSS that includes UE's mobility restriction paramters related to 5GS and/or indication of support for interworking with 5GS for this APN to determine if PGW-C+SMF or a standalone PGW-C should be selected. An MME may select a PGW-C+SMF for a UE which is not 5GC NAS capable but whose subscription has been upgraded for 5GS (eg. subscription data is in HSS+UDM).

NOTE:	If restriction for Core Network Type indicates that the UE can access to 5GC, it implies that the UE has 5G subscription data.

* * * End Of Change * * * *