3GPP TSG-SA WG2 Meeting #133	S2-1905748

13th – 17th May, 2019, Reno, NV, US

Source:	Samsung

Title:	Update on 6.3 Slice load level analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA/ Rel-16

Abstract of the contribution: This paper proposes changes for clarification of the analytics ID and the filter which are used for fetching slice load level related analytics from the NWDAF.

1. Introduction

This contribution proposes changes for clarification of an analytics ID and filters which are used for fetching slice load level analytics from the NWDAF. Current text in clause 6.3.3 has mixed descriptions for analytics ID and analytics filters and cause ambiguity.

2. Proposal

It is proposed to adopt the following text changes in TS 23.288.

**** Start of 1st Changes ****

6.3	Slice load level related network data analytics

6.3.1	General

The NWDAF provides load level information to an NF on a network slice instance level. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the NFs that are subscribed to it. An NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

6.3.2	Data collection

There is no definition for information for support of load level analytics in this release of the specification.

6.3.3	Analytics exposure

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose load level analytics from the NWDAF to the consumer NF (e.g. PCF or NSSF).

Following Analytics ID isused to request and deliver the slice load level analytics:

-	Load level information

Following Analytics Filters can be included in the related Subscribe or Request messages:

-	Network Slice Instance.

-	Load Level Threshold value: the NWDAF reports when the load level crosses the threshold provided in the analytics subscription; if no threshold is provided in the subscription, the reporting (Notify operation) is assumed to be periodic.

**** End of 1st Changes ****