3GPP TSG-SA WG2 Meeting #133	S2-1906159

13 - 17 May 2019, USA, Reno	rev. of 6107

Source:	ORANGE, Huawei

Title:	UE Mobility Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes a additional clauses on UE mobility analytics for UE groups, limitation of results, and aggregation of results per TA or cell

# Discussion

This discussion addresses several the following topics:

- Addition of general input parameters in the §General

- Mobility analytics for UE groups.

It is difficult to provide detailed per-UE analytics results in the case of a large group (on the contrary it is easy to provide detailed analytics per-UE for a small group by applying multiple analytics requests, one per each UE). As a consequence, several modifications are introduced in order to facilitate cumulative values, with the idea that UE-group analytics shall be requested only for large groups.

As a consequence to the introduction of analytics for UE groups, is has been necessary to redefine the structure of results (with no impact in the case of a single UE).

- Limitation of results

For large UE groups or for UEs moving on large areas, it may be necessary to limit the size of results. This can be done by requesting results on a dedicated area

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.10	UE mobility analytics

### 6.10.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from 5GC, OAM and/or AFs, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytic Filter Information containing optional list of TA or Cells, optional maximum number of results

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high)

### 6.10.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.10.2-1;

Table 6.10.2-1: UE location information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.10.2-2;

Table 6.10.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

### 6.10.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics, UE mobility predictions:

-	UE mobility statistics is defined in the Table 6.10.3-1.

Table 6.10.3-1: UE mobility statistics

| Information |  | Description |
| --- | --- | --- |
| UE group ID or UE ID |  | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) |  | List of time slots during the Observation period |
| > Time slot start | Time slot start within the Observation period |
| > Duration | Duration of the time slot (average and variance) |
| > UE location (1..max) | Observed location statistics |
| >>UE location |  | TA or cells which the UE or UE group stays |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

-	UE mobility predictions is defined in the Table 6.10.3-2.

Table 6.10.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Observation period |
| >Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Observation period |
| >> UE location | TA or cells where the UE or UE group may move into |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location  at a given time.

The number of time slots and UE locations is limited by the maximum number of results provided as input parameter.

The time slots shall be provided by order of time, possibly overlapping . The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less then 100%. Depending on the list size limitation, the least probable locations on a given observation period may not be provided.

* * * End of changes * * * *