SA WG2 Meeting #133 	S2-1906274

# May 13 – 17, 2019 ; Reno, Nevada, USA	                (revision of S2-1906242)

Source:	Nokia, Nokia Shanghai Bell

Title:	Update to 5G-RG Triggered Service Request procedure via W-5GAN

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5WWC / Rel-16

Abstract of the contribution: Update to 5G-RG Triggered Service Request procedure via W-5GAN

# Introduction

In this release of the specification NO selected PLMN sent;

# Proposal

It is proposed to update 23.316 as follows:

FIRST CHANGE

#### 7.2.2.1	5G-RG Service Request procedure via W-5GAN Access

The Service Request procedure via W-5GAN shall be used by a 5G-RG in CM-IDLE state over W-5GAN to request the re-establishment of the NAS signalling connection and the re-establishment of the user plane for all or some of the PDU Sessions which are associated to non-3GPP access.

NOTE 1:	for a W-5GAN access, the Service Request procedure is never a response to a Paging.

The Service Request procedure via W-5GAN shall be used by a 5G-RG in CM-CONNECTED state over wireline access to request the re-establishment of the user plane for one or more PDU Sessions which are associated to non-3GPP access.

Figure 7.2.2-1: UE Triggered Service Request procedure via W-5GAN

Editor's note:	EAP-5G is assumed to be used during authentication but need to be verified with BBF/CableLabs.

1.	The 5G-RG connects to a W-5GAN with procedures outside the scope of 3GPP and creates an initial not authenticated W-CP EAP connection. This connection shall support EAP messages transfer between 5G-RG and W-AGF.

2.	The W-AGF sends an EAP-Request/5G-Start packet over the W-CP connection. The EAP-Request/5G-Start packet informs the 5G-RG to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets.

3.	The 5G-RG sends an EAP-Response/5G-NAS packet that contains the Access Network parameters (5G-S-TMSI and Establishment Cause) and a NAS Service Request message (List Of PDU Sessions To Be Activated, security parameters, PDU Session status, Uplink Data Status, 5G-S-TMSI). The Establishment cause provides the reason for requesting a signalling connection with 5GC. In this release of the specification no Selected PLMN parameter is sent by a 5G RG.

4.	The W-AGF shall then forward the Service Request received from the 5G-RG to the selected AMF within an N2 initial UE message (NAS Service Request message, Line-id based ULI, Establishment cause, UE context request).

5.	If the Service Request was not sent integrity protected or integrity protection verification failed, the AMF shall initiate NAS authentication/security procedure as defined in step 6 and step 7 in clause 7.2.1.1.

If the UE in CM-IDLE state triggered the Service Request to establish a signalling connection only, after successful establishment of the signalling connection the UE and the network can exchange NAS signalling and steps 6 and 14 are skipped.

6.	Steps 4-11 in TS 23.502 [3] figure 4.2.3.2-1 are performed for each requested PDU session user plane.

7.	(if the 5G RG was CM-IDLE) AMF sends an N2 Initial Context Setup Request message (N2 SM information received from SMF(s), UE Aggregate MBR, UE Total MBR, GUAMI, Allowed NSSAI, UE security capability, Security Key, Trace Activation, Masked IMEISV).

If the 5G RG was CM-CONNECTED the AMF sends N2 SM information received from SMF(s),

8.	(if the 5G RG was CM-IDLE) This triggers the W-AGF to send an EAP-Success to 5G-RG, which completes the EAP-5G session. After this step, NAS messages between 5G-RG and W-AGF are transported without EAP-5G using W-CP signalling connection.

9.	[Conditional, if the 5G RG was CM-IDLE] An W-CP signalling connection is established between the 5G-RG and W-AGF.

Note:	Step 9-11 are defined by BBF/Cablelabs.

Editor's note:	The description below for how W-AGF maps the PDU Session resource management towards 5GC with the PDU Session resource management towards the RG needs to be verified by BBF/Cablelabs. Also, how W-AGF maps the PDU Session N3 tunnel towards 5GC with the PDU Session user plane towards the RG needs to be verified by BBF/Cablelabs.

Steps 10 and 11 are carried out for each PDU Session indicated in step 7

10.	Based on its own policies and configuration and based on the QoS flows and QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session

11.	The W-AGF sets up the W-UP resources for the PDU session. This step is specified by BBF for W-5BGAN and by CableLabs for W-5GCAN. The access dependent W-UP resource setup procedure shall map to the identity of the PDU Session associated with the W-UP resource..

12.	W-AGF notifies the AMF that the 5G-RG context was created by sending a N2 Initial Context Setup Response (N2 SM information that provides (AN Tunnel Info, List of accepted QoS Flows for, List of rejected QoS Flows) per PDU Session ID for PDU Sessions whose UP connections are activated).

13.	AMF sends NAS Service Accept via W-AGF to the 5G-RG.

14.	All steps after step 14 in TS 23.502 [3] figure 4.2.3.2-1 are performed for each requested PDU session user plane.

When the 5G-RG is in CM-CONNECTED state over W-5GAN access and the network receives downlink data for a PDU Session over wireline access that has no user plane connection, the steps 1-4a in TS 23.502 [3], clause 4.2.3.3 (Network Triggered Service Request) shall be performed with the following exceptions:

-	The (R)AN corresponds to an W-AGF.

-	The UE corresponds to the 5G-RG.

-	In step 4a, the steps 7-14 in figure 7.2.2-1 are performed to establish the W-UP resources and to establish N3 tunnel. In step 7, the AMF does not send the NAS Service Accept message to the UE.

NEXT CHANGE

END OF CHANGES