3GPP TSG-SA WG2 Meeting #133 	S2-1906396

# May 13 - 17, 2019, Reno, NV, USA							     (revision of S2-1905500 + S2-1905099)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1179 | rev | 7 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Traffic offload by UPF controlled by the I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - See paper S2-1903554 on how the information is transferred at N16a interface to support traffic offload by UPF controlled by the I-SMF. - Some other editorial change. Rev5: In S2#132 meeting,for UL-CL/BP controlled by the I-SMF, the main contentious issue is whether the SMF shall explicitly indicate to I-SMF which UPF the N4 rule is applied, i.e. whether the rule need be separated as the rule applied to UL-CL/BP and the rule applied to local PSA. The SMF does not need to know whether local PSA is combined or seperated from the UL CL/BP. However if more than one L-PSA separated from the UL CL/BP were selected by the I-SMF, it may be more complexity for the I-SMF to generate the rule installed at the standalone UL-CL/BP. For example how to set the precedence value in the UL-CL/BP PDR. On the other hand, if the L-PSA and UL-CL/BP is combined, the I-SMF only need combine the UL-CL/BP FAR with the L-PSA PDR into one. As the I-SMF always need check the received N4 information to fill some local information, it seems natural when the I-SMF check the N4 information also do the merged work if the L-PSA and UL-CL/BP are combined. So it is proposed the N4 information sent from SMF to the UL-CL is always splited between the UL-CL and L-PSA two parts. And I-SMF may merge the two parts into one depending on whether two UPF are splitted or combined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify which N4 information is sent from SMF to I-SMF, and how I-SMF generate N4 rules based on N4 information. - Editorial change. Rev5: - Clairfy whether the N4 information indicate which UPF the rule applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is not clear how the I-SMF generate N4 rules based on this information. - Some unclear description. Rev5: - Unclear whether one rule sent to the UPF and I-SMF need split the rule into two rule if the L-PSA and UL-CL/BP are separated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.6, 5.34.6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.34.6	Interaction between I-SMF and SMF for the support of traffic offload by UPF controlled by the I-SMF

5.34.6.1 General

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/Branching Point in VPLMN is not supported in HR case. It applies for the architectures described in clauses 5.34.4 and 5.34.5

When the I-SMF is inserted into a PDU Session, e.g. during PDU Session establishment or due to UE mobility, the I-SMF may provide the DNAI list it supports to the SMF. Based on the DNAI list information received from I-SMF, the SMF may provide the DNAI(s) of interest for this PDU Session for local traffic steering to the I-SMF e.g. immediately or when a new or updated or removed PCC rule(s) is/are received. The DNAI(s) of interest is derived from PCC rules.

The I-SMF is responsible for the insertion, modification and removal of UPF(s) to ensure local traffic steering. The SMF does not need to have access to local configuration or NRF output related with UPF(s) controlled by I-SMF. Based on the DNAI(s) of interest for this PDU Session for local traffic steering and UE location the I-SMF determines which DNAI(s) are to be selected, selects UPF(s) acting as UL CL/BP and/or PDU Session Anchor based on selected DNAI, and insert these UPF(s) into the data path of the PDU Session.

When a UL CL/BP has been inserted, changed or removed, the I-SMF indicates to the SMF that traffic offload have been inserted, updated or removed for a DNAI, providing also the IPv6 prefix that has been allocated in case a new IPv6 prefix has been allocated for the PDU Session.

From now on the SMF and I-SMF interactions entail:

-	Notifying the SMF with the new Prefix (multi-Homing case): the SMF is responsible of Issuing Router advertisement to the UE including the priorities provided to the UE to select among the prefixes related with the PDU Session.

-	N4 interactions related with traffic offloading. The SMF provide N4 information to the I-SMF for how the traffic shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF: the SMF issues requests to the I-SMF containing N4 information to be used for creating / updating /removing PDR, FAR, QER, URR, etc. The N4 information for local traffic offload provided by the SMF to the I-SMF are described in clause 5.34.6.x.

-	Receiving N4 notifications related with traffic usage reporting: the I-SMF forwards to the SMF N4 information corresponding to UPF notifications related with traffic usage reporting; the SMF aggregates and constructs usage reports towards PCF/CHF.

-	Sending Trace Requirements to the I-SMF; upon reception of Trace Requirements from the SMF the I-SMF shall provide Trace Requirements on the N4 interface towards the UPF(s) it is controlling.

NOTE:	How the SMF decides what traffic steering and enforcement actions are enforced in UPF(s) controlled by I-SMF is left for implementation.

The I-SMF is responsible of the N4 interface towards the local UPF(s) including:

  - the usage of AN Tunnel Info received from the 5G AN via the AMF in order to build PDR and FAR.

  - the CN Tunnel Info between local UPFs (if needed).

  - to control UPF actions when the UP of the PDU Session becomes INACTIVE

Next CHANGE

5.34.6.x	N4 information sent from SMF to I-SMF for local traffic offload

The SMF generates N4 information for local traffic offload based on the available DNAI(s) indicated by the I-SMF, PCC rules associated with these DNAI(s)  and charging requirement. This N4 information is sent from the SMF to the I-SMF after UL CL/Branching Point insertion/update/removal, and the I-SMF uses this N4 information to derive rules installed in the UPFs controlled by the I-SMF.

The N4 information for local traffic offload corresponds to rules and parameters defined in clause 5.8.2.11, i.e. PDR, FAR, URR and QER. It contains identifiers allowing the SMF to later modify or delete these rules.

N4 information for local traffic offload is generated by the SMF without knowledge of how many local UPF(s) are actually used by the I-SMF. The SMF indicates whether a rule within N4 information is enforced in UL CL/ Branching Point or local PSA. If the rule is applied to the local PSA, the N4 information includes the associated DNAI. If the local PSA(s) is collocated with UL CL/ Branching Point, the I-SMF combine the rule for UL CL/ Branching Point and local PSA(s) and send the combined rule to the UPF, which acts as the  UL-CL/Branching Point and PSA(s).

NOTE: The SMF is not aware of whether there is a single PSA or multiple PSA controlled by I-SMF.

The following parameters are generated by the I-SMF:

-    The 5G AN Tunnel Info

-	CN tunnel info between local UPFs.

-	Network instance (if needed).

The N4 information exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with an N16a association allowing the SMF to modify or delete the N4 information at a later stage.

The I-SMF generates an N4 Session ID and for each rule a Rule ID (unless the ones received from the SMF can be used) and maintains a mapping between the locally generated identifiers and the ones received from the SMF. The I-SMF replaces those IDs in the PDR(s), QER(s), URR(s) and FAR(s) received from the SMF. When the I-SMF receives the N4 information, the Network instance (if needed) included in the rules sent to the UPF is generated by I-SMF.

END OF CHANGES