SA WG2 Meeting #133	S2-1906476

13 – 17 May, 2019, Reno, Nevada, USA	(was S2-1905456)

Source:	CATT

Title:	Update 5GC-NI-LR Procedure when UE is served by the same PLMN

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to update 5GC-NI-LR procedure when UE is served by the same PLMN.

1.	Discussion

According to description in clause 5.16.4.4 in TS 23.501:

Over untrusted non-3GPP access, an Emergency Registered UE is only reachable in CM-CONNECTED state: since the UE may only use Emergency Services over untrusted Non-3GPP access when it is not possible over 3GPP access, 3GPP access is assumed to be unavailable for paging the UE.

Only when 3GPP access is unavailable, the non-3GPP access is used by UE to request emergency service.

When UE requests emergency service, AMF initiates the 5GC-NI-LR procedure. Based on the common positioning procedure in clause 6.9.1, LMF selects access type for the positioning procedure. If non-3GPP access is used by UE to request the emergency service, it means that the 3GPP access is not available, so there is no need for LMF to select access.

In order to simplify the 5GC-NI-LR procedure, it is proposed that the AMF sends access type selected by UE to LMF. The LMF uses the access type to perform the positioning procedure.

2.	Proposal

It is proposed to update TS 23.273 as follows.

*** First change ***

6.9.1	Common Positioning Procedures when UE is served by the same PLMN

Figure 6.9.1-1 shows the common positioning procedures to support location services with non-3GPP access when UE is served by the same PLMN, and the common positioning procedures can be triggered by:

-	5GC-NI-LR procedure (described in clause 6.10.1), which assumes that the serving AMF is aware of the emergency session initiation - e.g. due to supporting an Emergency Registration procedure or assisting in establishing an emergency PDU Session. If such emergency session procedure is initiated via non-3GPP access, the AMF provides access type to LMF in step 2 in clause 6.10.1,the LMF shall use the access type provided by AMF in step 2 in clause 6.9.1; or

-	5GC-MT-LR procedure (described in clause 6.1.1 and 6.1.2 and 6.10.2), which is applicable to a request from an LCS client for a current location.

Editor's note:	How the procedure in figure 6.9.1-1 triggered by other procedures, e.g. MO-LR procedure, depends on the progress of the study and FFS.

Figure 6.9.1-1: Common positioning procedures when UE is served by the one PLMN

1.	The LMF may initiate Namf_EventExposure_Subscribe_service operation towards the AMF to fetch the location information and/or CM state per access type in step 1a. The AMF invokes the Namf_EventExposure_Notify service operation to notify the events in step 1b. In case the location information provided by the AMF satisfies the requested LCS QoS requirement (e.g. accuracy), further steps may be skipped.

2.	By considering UE/network position capacity, position QoS requirement, and/or the CM state per access type, the access type if received from the H-GMLC that requests to perform Positioning procedure, the LMF determine the positioning methods and the access for which to determine positioning.

Furthermore, when the NRPPa protocol in TS 38.455 [15] is determined to be used to obtain location information from the NG-RAN or N3IWF/TNGF, the LMF shall decide to obtain the location information from which access node. Based on other information in the LMF, e.g. UE/network position capacity, the CM state per access type, position QoS requirement, the LMF may override the access type received from the H-GMLC and determine to obtain location information from the other access type.

When the LPP protocol in TS 36.355 [20] is determined to be used to obtain location information from UE, the AMF shall decide which access node should be chosen to transmit the location information. For example, if the UE is in CM-IDLE state in 3GPP access and in CM-CONNECTED state in Non-3GPP access, AMF may prefer to select the N3IWF/TNGF to relay positioning signalling to UE, rather than perform paging procedures via the 3GPP access.

When the NRPPa protocol in TS 38.455 [15] is determined to be used to obtain location information from the NG-RAN or N3IWF/TNGF:

-	In the case that 3GPP Access is determined to be used to transmit the location information, the Network Assisted Positioning procedure in step 3b or the obtaining Non-UE Associated Network Assistance Data in step 3c should be performed.

-	In the case that non-3GPP Access is determined to be used to transmit the location information, the Network Assisted Positioning procedure in step 3e should be performed.

When the LPP protocol in TS 36.355 [20] is determined to be used to obtain location information from the UE:

-	In the case that 3GPP Access is determined to be used to relay the location information, the UE Assisted and UE Based Positioning procedure in step 3a should be performed.

-	In the case that non-3GPP Access is determined to be used to relay the location information, the UE Assisted and UE Based Positioning procedure in step 3d should be performed.

3a.	This step is same as the UE Assisted and UE Based Positioning procedure in clause 6.11.1.

3b.	This step is same as the Network Assisted Positioning procedure in clause 6.112 with the difference that Access Type, i.e. 3GPP access type, which selected to obtain the location information from is included in step 1 of clause 6.11.2. The AMF forwards the Network Positioning message to the NG-RAN based on the received Access Type in step 3 of . clause 6.11.2.

3c.	This step is same as the Obtaining Non-UE Associated Network Assistance Data procedure in clause 6.11.3 with the difference that Access Type, i.e. 3GPP access type, which selected to obtain the location information from is included in step 1 of clause 6.11.3. The AMF forwards the Network Positioning message to the NG-RAN based on the received Access Type in step 2 of clause 6.11.3.

3d.	This step is same as the UE Assisted and UE Based Positioning procedure in clause 6.11.1 with the following difference:

-	The NG-RAN in clause 6.11.1 is corresponding to the N3IWF/TNGF in Figure 6.9.1-1;

-	Steps 2 and 5 in clause 6.11.1 should be skipped;

-	The location information returned is specified in clause 5.3.1.

3e.	This step is same as the Network Assisted Positioning in clause 6.11.2 with the following difference:

-	The NG-RAN in clause 6.11.2 is corresponding to the N3IWF/TNGF in Figure 6.9.1-1;

-	Steps 2 and 4 in clause 6.11.2 should be skipped;

-	The Access Type, i.e. non-3GPP access type, which selected to obtain the location information from is included in step 1 of clause 6.12. The AMF forwards the Network Positioning message to the N3IWF/TNGF based on the received Access Type in step 1 of .clause 6.11.2.

-	The location information returned is specified in clause 5.3.1.

*** End of change ***