3GPP TSG-SA2 Meeting #134	S2-1906899

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0003 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Network Performance related network data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Text not aligned with the conclusions of TR 23.791 (solutions #18 and #28 selected for normative work) - Inconsistencies: reference to "any UE", Tables 6.6.3-1 and 6.6.3-2 - Input with incomplete references on RAN and CN load - Input missing on mobility aspects |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Network performance analytics provide a correlation between the trajectories of UEs and the performance of the network elements (RAN, CN) that they use. - Removal of inconsistencies mentionned above - Input : references on RAN and CN load, mobility aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.6.2, 6.6.3, 6.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## 6.6	Network Performance Analytics

### 6.6.1	General

NWDAF analyses and correlates mobility information and network performance information, in order to provide statistics or predictions on the performance of network elements (CN, RAN) used by a UE or group of UEs during their different locations across time.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance".

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifier.

-	Analytics Filter Information containing:

-	Optional maximum number of results,

-	Optional Area of Interest (list of TA or Cells) which restricts the area in focus,

-	An Observation period indicates the time when the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

NOTE:	The tables below shows the information provided to the NWDAF, in other words the information provided by NWDAF is not shown here but in the procedures in clause 6.6.4.

Table 6.6.2-1:  Load and Performance information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Load and performance information | OAM | Statistics on RAN load and performance per Cell Id in the Area of Interest: usage of assigned virtual resources currently in use for a specific NF instance (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7. |
| Load information | NRF | Load per NF in the NF profile as defined per TS.29.510 [18]. |

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

The NWDAF shall be able to collect UE mobility information from OAM, 5GC and AFs as stated in clause 6.7.2.2.

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |  |  |
| --- | --- | --- | --- |
| Time slot entry (1..max) | Time slot(s) over which the statistics apply during the observation period. |  |  |
| > Time slot start | Time slot start within the Observation period. |  |
| > Duration | Duration of the time slot. |  |
| > Area subset (1..max) | Location within the requested area of interest where the UE or (part of) the UE group stays during the time slot. |  |  |
| >> Area subset description |  | List of TAI(s) or Cell ID(s) within the requested area. |  |
| >> RAN resource usage |  | Average usage of assigned resources (CPU, memory, disk). |  |
| >> NF load |  | Average load of the NF instances serving the area subset over the time slot. |  |
| >> Ratio of UEs |  | In case of a UE group, percentage of UEs in the group that are in this area subset. |  |
| >> Number of UEs | Number of UEs predicted in the area subnet. |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information |  | Description |  |
| --- | --- | --- | --- |
| Time slot entry (1..max) |  | Time slot(s) over which the prediction applies. |  |
| >Time slot start | Time slot start time within the Observation period. |
| > Duration | Duration of the time slot. |
| > Area subset (1..max) |  | Predicted location within the requested area where the UE or (part of) the UE group may move into during the time slot. |  |
| >> Area subset description |  | List of TAI(s) or Cell ID(s) within the requested area. |  |
| >> RAN resource usage |  | Average usage of assigned resources (CPU, memory, disk). |  |
| >> NF load |  | Average load of the NF instances serving the area subset over the time slot. |  |
| >> Ratio of UEs | In case of a UE group, percentage of UEs in the group that are predicted to be in the area subset. |
| >> Number of UEs |  | Number of UEs predicted in the area subset. |  |
| > Confidence |  | Confidence of this prediction. |  |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of results indicated in the request. Depending on this limitation, the least probable locations on a given observation period may not be provided.

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Observation Period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Observation period.

### 6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Analytics Filter="Area of interest" or "Area of interest" and "Internal Group Identifier", Observation Period(s)) to the NWDAF.

2a-2d.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest and subscribes to NF load information from NRF about these AMF(s).

3a-3b.	The NWDAF subscribes to OAM services to get the load and resource usage information for RAN nodes, on the Area of Interest, following the procedure captured in Clause 6.2.3.2.

Editor's note:	The exact SA WG5 service, granularity (NF, eNB) and measurement or KPI that are used for load.

4a-4b.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

Editor's note:	An Event ID such as "Number of UEs present in a geographical area" needs to be added to TS 23.502 [3] clause 5.2.2.3.1, as currently this Event ID is only specified for NEF.

5.	The NWDAF derives the requested analytics.

6.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion).

7-8. A change of network performance information, i.e. change in the load in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics based on the most recent data collected.

10.	The NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion.

* * * End of changes * * * *