SA WG2 Meeting #134	S2-1906916

June 24 - 28, 2019, Sapporo, Japan	(revision of S2-190xxxx)

Title:	DRAFT Response LS to (R2-1908465): LS to RAN1 and SA2 on mapping restriction for LCP procedure

Response to:	R2-1908465/S2-1906871

Release:	Rel-16

Work Item:	TBA

Source:	SA2

To:	RAN2

Cc:	RAN1

Attachments:

# Contact Person:

# Name:	               Adrian Buckley

E-mail Address:	adrian(dot)buckley(at)vivo.com

# 1. Overall Description:

SA2 would like to thank RAN2 for their LS (R2-1908465/S2-1906871).  SA2 have discussed the question asked by RAN2, copied below, and have provided an answer below.

  - Question 2: RAN2 has previously made some agreement on PC5 RRC, as highlighted in ANNEX A. The Layer 2 ID definition may impact RAN2 design on PC5-RRC message and procedure. Further, with regard to agreement 5 above, RAN2 has discussed if SL unicast for different destinations can be multiplexed into the same MAC PDU for the same target UE or not. RAN2 would like to ask SA2 how is the destination Layer 2 ID defined for SL unicast?

Answer:  SA2 would like to refer RAN2 to 3GPP TS 23.287 sub-clause 5.2.1.3, 5.6.1.4 and clause 6.3.  These sections state that V2X services can be multiplexed in the same pair of destination Layer-2 IDs.  Figure 5.2.1.4-1 provides a diagrammatic view of the answer to RAN2 where are 2 single unicast streams between 1 destination destination Layer-2 ID and 1 origination destination Layer 2-id.  The streams are differentiated by the Application Layer ID’s.

#### 5.2.1.4 Unicast mode communication over PC5 reference point … Figure 5.2.1.4-1: Granularity of PC5 Unicast Link

Table 1 – Extract from 3GPP TS 23.287

Sub-clause 5.1.2.1 bullet 6 also states that that multiple V2X services can be identified by the same initial Destination layer-2 ID.

#### 5.1.2.1 Policy/Parameter provisioning ….. 6) Policy/parameters when NR PC5 is selected: …. - The mapping of default Destination Layer-2 ID(s) for initial signalling to establish unicast connection and the V2X services, e.g. PSIDs or ITS-AIDs of the V2X application.

Table 2 – Extract from 3GPP TS 23.287

# 2. Actions:

# To RAN WG2

# SA2 kindly ask RAN2 to take into account the answer to Question 2

# 3. Date of Next RAN2 Meetings:

TSG-SA2 Meeting #135	14 -18 Oct 2019	Split, Croatia

TSG-SA2 Meeting #136				  18 – 22 November 2019		 Reno NV, USA