3GPP TSG-SA WG2 Meeting #134 	S2- 1906964

Sapporo, Japan, June 24-28, 2019 	(revision of S2-19xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1471 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On network reliability and context transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.21.3 describe the possible Network reliability enhancement with NF sets and NF Service sets. The set are defined to have access to the same context data. The objective of the context transfer procedure is, quoting TR 23.742, to “allow some service procedures to be handled by one service instance and the following procedures of the same service to be handled by the service instance from a different vendor” e.g. triggered by operator's OA&M system. Also, it is not clear that a context transfer procedure will improve the network reliability, as in general, if the source NF/NF service fails, then the context transfer will not be executed. As the instances in a set have access to the same context data, there is no relation between sets and context transfer procedures. Therefore, those procedures should not be mentioned in clause 5.21.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing some clarification on the text describing network reliability through equivalent NF Set and NF Service Sets, and removing a sentence about Context Transfer procedures in the context of Network Reliability support with Sets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading text in the standard |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.21.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.21.3	Network Reliability support with Sets

#### 5.21.3.1	General

A Network Function instance can be deployed such that several equivalent network function instances are present within an NF Set to provide distribution, redundancy and scalability together as a Set of NF instances. The same is also supported for NF Services. This can be achieved when the equivalent NFs and NF Services share the same context data, i.e. they are in the same NF Set or NF Service Set.

NOTE:	A NF can be replaced by an equivalent NF within the same NF Set in case of scenarios such as failure, load balancing, load re-balancing.

Such a network reliability design shall work in both communication modes, i.e. Direct Communication and Indirect Communication. In the Direct Communication mode, the NF Service consumer is involved in the reliability related procedures. In Indirect Communication mode, the SCP is involved in the reliability related procedures.

*** END CHANGES ***