3GPP TSG-SA2 Meeting #134	S2-1906983

# June 24 - 28, 2019, Sapporo, Japan.

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1476 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update BSF services with Binding Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-06-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) |  |
| Reason for change: |  | This is an alignment CR with the 23.503 CR of PCF Binding in BSF (S2-1906982). The highlighted information below needs to be updated in the affected BSF services. Binding information stored in the BSF User ID; UE Address; Slice info; DNN; PCF address; [PCF set ID]; PCF producer Binding Ind.. SUPI or/and GPSI; UE IP address/ Ethernet addr.; S-NSSAI; DNN; FQDN or IP Address of PCF; NF Set ID of the PCF; All information in either NF Instance or NF SET row of table 23.501 Table 6.3.1.0-1. |
|  |  | In addition, PCF has to be added as a possible consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating some of the input and output parameters in the BSF services to enable the BSF to store PCF Binding indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BSF services will not include new essential information added by corresponding CRs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.2.13	BSF Services

#### 5.2.13.1	General

The following table shows the BSF Services and Service Operations:

Table 5.2.13.1-1: NF services provided by the BSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nbsf_management | Register | Request/Response | PCF |
|  | Deregister | Request/Response | PCF |
|  | Discovery | Request/Response | NEF, AF, PCF |

#### 5.2.13.2	Nbsf_Management service

##### 5.2.13.2.1	General

The Nbsf provides the Nbsf Management Register, Nbsf Management Remove and the Nbsf Management Discovery service operations. PCF producer binding indication in the services below refers to the information in either NF Instance or NF SET row of table 23.501 Table 6.3.1.0-1.

##### 5.2.13.2.2	Nbsf_Management_Register service operation

Service Operation name: Nbsf_Management_Register.

Description: Registers the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI), PCF ID, PCF Set ID, PCF producer binding indication).

Inputs, Required: UE address(es), PCF ID, PCF producer binding indication.

UE address can contain IP address/prefix or Ethernet address as defined in TS 23.501 [2]. It can optionally refer to Frame Routes.

Frame Routes are defined in Table 5.2.3.3.1-1.

Inputs, Optional: DNN, SUPI, GPSI, DN information (e.g. S-NSSAI), PCF Set ID

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.13.2.3	Nbsf_Management_Deregister service operation

Service Operation name: Nbsf_Management_Deregister

Description: Removes the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI), PCF ID, PCF Set ID, PCF producer binding indication).

Inputs, Required: UE address(es)

UE address can contain IP address/prefix or Ethernet address as defined in TS 23.501 [2].

Inputs, Optional: DNN, SUPI, GPSI, DN information (e.g. S-NSSAI)

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF instance and PCF Set for the tuple (UE IP address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI)).

Inputs, Required: UE IP address, DNN [Conditional], DN information (e.g. S-NSSAI) [Conditional]

Inputs, Optional: SUPI, GPSI

Outputs, Required: PCF ID, PCF Set ID, PCF producer binding indication.

Outputs, Optional: None.

*** END CHANGES  ***