3GPP TSG SA2 Meeting #134	S2-1906984

# Sapporo, Japan, June 24th - 28th , 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0005 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing Editor´s note on Potential QoS change Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 18-06-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description of the Potential QoS change Analytics ID shows that NWDAF detects the need for notification about potential QoS change comparing the expected value for the KPI of the target 5QI against the threshold(s) provided by the consumer, in the location and the time required by the consumer. This Analytics ID does not provide a Potential QoS change, but more accurately, the detection of QoS KPIs threshold surpassed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to remove the Editor's note: “The usability of an existing Analytics ID instead of defining a new one is FFS.”. Propose to rename “Potential QoS change” into “QoS KPIs threshold surpassed”. |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved Editor´s note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9, 6.9.1, 6.9.2, 6.9.3, 6.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.9	QoS KPIs threshold surpassedanalytics

### 6.9.1	General

The consumer may request the NWDAF analytics information regarding QoS KPIs threshold surpassed  in a geographic area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters, as defined in clause 6.1.3:

-	Analytics ID = "QoS KPIs threshold surpassed”".

-	Analytics Filter Information:

-	QoS requirements:

-	Standardized 5QI, and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR)

-	For non-standardized QoS Characteristics, the QoS Characteristics attributes PDB and PER and their values (optional)

-	Location information: could be a path of interest, or a form of Geographical Area(s), e.g. geographical coordinates or a polygon describing an area. The location information could reflect a list of waypoints.

-	Threshold(s): The threshold(s) indicate level(s) which, if crossed, trigger the notification (improvement or worsening) can happen. The level(s) relate to the QoS KPIs, i.e. the average UL/DL throughput, DRB accessibility/retainability, etc., for the relevant 5QI(s) defined in TS 28.552 [8].

- Analytics Reporting Information: Observation Period (optional): could be relative time interval that indicates the time period for which the analytics about the potential QoS change is requested. If not provided, the NWDAF uses a default value set by the operator based e.g. on local policy.

- Target of Analytics Reporting: S-NSSAI (optional).

The NWDAF collects the corresponding statistics information on the QoS KPIs of interests from the OAM.

The NWDAF verifies whether the triggering conditions for the notification about QoS KPIs threshold surpassed are met and if so, generates for the consumer one or more notifications about QoS KPIs threshold surpassed .

The analytics feedback contains the information on the location in the geographic area and the time when QoS KPIs threshold mat be surpassed (and what threshold is crossed).

The QoS KPIs threshold surpassed detected by the NWDAF is based on data from the OAM, e.g. average UL/DL throughput, DRB accessibility/retainability, etc., for the relevant 5QI(s) as defined in TS 28.552 [8], and for parameters in the request from the consumer.

The NWDAF detects the need for notification about "QoS KPIs threshold surpassed " based on comparing the expected value for the KPI of the target 5QI against the threshold(s) provided by the consumer in any cell in the requested geographic area for the requested time information. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

The NWDAF provides notification on QoS KPIs threshold surpassed to the consumer.

### 6.9.2	Input data

Table 6.9.2-1: Data collection for QoS KPIs threshold surpassed analytics

| Information | Source | Description |
| --- | --- | --- |
| UE non-GBR average bitrate | OAM TS 28.552 [8] | Average UE bitrate in the cell per timeslot, per cell, per 5QI and per S-NSSAI. |
| GBR average failure rate | OAM TS 28.552 [8] | Average failure rate in the cell per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

NOTE 2:	Parameter "GBR average failure rate" is to be defined in TS 28.552 [8].

### 6.9.3	Output analytics

The analytics feedback contains the following information:

-	Information on the location in the geographic area and the period of time when a QoS KPIs threshold may be surpassed  (and what threshold(s) provided by the consumer is/are crossed).

### 6.9.4	Procedures

Figure 6.9.4-1 depicts a procedure for QoS KPIs threshold surpassed analytics provided by NWDAF.

Figure 6.9.4-1: QoS KPIs threshold surpassed analytics provided by NWDAF.

1.	The consumer requests or subscribes to analytics information on QoS KPIs threshold surpassed provided by NWDAF. The parameters included in the request are described in clause 6.9.1.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "time information" when requesting analytics information regarding potential QoS change for monitoring for a path of interest.

2.	The NWDAF collects, if necessary, the data, as described in clause 6.9.2 from the OAM.

3.	The NWDAF verifies whether the triggering conditions for the notification about QoS KPIs threshold surpassed are met. The NWDAF can detect the need for notification about " QoS KPIs threshold surpassed" based on comparing the expected value for the KPI of the target 5QI against the threshold(s) provided by consumer in any cell over the requested time information. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP.

4.	The NWDAF provides response or notification on QoS KPIs threshold surpassed to the consumer.

End of changes