SA WG2 Meeting #134 	S2-1907303

# June 24-28th, 2019 ; Sapporro, Japan		                                               (revision of S2-1907303)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0028 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Session Management procedures for RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Session Management procedures for RG require some clean up e.g. - Referring to BBF/Cable specifications - Editor's note: Applicability of multi-access PDU sessions is FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Session Management procedures for RG is updated to ensure the clean up indicated above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent or sometimes incorrect 23.316 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 ; 7.3.2 ; 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Basically almost same Tdoc than contributed for SA2 Reno (but more info provided on the support of multi-access PDU sessions) |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 7.3.0	General

This clause specifies the delta for Session Management procedure defined in TS 23.502 [3] clause 4.3 for 5G-RG and FN-RG.

Editor's note:	The support for FN-RG is FFS.

### 7.3.1	5G-RG Requested PDU Session Establishment via W-5GAN

#### 7.3.1.1	5G-RG PDU Session establishment via W-5GAN

Clause 7.3.1.1 specifies how a 5G-RG can establish a PDU Session via an W-5GAN as well as to hand over an existing PDU Session between 3GPP access and W-5GAN. The procedure applies in non-roaming scenarios.

The PDU Session Establishment procedure specified in TS 23.502 [3] clause 4.3.2.2.1 applies with the following changes.

Figure 7.3.1-1: 5G-RG PDU Session establishment via W-5GAN

- The 5G-RG shall send a PDU Session Establishment Request  message to AMF as specified in step 1 of clause 4.3.2.2.1. This message shall be sent to W-AGF via the W-CP signalling connection and the W-AGF shall transparently forward it in a N2 Uplink NAS transport message (NAS message, User location information, W-AGF identities) to AMF in the 5GC.

The W-AGF identities parameter may be included by the W-AGF and contains a list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF and can be used by SMF in step 8 in TS 23.502 [3] clause 4.3.2.2.1 as input to select an UPF.

In case the 5G-RG needs Hybrid Access for the PDU Session, the 5G-RG requests a MA PDU Session as defined in TS 23.501 [2] clause 5.32. In that case, Steps of TS 23.502 [3] clause 4.3.2.2.1  apply as modified by TS 23.502 [3] clause  4.22.2.1.

2a.	Steps 2-11 specified in TS 23.502 [3] clause 4.3.2.2.1 are executed according to the PDU Session Establishment procedure over 3GPP access with the deviation that in step 3 an additional parameter W-AGF identities received by the the AMF from the W-AGF can be sent from AMF to SMF. SMF can use W-AGF identities in step 8 of TS 23.502 [3] clause 4.3.2.2.1 for UPF selection.

For the LADN service, if the AMF detects the requested DNN is corresponding to a LADN DNN or the default DNN of the requesting S-NSSAI is a LADN DNN, and the access type of 5G-RG equals to wireline access, the AMF will assign "UE Presence in LADN service area" indication to be "OUT", and provide this indication to SMF.

NOTE:	This induces the SMF to reject the PDU Session establishment request

2b.	As described in step 11 and 12 of TS 23.502 [3] clause 4.3.2.2.1, the AMF shall under request of the SMF  send a N2 PDU Session Request message to W-AGF to establish the access resources for this PDU Session. The differences with step 11/12 of TS 23.502 [3] clause 4.3.2.2.1 are

-	N2 SM information does not contain (User Plane Security Enforcement information, UE Integrity Protection Maximum Data Rate, RSN) which do not apply top Wireline access. The W-AGF shall ignore RSN if received from 5GC. The W-AGF should reject N2 SM information that it has received with User Plane Security Enforcement information with a value of Required.

-	N2 SM information does not contain a Session-AMBR but a Session-TMBR,

3.	Based on its own policies and configuration and based on the QoS flows and QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session. For example, the W-AGF may decide to establish one W-UP resource and associate all QoS profiles with this W-UP resource. In this case, all QoS Flows of the PDU Session would be transferred over one W-UP resource.

4a.	The W-AGF sets up the W-UP resources for the PDU session. This step is specified by BBF for W-5BGAN and by CableLabs for W-5GCAN. The access dependent W-UP resource setup procedure shall provide the identity of the PDU Session associated with the W-UP resource. The W-UP resource setup procedure should support to bind W-UP resources to individual QFI(s) as specified in clause 4.4. The W-UP resource request may also contain other access layer information (e.g., VLAN id) specific for the W5GAN.

5.	After all W-UP resources are established, the W-AGF shall forward to 5G-RG via the W-CP signalling connection the PDU Session Establishment Accept message received in step 2b.

6.	The W-AGF shall send to AMF an N2 PDU Session Request Resource Setup Response (PDU Session ID, AN Tunnel Info, List of accepted/rejected QFI(s), User Plane Security Enforcement Policy Notification).

7.	All steps specified in TS 23.502 [3] clause 4.3.2.2.1 after step 14 are executed according to the PDU Session Establishment procedure over 3GPP access.

#### 7.3.1.2	PDU Session Establishment with ACS Discovery

This clause specifies how a 5G-RG can establish a PDU Session with ACS Discovery. The ACS discovery mechanism is further specified in clause 9.6.2.

When a 5G-RG performs ACS Discovery during PDU session establishment procedure, the UE Requested PDU Session Establishment via 3GPP Access as described in TS 23.502 [3] clause 4.3.2.2.1 is used, with the following differences:

-	UE is replaced by 5G-RG.

-	In FWA scenario the (R)AN is replaced by NG RAN access network, in wireline scenario the (R)AN is replaced by W-5GAN (as described in clause 6.2.3.1) and in HA scenario the (R)AN represents the selected access where the PDU session is being established.

-	Step 1. When the 5G-RG sends the PDU session establishment request it includes the DNN which is corresponding to the ACS (per local configuration or per URSP policies). The UE may send in PCO a request to provide ACS addressing information. The UE may include the ACS addressing information request in a DHCP request sent after PDU Session has been established.

-	Step 4. The UDM may send the ACSadressing  information to the SMF together with the subscription data based on the DNN.

-	Step 10. The SMF includes the ACS addressing information in the N1 SM information (PCO) if it has been requested by the UE in step 1 and if it is available at the SMF (if received in Step4).

-	Step 19: The 5G-RG may request to receive ACS addressing information via DHCP as described in clause 9.8.2.

-	The 5G-RG uses the received ACS addressing information to establish a connection with the ACS.

NOTE:	The ACS discovery via PCO or via DHCP are mutually exclusive.

### 7.3.2	5G-RG or Network Requested PDU Session Modification via W-5GAN

The UE or network requested PDU Session Modification procedure via W-5GAN access is depicted in figure 7.3.2-1. The procedure applies in non-roaming scenarios.

The procedure below is based on the PDU Session Modification procedure specified in TS 23.502 [3] clause 4.3.3.2.

Figure 7.3.2-1: 5G-RG or Network Requested PDU Session Modification via W-5GAN

1.	If the PDU Session Modification procedure is initiated by the UE, the UE shall send a PDU Session Modification Request message to AMF as specified in TS 23.502 [3] step 1 of clause 4.3.2.2. The message shall be sent to W-AGF via W-CP signalling connection. The W-AGF shall transparently forward the PDU Session Modification Request to AMF/SMF.

2.	The steps 1a (from AMF) to 1e and steps 2-3 as per the PDU Session Modification procedure in TS 23.502 [3] clause 4.3.3.2 are executed.

3.	The AMF sends N2 PDU Session Request (N2 SM information received from SMF, NAS message) message to the W-AGF. This step is the same as step 4 in clause 4.3.3.2.

4.	The W-AGF may issue W-CP resource modification procedure (out of scope of 3GPP) with the 5G-RG that is related with the information received from SMF. Based on the N2 SM information received from the SMF, the W-AGF may perform following:

4a.	[Conditional] The W-AGF may decide to create a new W-UP resource for the new QoS Flow(s).

4b.	[Conditional] The W-AGF may decide to add or remove QoS Flow(s) to/from an existing W-UP resource.

4c.	[Conditional] The W-AGF may decide to delete an existing W-UP resource, e.g. when there is no QoS Flow mapped to this W-UP resource.

NOTE:	If the W-AGF has included the Default W-UP resource indication during the establishment of one of the W-UP resources of the PDU Session, the W-AGF may not update the mapping between QoS Flows and W-UP resources.

5.	The W-AGF acknowledges N2 PDU Session Request by sending a N2 PDU Session Response Message to the AMF to acknowledge the success or failure of the request.

6.	Step 7 as per the PDU Session Modification procedure in TS 23.502 [3] clause 4.3.3.2 is executed.

7.	The W-AGF sends the PDU Session Modification Command to 5G-RG (if received in step 3) and receives the response message from 5G-RG.

Steps 4a/4c and step 7 may happen consecutively. Steps 7b map happen before step 4b/4d.

8.	The W-AGF forwards the NAS message to the AMF.

9.	All the steps after step 10 in TS 23.502 [3] clause 4.3.3.2 are executed according to the general PDU Session Modification procedure.

### 7.3.3	5G-RG or Network Requested PDU Session Release via W-5GAN

Clause 7.3.3 specifies how a 5G-RG or network can release a PDU Session via a W-5GAN. The 5G-RG requested PDU Session Release procedure via W-5GAN access applies in non-roaming scenarios.

If the 5G-RG is simultaneously registered to a 3GPP access in a PLMN different from the PLMN of the W-AGF, the functional entities in the following procedures are located in the PLMN of the W-AGF.

NOTE:	If the 5G-RG is simultaneously registered to 3GPP access in the same PLMN as W-5GAN access, when W-5GAN is not available to the 5G-RG (e.g. 5G-RG is disconnected from W-5GAN) or 5G-RG is in CM-IDLE for W-5GAN access, the 5G-RG may perform the PDU Session Release procedure via 3GPP access as described in TS 23.502 [3] clause 4.3.4.

Figure 7.3.3-1: 5G-RG or Network Requested PDU Session Release via W-5GAN access

1.	One or more PDU Sessions are already established for the 5G-RG using the procedure described in clause 7.3.1.

2.	The 5G-RG sends a PDU session release request (N1 SM container (PDU Session Release Request), PDU Session ID) to the AMF via the W-AGF as defined in clause 4.3.4 of TS 23.502 [3].

3.	The steps 1a (from AMF) to 4 according to the PDU Session Release procedure defined in TS 23.502 [3] clause 4.3.4.2 are executed.

4.	This step is the same as step 4 in clause 4.3.4.2 of TS 23.502 [3].

5.	Upon receiving AN session release request message from the AMF, the W-AGF can trigger the release of the corresponding W-UP resource with procedure out of scope of 3GPP.

6.	This step is the same as step 6 in clause 4.3.4.2 of TS 23.502 [3].

7.	Steps 7 according to the PDU Session Release procedure defined in clause 4.3.4.2 are executed.

8.	The W-AGF delivers the NAS message (N1 SM container (PDU Session Release Command), PDU Session ID, Cause) to the 5G-RG.

9.	The 5G-RG sends a NAS message (N1 SM container (PDU Session Release Ack), PDU Session ID) to the W-AGF.

10.	This step is the same as TS 23.502 [3] step 9 in clause 4.3.4.2.

Steps 5 and 8 may happen consecutively. Step 9 may happen before step 5.

11.	All steps after step 10 in the PDU Session Release procedure defined in TS 23.502 [3] clause 4.3.4.2 are executed.

The network requested PDU Session Release procedure via W-5GAN access is the same as the network requested PDU Session Release Procedure specified in TS 23.502 [3] clause 4.3.4.2 for Non-Roaming with the following differences:

-	The (R)AN corresponds to a W-AGF.

-	In step 5 upon receiving N2 SM request to release the AN resources associated with the PDU Session from the AMF, the W-AGF can trigger the release of the corresponding W-UP resource to the 5G-RG as specified in step 5, in Figure 7.3.3-1.

NEXT CHANGE

END OF CHANGES