3GPP TSG-SA2 Meeting #134	S2-1907321

# Sapporo, Japan, 24th June 2019 - 28th June 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0026 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the correlation information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph2 |  |  |  |  |  | Date: |  |  | 2019-06-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Observation 1: At the moment, it is unclear what’s the relationship between the clause 6.2.4 i.e. Correlation between network data and service data and the input data which are distributed in clause 6.4~6.9. Observation 2: Currently, the small data transfer in 5G IoT solutions in which the N3 interface is not involved is as follows: UE-RAN-AMF-SMF-UPF-DN as defined in clause 4.24, TS 23.502; So it is proposed to use the Time Interval and RAN UE NGAP ID to correlate the AMF data and the OAM data (reported by RAN). |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that: - The correlatin information per NF is subject to all the network data analytics; - The correlation information is not listed in the input data per network data analytics; - Use the Time Interval and RAN UE NGAP ID to correlate the AMF data and the OAM data (reported by RAN). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relationship between the clause 6.2.4 i.e. Correlation between network data and service data and the input data which are distributed in clause 6.4~6.9 is not clear. And it is unclear how to correlate the AMF data and the RAN data in the 5G IoT scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 6.4.2, 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

6.2.4	Correlation between network data and service data

The Correlation information per each NF is to help NWDAF correlate data from different NFs are defined in Table 6.2.4-1 here below, which is subject to all the network data analytics.

NOTE: For simplicity, the correlation information is not listed in the input data per network data analytics.

Table 6.2.4-1: Correlation Information per NF

| Correlation Information | Description |
| --- | --- |
| Time Interval and IP address 5-tuple | To correlate the data from AF and from UPF. |
| Time Interval and AN Tunnel Info (Clause 9.3.2.2, TS 38.413 [16]) | To correlate the UPF data and OAM data which are reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Time Interval and UE IP address | To correlate the data from UPF and SMF. |
| SUPI | To correlate data from SMF and AMF |
| SUPI, DNN, S-NSSAI or UE IP address | To correlate data from SMF and PCF |
| Time Interval and RAN UE NGAP ID (Clause 9.3.3.2, TS 38.413 [16]) | To correlate the AMF data and OAM data reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |

**** Next Change ****

6.4.2	Input Data

The service data from the AF and the network data from 5GC NFs for observed service experience are defined in Table 6.4.2-1 and Table 6.4.2-2 here below.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [3].

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | To identify the DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | PCF/SMF | Provided by the AF, which is used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information |  | Provided by the AF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | AMF/PCF/SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

**** Next Change ****

6.7.3.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.7.3.2-1.

Table 6.7.3.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Internal Group ID |  | To identify UE group if available |
| Application ID | AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 [3] |
| UE communication (1..max) | AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication expected by the application |
| >DL data rate |  | DL data rate of this communication expected by the application |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

**** End of Change ****