SA WG2 Meeting #134	S2-1907369

# 24 - 28 June 2019, Sapporo, Japan	 										(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1573 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conditions to use CP or UP CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.501 gives inaccurate conditions for the AMF and SMF to determine whether switching from CP to UP is possible. The negotiation of Preferred and Supported Network Behaviour is only negotiated during registration time. If the result of that negotiation allows only Control Plane CIoT 5GS Optimisation, then the AMF may, based on local policy, choose to restrict the PDU session to CP only, and flag it to SMF accordingly. The existing text without change implies that the AMF would re-do the Preferred and Supported Network Behaviour also at PDU session establishment time, which is not correct. For the sake of sequencing, it is also necessary to use just one small data path at a time, so CP CIoT 5GS Optimisation should not be used on a PDU session where UP CIOT 5GS Optimisation is enabled. CIoT EPS Optimisations in TS 23.401 already mandates this behaviour for the UE and the MME. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All AMF input parameters to determine whether to apply Control Plane Only Indicator or not is enumerated in the summary clause to avoid impression that the AMF local policy alone could determine the use of Control Plane Only Indicator. The use of Control Plane CIoT 5GS Optimisation at the same time as User Plane CIoT 5GS Optimisation is forbidden. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect criteria for using Control Plane CIoT 5GS Optimisation only and possible small data sequencing errors if CP and UP CIoT 5GS Optimisation are used at the same time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.4.1, 5.31.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 5.31.4.1	General

The Control Plane CIoT 5GS Optimisation is used to exchange user data between the UE and the SMF as payload of a NAS message in both uplink and downlink directions, avoiding the establishment of a user plane connection for the PDU Session. The UE and the AMF perform integrity protection and ciphering for the user data by using NAS PDU integrity protection and ciphering. For IP and Ethernet data, the UE and the SMF may negotiate and perform header compression.

UE and AMF negotiate support and use of Control Plane CIoT 5GS Optimisation as defined in clause 5.31.2. When the Control Plane CIoT 5GS Optimisation feature is used and the PDU session type is unstructured, the SMF selects either NEF or UPF based on information in the UE's subscription.

If UE and network have negotiated support and use of Control Plane CIoT 5GS Optimisation then the following paragraphs of this clause apply.

During the PDU Session Establishment procedure the AMF indicates to the SMF Control Plane CIoT 5GS Optimisation is available for data transmission.

During the PDU Session Establishment procedure the AMF also determines based on Preferred and Supported Network Behaviour (see clause 5.31.2), subscriber data, other already established PDU Sessions and local policy whether a new PDU session shall only use the Control Plane 5GS Optimisation (i.e. that a user-plane connection shall never be established for the new PDU session). If a PDU session shall only use Control Plane 5GS Optimisation, the AMF provides a Control Plane Only Indicator to the SMF during the PDU session establishment. The SMF provides the Control Plane Only Indicator in the Session Management Request to the UE. A UE and SMF receiving the Control Plane Only Indicator for a PDU session shall always use the Control Plane CIoT 5GS Optimisation for this PDU session.

The following rules apply for the use of the Control Plane Only Indicator during PDU session establishment:

-	If N3 data transfer was not successfully negotiated, all PDU sessions shall include Control Plane Only Indicator.

-	For a new PDU session of type unstructured towards a DNN/S-NSSAI for which the subscription includes a NEF Identity for NIDD (i.e. for a PDU session which will be anchored in NEF), the AMF shall always include the Control Plane Only Indicator.

-	For a new PDU session towards a DNN/S-NSSAI for which the subscription does not include a NEF Identity for NIDD (i.e. for a PDU session which will be anchored in UPF) and that supports interworking with EPS based on the subscription data defined in TS 23.502 [3]:

-	If the other criteria for setting the Control Plane Only Indicator is met, then for the first PDU Session the AMF determines based on local policy whether to include the Control Plane Only Indicator or not;

-	if the AMF previously included a Control Plane Only Indicator for PDU sessions that support interworking with EPS based on the subscription data defined in TS 23.502 [3] and that are anchored in UPF, the AMF shall include it also for the new PDU session;

-	if the AMF previously did not include a Control Plane Only Indicator for any of the PDU sessions that support interworking with EPS based on the subscription data defined in TS 23.502 [3] and that are anchored in UPF, the AMF shall not include it for the new PDU session.

-	For a new PDU session towards a DNN/S-NSSAI for which the subscription does not include a NEF Identity for NIDD (i.e. for a PDU session which will be anchored in UPF) and that does not support interworking with EPS based on the subscription data defined in TS 23.502 [3], AMF determines individually per PDU session whether to include the Control Plane Only Indicator or not.

If the UE and AMF negotiate support and use of Control Plane CIoT 5GS Optimisation as defined in clause 5.31.2, the UE may initiate service request procedure for Control Plane CIoT 5GS Optimisation as described in TS 23.502 [3] for small data transmission. The UE may request to activate user plane resources for one or more PDU sessions for which Control Plane Only Indicator was not received as described in clause 5.31.4.2.

The SMF decides during service request procedure for Control Plane CIoT 5GS Optimisation, based on UE request, whether to activate user plane resources or not for each PDU session for which Control Plane Only Indicator was not indicated.

In CM-CONNECTED, the UE uses N3 delivery for PDU sessions for which user plane resources are established, and uses NAS for data transmission for PDU sessions for which user plane resources are not established.

Early Data Transmission may be initiated by the UE for mobile originated Control Plane CIoT 5GS Optimisation.

***** Next change *****

##### 5.31.4.2	Establishment of N3 data transfer during Data Transport in Control Plane CIoT 5GS Optimisation

If UE and AMF have successfully negotiated N3 data transfer in addition to Control Plane CIoT 5GS Optimisation based on the Preferred and Supported Network Behaviour as defined in clause 5.31.2, then the SMF may decide to establish N3 data transfer for any PDU session for which Control Plane Only Indicator was not included based on local SMF decision e.g. based on the amount of data transferred in UL or DL using Control Plane CIoT 5GS Optimisation. In that case the SMF initiates network initiated N3 data transfer establishment procedure as described in TS 23.502 [3].

If UE and AMF successfully negotiate N3 data transfer in addition to Control Plane CIoT 5GS Optimisation based on the Preferred and Supported Network Behaviour as defined in clause 5.31.2, then the UE may decide to establish N3 data transfer for any PDU session for which Control Plane Only Indicator was not included based on local decision, e.g. based on the amount of data to be transferred. In that case the UE performs UE initiated N3 data transfer establishment procedure as described in TS 23.502 [3].

#### When the N3 data transfer is set up for a PDU session, the UE and the SMF shall only use user plane radio bearers to transfer data PDUs on that PDU Session. This is done in order to minimise the risk of out of sequence data packets transferred over Control Plane CIoT 5GS Optimisation and User Plane CIoT 5GS Optimisation,