3GPP TSG-SA2 Meeting #134	S2-1907393

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1566 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SM Context Data Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resolution of both editor's note: Editor's note: This table is known to be incomplete and to require updates. Some items listed in the table may be removed. This table requires also updates to cover all kind of SM contexts not limited to I-SMF and V-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SM Context Data Table is updated to include missing parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.2.10, 5.2.8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First change * * * *

##### 5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO), or during mobility procedure with I-SMF changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, PDU Session ID (include PDU Session ID when available).

Output, Required: SM Context Container.

Output, Optional:Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requrested, e.g. PDN Connection Context ,5G SM Context or both. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between SMF(s), I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context that may be transferred betweenSMF(s), I-SMF(s) or between V-SMF(s)

| Field |  | applicable for I-SMF/V-SMF | Description |
| --- | --- | --- | --- |
| SUPI |  | x | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |
| Unauthenticated SUPI |  |  | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. |
| PEI |  |  | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. When present, it shall contain the PDU Session ID. |
| GPSI |  |  | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |
| Trace reference |  | x | Identifies a record or a collection of records for a particular trace. |
| Trace type |  | x | Indicates the type of trace |
| OMC identity |  | x | Identifies the OMC that shall receive the trace record(s). |
| S-NSSAI |  | x | The requested S-NSSAI for the serving PLMN to the PDU Session. |
| HPLMN S-NSSAI |  | x | Requested S-NSSAI for the HPLMN to the Home-Routed PDU Session |
| Network Slice Instance id |  | x | The network Slice Instance information for the PDU Session |
| DNN |  | x | The associated DNN for the PDU Session. |
| DNN selection mode |  |  | Indicate whether the requested DNN corresponds to an explicitly subscribed DNN or to the usage of a wildcard subscription. |
| AMF Information |  | x | The associated AMF instance identifier and GUAMI. |
| Service Name (AMF) |  |  | When present, this IE shall contain the name of the AMF service to which SM context status notifications are to be sent |
| Access Type |  | x | The current access type for this PDU Session. |
| RAT Type |  | x | RAT Type used by the UE. |
| PDU Session ID |  | x | The identifier of the PDU Session. |
| PDN type |  |  | IPv4, IPv6, IPv4v6 ,Ethernet or Unstructured |
| UE IP address(es) |  |  | IPv4 address and or/ IPv6 prefix |
| MAC address(es) |  |  | UE MAC address |
| PDU session IPv6 prefix |  |  | IPv6 Prefix allocated to the PDU Session |
| H-SMF Information |  | x | The associated H-SMF identifier and H-SMF address for the HR PDU Session.( applies only for a V-SMF) |
| Presence in LADN |  |  | This IE shall be present if the DNN corresponds to a LADN |
| UE location |  |  | This IE shall contain the UE location information, if it is available. |
| UE time zone |  |  | This IE shall contain the UE time zone information, if it is available. |
| Allocated EBI list |  |  | This IE shall be present if the consumer NF is an AMF and Inter-system mobility happens. When present, it shall contain an array of EBI to ARP mappings currently allocated to the PDU session. |
| PCF ID |  |  | When present, this IE shall contain the identifier of the PCF selected by the AMF for the UE (for Access and Mobility Policy Control); it shall be the V-PCF in LBO roaming and the H-PCF in HR roaming. |
| UDM group ID |  |  | When present, it shall indicate the identity of the UDM group serving the UE. |
| UPF Information for N4 (control plane) |  |  | The associated Control Plane UPF identifier and UPF address of N4 interface for the PDU Session. |
| UPF Information for N4 (user plane) |  |  | The associated User Plane UPF identifier and UPF address of the N4 interface for the PDU Session. |
| UPF Information for N3/9 interface |  |  | UPF TEID and UPF IP address for N9 UL/DL or N3 UL. |
| AN Information for N3 DL |  |  | AN N3 TEID and IP address for DL |
| SM Policy information |  |  | Information on SM policy provided by PCF. Includes the Policy Control Request Triggers and the Policy Control Request Information. |
| Event subscriptions |  |  | List of the event subscriptions by other CP NFs. Indicating the events being subscribed as well as any information on how to send the corresponding notifications. |
|  | For each QoS Flow in the PDU Session: |  |  |
| 5G QoS Identifier (5QI) |  | x | Identifier for the authorized QoS parameters for the service data flow. |
| ARP |  | x | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |
| GFBR |  | x | Guaranteed Flow Bit Rate (GFBR) - UL and DL |
| MFBR |  | x | Maximum Flow Bit Rate (MFBR) - UL and DL |
| Priority Level |  | x | Indicates a priority in scheduling resources among QoS Flows. |
| Averaging Window |  | x | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |
| Maximum Data Burst Volume |  | x | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |
| Reflective QoS Control |  | x | Indicates to apply reflective QoS for the SDF in the TFT. |
| QoS Notification Control (QNC) |  | x | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |
| Maximum Packet Loss Rate |  | x | Maximum Packet Loss Rate - UL and DL. |
|  | Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |  |  |
| EPS Bearer Id |  | x | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN |
| Mapped EPS Bearer QoS |  | x | ARP, GBR, MBR, QCI. |
| PGW-U tunnel Information |  | x | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |
| TFT |  | x | Traffic Flow Template |

*** NEXT CHANGE ***

##### 5.2.8.2.11	Nsmf_PDUSession_ContextPush service operation

Service operation name: Nsmf_PDUSession_ContextPushRequest

Description: This service operation is used by the SMF as Service Consumer to push one SM Context to a another SMF as NF Service Producer. It may be triggered by OAM.

Input, Required: SM Context of identified PDU session.

The SM context includes SM context in I-SMF(or V-SMF) and SM context in SMF (or H-SMF) separately.

See Table 5.2.8.2.10-1 for single SM Context stored in SMF, I-SMF or V-SMF that may be transferred to another SMF instance.

Output, Required: Result Indication.

Output, Optional: Cause.

See clause 4.26.2 for an example usage of this service operation.

* * * * End of changes * * * *