SA WG2 Meeting #134	S2-1907421

# 24 - 28 June 2019, Sapporo, Japan	 										(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3529 | rev |  | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location Reporting: LI and signalling load |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5, LI15 |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TSG SA #84 approved CRs S2-1904832, S2-1904834 and S2-1906165 on reporting of Primary cell of the Secondary node to the core network. When approving the CRs, TSG SA sent an LS (SP-190580) to SA2 noting that: “it should be possible to enable or disable secondary reporting of Secondary Cell information on a per UE basis.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The recently added Location Reporting request to report all the PSCell ID changes is modified so that the MME can request (or not): Reporting of all PSCell ID changes for that UE (as well as changes in Master RAN node primary cell) Local LI policy can be used to control whether this applies to all or only some of the UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI requirements and signalling load requirements not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.9.1	Location Reporting Procedure

This procedure is used by an MME to request the eNodeB to report where the UE is currently located when the target UE is in ECM-CONNECTED state. The need for the eNodeB to continue reporting ceases when the UE transitions to ECM-IDLE. This procedure may be used for services that require accurate cell identification (e.g. for emergency services, lawful intercept, charging).

Figure 5.9.1-1: Location Reporting Procedure

1)	The MME sends a Location Reporting Control message to the eNodeB. The Location Reporting Control message shall identify the UE for which reports are requested, the requested location information and may contain information such as reporting type. Requested location information is TAI+EGCI, and if requested by the MME, PSCell ID.

Reporting type indicates whether the message is intended to trigger:

a single stand-alone report about the current Cell ID serving the UE, or,

start the eNodeB to report whenever the UE changes cell.

In addition, the MME shall be able to control whether or not the RAN reports all changes in the UE’s PSCell ID.

2)	The eNodeB sends a Location Report message informing the MME about the location of the UE which shall include the requested location information.

3)	The MME can send a Cancel Location Reporting message to inform the eNodeB that it should terminate location reporting for a given UE. This message is needed only when the reporting was requested for a reporting period.

NOTE:	Location reporting is transferred during X2 handover, although new control signalling is not transferred during an active handover.

************ end of changes ********************