3GPP TSG-SA WG2 Meeting #134 	S2- 1907477

Sapporo, Japan, 24 - 28 June, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1603 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revision of configuration parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the list of 802.1Qbv parameters to be mapped in 5GS (see 5.28.2), Traffic Class and its Priorities are listed. However, 802.1Qbv only refers to Traffic Classes. Moreover, there is no need to map Priorities because every Traffic Class and its Priorities have already been linked in 5GS at the reporting phase (see 5.28.1). Also, 802.1Qbv does not provide information about the ingress ports. The traffic forwarding parameters listed cannot be extracted from 802.1Qbv. A reference on the standard providing them is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Priorities and ingress ports have been removed from the list of parameters to map from 802.1Qbv. The traffic forwarding parameters need an IEEE source/reference on how they are provided via CNC. We moved the traffic forwarding list of parameters away from the previous list related to 802.1Qbv, and added an Editor’s Note in order to request a source/reference for such information according to IEEE standards. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifying mapping of certain parameters that cannot be extracted from IEEE standard information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.28.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The configuration information of 5GS Bridge as defined in 802.1Qbv [96], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes.

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

Traffic forwarding information:

-	Destination MAC address, Traffic class and VLAN ID of TSN stream;

-	The ingress port ID and egress port ID on 5GS Bridge.

Editor's note:	IEEE standard reference on the traffic forwarding information provided via CNC is FFS.

Editor's note:	Support of "exclusive gating mechanism" is FFS.

The association between the MAC address used by the PDU Session, 5GS Bridge ID and port on UE side is maintained and further used to assist to bind the TSN traffic with the UE.

In the case of provisioning traffic forwarding information, the AF decides the MAC address used by the PDU Session for the TSN traffic, and requests to influence traffic routing as defined in clause 5.6.7 with the UE MAC address of the PDU Session, traffic class, stream destination MAC address, ingress port ID or egress port ID on the NW-TT side and VLAN ID.

*** END CHANGES ***