3GPP TSG-SA WG2 Meeting #134 	S2- 1907479

Sapporo, Japan, 24 - 28 June, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1599 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSN QoS mapping additions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DP S2-1903376 “ TSN QoS parameter mapping” explains the background the fundaments to build the 5GS-TSN QoS mapping table. Although most changes were agreed upon, the detail on the mapping table and particularly how to map PDB and independentMaxDelay is included in this paper. The reason is that independentMaxDelay (mandatory QoS parameter in TSN) is tightly related to PDB in 5GS and is a key parameter for QoS mapping. In this CR paper, we propose that PDB can be mapped to: independentDelayMax - UE-DS-TT Residence Time. DP S2-1903376 “Principle of 5G bridge port configuration” introduces traffic class to QoS Flow mapping principle. In this paper, we indicate that “a traffic class may be mapped to a QoS flow”, which does not exclude other possibilities. More importantly, we indicate here that different traffic classes can not be mapped into the same QoS flow, as every traffic class has a different QoS profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper indicates necessary parameters that are included in TSN-5GS QoS mapping table, and how to map PDB and independentMaxDelay. We indicate that a TSN traffic class could be mapped to a QoS flow. Moreover, we propose that more than one traffic class should not be mapped to a single QoS flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the TSN-5GS QoS mapping. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.4 |  |  |  |  |  |  |  |  |
| different traffic classes can not be mapped into same QoS flowed. |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.28.4	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.28.1, and (2) QoS configuration phase as in clause 5.28.2

(1)	The TSN Translator AF shall be pre-configured via OAM the mapping table and represents what is being reported or exposed to the TSN system (i.e. the CNC) through the relevant TSN information objects, such as the Traffic Class Table for every port, see the IEEE 802.1Q [X] and IEEE 802.1Qcc [95].

(2)	CNC distributes the TSN QoS requirements and TSN scheduling parameters (specific for current node) to 5G virtual bridge via TSN AF. Alternatively, the 5GS virtual bridge may pre-request or query CNC for the TSN QoS and traffic information.

The PCF mapping table provides a mapping from TSN QoS information to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS flow for the requested traffic class according to the selected QoS policies from the TSN AF traffic requirements.

Figure 5.28.4-1 illustrates the functional distribution of the mapping tables.

Figure 5.28.4-1: QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements in the 5GS are: traffic classes and their priorities per port, bridge delays per port pair and traffic class (independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), and propagation delay per port (txPropagationDelay).

QoS mapping table between port traffic classes and 5QI should be matching the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles.

Once the 5QIs to be used for TSN streams are identified, then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs. The QoS mapping table includes the parameters that are reported to the CNC as possible QoS classes that can be configured in the 5GS virtual bridge: Traffic Class, Priority, and Delay are mapped to the respective 5QI. Specifically, PDB can be mapped to: independentDelayMax minus UE-DS-TT Residence Time.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge per port and per traffic class. The most relevant information received is the scheduling for every traffic class and port of the bridge. At this point, it is possible to retrieve the real QoS requirements by identifying the traffic class of the port. Then the traffic class to 5QI mapping can be performed using the QoS mapping table in the TSN AF. Subsequently, the TSC QoS flow can be configured using the 5QI retrieved from the QoS mapping table. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS. The scheduling configuration information per traffic class is mapped to trigger creation/modification of a QoS flow in 5GS.

In a 5G virtual bridge, each traffic class may be mapped into a QoS flow. In case the traffic classes need to be differentiated within 5GS, then the different traffic classes can not be mapped into same QoS flow.

*** END CHANGES ***