3GPP TSG-SA2 Meeting #134	S2-1907559

# Sapporo, Japan，24 - 28 June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1604 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for N19 Tunnel establishment and release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear when the SMF determines to establish and release N19 tunnels between UPFs in 5G VN group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal about the procedures of N19 tunnels establishment and release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear when the SMF determines to establish and release N19 tunnels between UPFs in 5G VN group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X, 4.3.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

##### 4.3.X	N19 Tunnel Establishment

There are two cases for N19 tunnel establishment. One is that SMF may pre-configure the N19 tunnels between UPFs before any 5G VN PDU session establishment. The other one is SMF may determine to establishment the N19 tunnels between UPFs during the PDU session establishment procedure when the UPF is a new UPF for the 5G VN group.

Figure 4.3.X-1 Procedure for N19 Tunnel Establishment

1. [Conditional] Step 1-9 of UE requestes PDU Session Establishment in clause 4.3.2.2.1. And the UPF in this PDU session establishment is a new UPF for 5G VN group.

2. SMF determines to establish N19 tunnels between the UPF and UPF1

3a.The SMF sends an N4 Session Establishment Request to the UPF with N19 CN tunnel info of UPF1, if N19 CN tunnel info is allocated by SMF.

3b.UPF sends a N4 Session Establishment Response to SMF, with the N19 CN tunnel info of UPF, if CN tunnel info is allocated by the UPF

4a. The SMF sends a N4 Session Modification Request to UPF1 with N19 CN tunnel info of UPF and N19 CN tunnel of UPF1 if N19 CN tunnel info is allocated by the SMF.

4b.UPF1 sends a N4 Session Modification Response to SMF with N19 CN tunnel info if N19 CN tunnel info is allocated by UPF1.

5a. [Conditional] If N19 CN tunnel info is allocated by UPF, SMF sends a N4 Session Modification Request to UPF with N19 CN tunnel info of UPF1.

5b.[Conditional] UPF sends a N4 Session Modification Response to SMF.

<<< End of change1 >>>

##### 4.3.Y	N19 Tunnel Release

SMF determines to release N19 tunnel between a UPF and other UPF(s) belongs to a 5G VN group when there is no more PDU sessions of the 5G VN group in that UPF. N19 tunnel(s) may be released during the procedure of PDU Session release.In the procedure, UPF is the one that related with 5G VN PDU session release,  and UPF1 is an exsiting UPF in this 5G LAN group.

Figure 4.3.Y-1 Procedure for N19 Tunnel Release

1. [Conditional]step 1 of UE or network requested PDU Session Release in clause 4.3.4.2-1.

2. SMF determines to release N19 tunnel. If this PDU session is the last one of 5G VN group in the UPF, SMF will determine to release the N19 tunnel between the UPF and UPF1 in the 5G VN group.

3a. The SMF sends an N4 Session Release Request (N4 Session ID, N19 CN tunnel Info) message to the UPF. The UPF shall drop any remaining packets of the PDU Session and release N19 CN tunnel info resource and contexts associated with the N4 Session.

3b.	The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response message to the SMF.

4a.The SMF sends a N4 Session Modification Request to UPF1. The UPF1 shall drop any remaining packets of the PDU Session and release N19 CN tunnel info resource and contexts associated with the N4 Session.

4b. The UPF acknowledges the N4 Session Modification Request.

<<< End of changes >>>