3GPP TSG-SA WG2 Meeting #134	S2-1907571

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-190XXXX)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1610 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MA PDU Session Release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Different terminology is compared to single access PDU Session Release procedure. 2. In home routed roaming scenario, current description says that the H-SMF sends release notification only when the MA PDU Session is released over both accesses. However, when the MA PDU Session is established in two PLMNs, the release notification should be sent to the AMF so that the AMF release the associated PDU Session Contexts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change terminology "N2 SM Information" to "N2 SM Resource Release request" 2. Clarify that the H-SMF sends release notification when the MA PDU Session is established in two PLMNs and the MA PDU Session is released over single access. 3. Change style of title 4.22.10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong SMF behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.10	MA PDU Session Release

4.22.10.1	General

The MA PDU Session Release procedure is used to release the MA PDU Session or release the MA PDU Session over a single access. The MA PDU Session release over a single access may be triggered by the network due to e.g. when the UE is deregistered over an access or when S-NSSAI of the MA PDU Session is not in the Allowed NSSAI over an access.

4.22.10.2	UE or network requested MA PDU Session Release (non-roaming and roaming with local breakout)

The signalling flow for a MA PDU Session Release when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.4.2-1 with the following differences and clarifications:

-	In step 1, If the AMF need to release the MA PDU Session over a single access, the AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation to request the release of the MA PDU Session over a single access. In this case, the AMF includes in which access the MA PDU Session should be released.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session completely released or released over a single access based on its local policy.

-	In step 1, if the AMF need to release the MA PDU Session (e.g. locally released when the UE is CM-IDLE), the AMF invokes the Nsmf_PDUSession_ReleaseSMContext service operation to request the release of the MA PDU Session.

-	In step 1, when the SMF initiates MA PDU Session Release procedure, the SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the PDU Session Release Command message.

-	In step 3, if the SMF releases the MA PDU Session over a single access, the SMF shall not include "skip indicator" in the Namf_Communication_N1N2MessageTransfer service.

In step 3, if the SMF releases the MA PDU Session over both accesses and user plane resources are established in both accesses, the SMF includes both N1 SM container (PDU Session Release Command) and N2 SM Resource Release request together in the Nsmf_PDUSession_UpdateSMContext or Namf_Communication_N1N2MessageTransfer service so that the UE does not request to activate user plane resources. The SMF releases user plane resources of the other access by including N2 SM Resource Release request only in Namf_Communication_N1N2MessageTransfer service.

-	In step 3, when the SMF provides N1 SM container and/orN2 SM Resource Release request, the SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the AMF.

-	In step 11, the SMF triggers Nsmf_PDUSession_SMContextStatusNotify service only when the MA PDU Session is released in both accesses.

4.22.10.3	UE or network requested MA PDU Session Release (home-routed roaming)

The signalling flow for a MA PDU Session Modification when the UE is roaming and the PDU Session Anchor (PSA) is located in the HPLMN, is based on the signalling flow in Figure 4.3.4.3-1 with the following differences and clarifications:

-	In step 1, If the V-AMF need to release the MA PDU Session over a single access, the V-AMF may invoke the Nsmf_PDUSession_UpdateSMContext service operation to request the release of the MA PDU Session over a single access. In this case, the AMF shall include in which access the MA PDU Session should be released. The V-SMF invokes Nsmf_PDUSession_Update service operation to request the release of the MA PDU Session over a single access. The V-SMF shall include in which access the MA PDU Session should be released.

NOTE:	When the H-SMF received the release request from the V-SMF, the H-SMF decides whether the MA PDU Session released or released over a single access based on its local policy.

-	In step 1, if the V-AMF need to release the MA PDU Session (e.g. locally released when the UE is CM-IDLE), the AMF invokes the Nsmf_PDUSession_ReleaseSMContext service operation to request the release of the MA PDU Session. The V-SMF invokes Nsmf_PDUSession_Release service operation to request the release of the MA PDU Session.

-	In step 1, when the H-SMF initiates MA PDU Session Release procedure, the H-SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the PDUSession_Update service operation.

-	In step 5, when the V-SMF initiates MA PDU Session Release procedure, the V-SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the PDU Session Release Request or PDU Session Release Command message.

-	In step 5, if the V-SMF releases the MA PDU Session over a single access network, the V-SMF shall not include "skip indicator" in the Namf_Communication_N1N2MessageTransfer service.

-	In step 5, if the V-SMF releases the MA PDU Session over both accesses and user plane resources are established in both accesses, the V-SMF includes both N1 SM container (PDU Session Release Command) and N2 SM Resource Release request together in the Nsmf_PDUSession_UpdateSMContext or Namf_Communication_N1N2MessageTransfer service so that the UE does not request to activate user plane resources. The V-SMF releases user plane resources of the other access by including N2 SM Resource Release only in Namf_Communication_N1N2MessageTransfer service.

-	In step 5, when the V-SMF provides N1 SM container and/orN2 SM Resource Release request, the V-SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the V-AMF.

-	In step 16, the H-SMF triggers Nsmf_PDUSession_StatusNotify service when the MA PDU Session is released in both accesses or when the MA PDU Session is established over two PLMNs and the MA PDU Session is released over single access. In the latter case, the H-SMF sends the notification over the released access. When the V-SMF received the indication, the V-SMF triggers Nsmf_PDUSession_SMContextStatusNotify service.

* * * * End of Changes * * * *