3GPP TSG-SA2 Meeting #134	S2-1907616

# Sapporo, Japan, June 24th – June 28th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0026 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Deferred 5GC-MT-LR for periodic, triggered and UE available location events for TS 23.273 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-5-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For accordance with 5GC-MT-LR Procedure for the commercial location services described in TS, a second privacy check need to be added into Deferred 5GC-MT-LR for periodic, triggered and UE available location events procedures before (H-)GMLC forwarding the final location information to the client or AF, since target UE user may have defined different privacy settings requirements for different geographical area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add second privacy check before (H-)GMLC forwarding the final location information to the client or AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Deferred 5GC-MT-LR for periodic, triggered and UE available location events procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.3.1	Initiation and Reporting of Location Events

Figure 6.3.1-1 summarizes the initiation and reporting of location events for a deferred 5GC-MT-LR procedure for Periodic, Triggered and UE Available Location Events. The procedure supports mobility of a UE within a VPLMN 5GCN and from a 5GCN to an EPC.

Figure 6.3.1-1: Deferred 5GC-MT-LR for periodic, triggered and UE available location events

1.	The external location services client or the AF (via NEF) sends a request to the (H-)GMLC for location reporting for periodic, triggered or UE available location events. The request is sent as described for step 1 in clause 6.1.2 with the differences described here. The LCS Service Request provides the type of periodic or triggered location reporting being requested and associated parameters. For periodic location, the LCS Service Request includes the time interval between successive location reports, the total number of reports and may include location QoS. For area event reporting, the LCS Service Request includes details of the target area, whether the event to be reported is the UE being inside, entering into or leaving the target area, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates shall be included in event reports (and associated location QoS), and whether only one location report is required or more than one. If the target area is expressed by a local coordinate system or a geopolitical name, the (H-)GMLC shall convert the target area to a geographical area expressed by a shape as defined in TS 23.032 [8]. For motion event reporting, the LCS Service Request includes the threshold linear distance, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates shall be included in event reports (and associated location QoS), and whether only one location report is required or more than one.

1b-1	AF sends the LCS service request to the NEF.

1b-2	The NEF forwards the request to the (H-)GMLC,

2.	The (H-)GMLC may verify UE privacy requirements as for step 2 in clause 6.1.2.

3.	The (H-)GMLC queries the UDM for the AMF address and, in the case of roaming, a V-GMLC address as for step 3 in clause 6.1.2.

NOTE 1:	The HGMLC may also query the HSS of the target UE for a serving MME address as described in clause 9.1.1 of TS 23.271 [4]. The deferred EPC-MT-LR procedure for Periodic and Triggered Location described in clause 9.1.19 of TS 23.271 [4] or the EPC-MT-LR procedure for the UE availability event described in clause 9.1.15 of TS 23.271 [4] may then be performed instead of steps 4-31 - e.g. if the HSS returns an MME address but the UDM does not return an AMF address.

4.	This step is skipped for a non-roaming UE. For a roaming UE, the H-GMLC obtains a V-GMLC address if not received at step 3 and invokes an Ngmlc_Provide Location Request service operation to forward the location request to the V-GMLC as described for step 4 of in clause 6.1.2. The (H-)GMLC also includes a contact address for the (H-)GMLC (Notification Target Address, e.g. a URI) and an LDR reference number (Notification correlation ID) to be used for event reporting at steps 20 and 29.

5.	The (H-)GMLC or V-GMLC invokes the Namf_Location_ProvidePositioningInfo Request service operation to forward the location request to the serving AMF as described for step 5 in clause 6.1.2 and includes the (H-)GMLC contact address and LDR reference number.

6-8.	If the AMF supports a deferred location request, the AMF returns an acknowledgment to the external LCS client, via the (H-)GMLC and, in the case of roaming, the V-GMLC, indicating that the request for deferred location was accepted. The V-GMLC, when used, may optionally release resources for the deferred location request at this point.

9.	If the UE is not currently reachable (e.g. is using eDRX or PSM), the AMF waits for the UE to become reachable.

NOTE 2:	In the event of mobility of the UE to another AMF or to EPC when the UE becomes reachable, the old AMF can return an event indication to the (H-)GMLC as at steps 19 and 20 and may include the address of the new serving AMF or MME if known. If a new serving AMF or MME is not known, the (H-)GMLC can repeat step 3 to query the UDM and HSS for the new AMF or MME address. If a new AMF address is received, the (H-)GMLC can restart the procedure from step 4.

10.	Once the UE is reachable, if the UE is then in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.4 of TS 23.502 [19] to establish a signalling connection with the UE.

NOTE 3:	The AMF may decide to cancel the location request before the UE becomes reachable (e.g. due to lack of resources or due to a timeout on the UE becoming reachable) or when the UE becomes reachable (e.g. if the AMF executes NAS level congestion control on the UE, or for other reasons). The AMF then skips steps 10-18 and proceeds to step 19 to return an indication of location cancelation to the V-GMLC or (H-)GMLC.11-12.	The AMF performs steps 7-8 in clause 6.1.2 to notify the UE of the location request and verify privacy requirements if required by the location request received at step 5 and supported by the UE. The AMF includes in the notification to the UE the type of deferred location request in the case of periodic or triggered location.

13.	The LMF selects an LMF as described for step 6 in clause 6.1.1. The selection may take into account the type of deferred location request (e.g. whether periodic or triggered) and any parameters for the deferred location request (e.g. the number of event reports required and/or the duration).

14.	The AMF invokes the Nlmf_Location_DetermineLocation Request service operation towards the LMF to initiate a request for deferred UE location. For a request for periodic or triggered location, the service operation includes all the information received in step 4 or step 5 including the (H-)GMLC contact address and LDR reference number. For a request for the UE available location event, the (H-)GMLC contact address and LDR reference number are not included. In all cases, the service operation includes an LCS Correlation identifier, the AMF identifier, the serving cell identity, the client type and may include an indication if UE supports LPP,  the required QoS and Supported GAD shapes.

15.	The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 and as described for step 8 in clause 6.1.1. During this step, the LMF may request and obtain the UE positioning capabilities (e.g. which may indicate the type(s) of periodic and triggered location supported by the UE and the access types supported by the UE for event reporting). The LMF may also obtain the UE location - e.g. for a request for the UE available location event or when an initial location is requested for periodic or triggered UE location. For a request for the UE available location event, the LMF skips steps 16 and 17.

16.	If periodic or triggered location was requested, the LMF sends a supplementary services LCS Periodic-Triggered Invoke Request to the UE via the serving AMF by invoking the Namf_Communication_N1N2MessageTransfer service operation. The LCS Periodic-Triggered Location Invoke carries the location request information received from the AMF at step 14, including the (H-)GMLC contact address and LDR reference number. The LCS Periodic-Triggered Location Invoke also includes a deferred routing identifier, which can be the identification of the LMF when the LMF will act as a serving LMF or a default LMF identification otherwise. The LCS Periodic-Triggered Location Invoke may indicate the allowed access types for event reporting at step 25 (e.g. one or more of NR, E-UTRA connected to 5GC, E-UTRA connected to EPC, non-3GPP access connected to 5GC) and may include an embedded positioning messagewhich indicates certain allowed or required location measurements (or a location estimate) at step 24 for each location event reported (e.g. based on the positioning capabilities of the UE obtained in +step 14 and the allowed access types). As part of NAS transport of the LCS Periodic-Triggered Location Invoke from the serving AMF to the UE, the serving AMF includes an immediate routing identifier in the NAS transport message containing an LCS Correlation identifier - e.g. according to clause 6.11.1.

NOTE 4:	The deferred routing identifier may be global (e.g. an IP address, UUID or URI) or may be local. The deferred routing identifier is used for routing in step 25. However, the immediate routing identifier included by the AMF in step 15 is used for routing in step 17.

17.	If the request in step 16 can be supported, the UE returns a supplementary services acknowledgment to the LMF, which is transferred via the serving AMF using the immediate routing identifier and delivered to the LMF using an Namf_Communication_N1MessageNotify service operation.

18.	The LMF invokes the Nlmf_Location_DetermineLocation Response service operation towards the AMF to respond to the request at step 14. For a request for the UE available location event, the response includes any UE location obtained at step 15 and the LMF then releases all resources. For a periodic or triggered location request, the response includes any location obtained at step 15, a confirmation of whether periodic or triggered location was successfully activated in the UE according to steps 16 and 17 and the identification of the LMF in the case of successful activation with a serving LMF; the LMF also retains state information and resources for later steps if the LMF acts a serving LMF. If the UE cannot support the periodic and triggered location request, the service operation returned to the AMF shall include a suitable error cause.

19.	The AMF invokes the Namf_Location_EventNotify service operation towards the V-GMLC for roaming, or (H-)GMLC for non-roaming, and includes any location received at step 18 and, for periodic or triggered location, a confirmation of whether or not periodic or triggered location was successfully activated in the target UE. The V-GMLC, if used, may be the same V-GMLC used in steps 5 and 6 or may be a different V-GMLC. In the case of a different V-GMLC, the AMF includes the H-GMLC contact address and LDR reference number. The AMF also includes the LMF identification if received at step 18. The AMF may then release all resources for the location request and cease support for the procedure.

20.	This step is skipped for a non-roaming UE. For a roaming UE, The V-GMLC forwards the response received at step 19 to the H-GMLC using the H-GMLC contact address received at step 19 (for a different V-GMLC) or received and stored at step 4 (for the same V-GMLC) and includes the LDR reference number and any LMF identification that was received. The V-GMLC may then release all resources for the location request and cease support for the procedure.

NOTE 5:	As an optional optimization for a roaming UE, instead of performing steps 19 and 20, the AMF may invoke the Namf_Location_EventNotify service operation directly towards the H-GMLC (e.g. if a V-GMLC is not used or if the V-GMLC ceases support after step 8).

21.	If the request is for the UE available location event, the response includes any UE location obtained at step 15. And if the privacy check in step 2 indicates that further privacy checks are needed, the (H-)GMLC shall perform an additional privacy check in order to decide whether the (H-)GMLC can forward the location information to the LCS client or AF or send a notification if the result of the privacy check requires the notification and verification based on current location. One example when this additional privacy check is needed is when the target UE user has defined different privacy settings for different geographical locations. When an additional privacy check is not needed, the (H-)GMLC skips steps 22-28.

22.	If the result of privacy checks in step 21 indicates that the notification (and verification) based on current location is needed, and in the case of roaming, the (H-)GMLC shall send a location request to the V-GMLC indicating "notification only",

23.	The (H-)GMLC or V-GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request notification (and verification) based on current location.

24.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.4 of TS 23.502 [19] to establish a signalling connection with the UE.

25.	If the UE supports LCS notification, the AMF sends a notification invoke message to the target UE, indicating the identity of the LCS client the Requestor Identity (if that is both supported and available) and whether privacy verification is required.

26.	Step 26 is the same as step 12.

27.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the (V-)GMLC (or H-GMLC for roaming when the NL3 reference point is not supported) with an indication of the result of notification and verification procedure performed in steps 25-26.

28.	In case of roaming, the V-GMLC forwards an indication of the result of notification and verification procedure to the H-GMLC. For non-roaming scenario, this step is skipped.

29.	The (H-)GMLC forwards the response to the external LCS client or AF (via the NEF). If the location request at step 1 was for the UE available location event, the procedure terminates here and further steps 22-31 are not performed.

30.	For a periodic or triggered location request where steps 16 and 17 were successfully performed, the UE monitors for occurrence of the trigger or periodic event requested in step 16. For the area event or motion event, the UE monitors the requested event at intervals equal to or less than the maximum event sampling interval. An event trigger is detected by the UE when any of the following occur: (i) a requested area event or motion event has been detected and the minimum reporting time interval has elapsed since the last report (if this is not the first event report); (ii) a requested periodic location event has occurred; or (iii) the maximum reporting time for an area event or motion event has expired. When a trigger or periodic event is detected and if the UE is camped on or connected to (or can otherwise access) an access type allowed by the LMF at step 16, the UE proceeds to step 23. If the UE cannot access an allowed access type, the UE may skip reporting the trigger event or may report the trigger event at a later time when an allowed access type becomes available, according to requirements received from the LMF at step 16.

31.	The UE obtains any location measurements or a location estimate that were requested or allowed at step 16.

NOTE 6:	Obtaining a location estimate when requested also applies to the trigger event corresponding to expiration of the maximum reporting interval for an area event or motion event.

32.	The UE performs a UE triggered service request as defined in clause 4.2.3.2 of TS 23.502 [19] if in CM-IDLE state in order to establish a signalling connection with the AMF.

33.	The UE sends a supplementary services event report message to the LMF which is transferred via the serving AMF (which may be different to the original serving AMF for steps 14-16) and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The event report may indicate the type of event being reported (e.g. whether a normal event or expiration of the maximum reporting interval) and may include an embedded positioning message which includes any location measurements or location estimate obtained at step 23. The UE also includes the deferred routing identifier received in step 16 in the NAS Transport message used to transfer the event report from the UE to the AMF. The AMF then forwards the event report to either the serving LMF or any suitable LMF based on whether the deferred routing identifier indicates a particular LMF or any (default) LMF. In case a different LMF than the serving LMF is used, procedure in clause 6.4 is used. The UE also includes the (H-)GMLC contact address, the LDR reference number, whether location estimates are to be reported and if so the location QoS in the event report.

NOTE 7:	When forwarding the event report message to the LMF in step 25, the AMF does not include an LCS Correlation Identifier (because the AMF does not have an LCS Correlation Identifier). To allow the AMF to be aware that an LCS Correlation Identifier is not needed and to distinguish an immediate routing identifier (which includes an LCS Correlation identifier) from a deferred routing identifier (which includes an LMF identification), the deferred routing identifier sent by the LMF in step 15 and possibly reassigned in step 26 can indicate that it is deferred and not immediate. For example, a deferred routing identifier could include a flag or could be selected from a different range of identifiers to the immediate routing identifier sent by the AMF at step 16.

34.	When the LMF receives the event report and if it can handle this event report, the LMF returns a supplementary services acknowledgment for the event report to the UE. The acknowledgment may optionally include a new deferred routing identifier indicating a new serving LMF or a default (any) LMF. If the UE does not receive any response from the LMF after a predefined time, i.e. the current LMF does not support the deferred location request (for temporary or permanent reasons) or due to some radio access failures, the UE may re-send the report one or more times. If the UE sends the repeated event report more than the predefined maximum resending time and the UE still does not receive any response from AMF, the UE shall stop resending the report, then record a corresponding flag to indicate that a report has been sent unsuccessfully. When the UE performs location update and detects the TAI is changed, if the flag has been set, the UE shall send the report to the corresponding AMF, and the flag will be cleared upon a success of the sending.

NOTE 8:	Inclusion of a new deferred routing identifier in the event report acknowledgment at step 26 may be used to change the serving LMF (e.g. if a UE moves into an area or to an access type that is better supported by a different LMF or if the serving LMF is overloaded) or to enable a default LMF to become a serving LMF.

35.	If a location estimate is needed for event reporting, the LMF may perform one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 and as described for step 8 in clause 6.1.1. The LMF then determines the UE location using the location measurements and/or location estimate(s) obtained at this step and/or received at step 25.

36.	In the case of roaming, the LMF selects a V-GMLC (which may be different to the V-GMLC for steps 3-8 and steps 19-21), The LMF then invokes an Nlmf_EventNotify service operation towards the selected V-GMLC or (H-)GMLC with an indication of the type of event being reported, the (H-)GMLC contact address and LDR reference number, the identification of the LMF if this is a serving LMF, and any location estimate obtained at step 35.

NOTE 9:	In the case of roaming, the LMF may select the V-GMLC for step 28 using the NRF service or using configuration information in the LMF or may use the same V-GMLC as for steps 3-8 (e.g. if the LMF acts as a serving LMF and received the V-GMLC address from the AMF as part of step 14).

37.	This step is skipped for a non-roaming UE. For a roaming UE, the V-GMLC invokes an Ngmlc_EventNotify service operation to forward the information received in step 28 (e.g. including the type of event being reported, the LDR reference number and possibly the LMF identification) to the H-GMLC which identifies the periodic and triggered location request from the LDR reference number.

NOTE 10:	As an optional optimization for a roaming UE, instead of performing steps 28 and 29, the LMF may invoke the Nlmf_EventNotify service operation directly towards the H-GMLC.

38.	If UE location information is needed and obtained in step 35, and if the privacy check in step 2 indicates that further privacy checks are needed, the (H-)GMLC shall perform an additional privacy check in order to decide whether the (H-)GMLC can forward the location information to the LCS client or AF or send a notification if the result of the privacy check requires the notification and verification based on current location. One example when this additional privacy check is needed is when the target UE user has defined different privacy settings for different geographical locations. When an additional privacy check is not needed, the (H-)GMLC skips steps 39-45.

39.	If the result of privacy checks in step 38 indicates that the notification (and verification) based on current location is needed, and in the case of roaming, the (H-)GMLC shall send a location request to the V-GMLC indicating "notification only",

40.	The (H-)GMLC or V-GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request notification (and verification) based on current location.

41.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.4 of TS 23.502 [19] to establish a signalling connection with the UE.

42.	If the UE supports LCS notification, the AMF sends a notification invoke message to the target UE, indicating the identity of the LCS client the Requestor Identity (if that is both supported and available) and whether privacy verification is required.

43.	Step 43 is the same as step 12.

44.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the (V-)GMLC (or H-GMLC for roaming when the NL3 reference point is not supported) with an indication of the result of notification and verification procedure performed in steps 42-43.

45.	In case of roaming, the V-GMLC forwards an indication of the result of notification and verification procedure to the H-GMLC. For non-roaming scenario, this step is skipped.

46.	The (H-)GMLC uses the LDR reference number received in step 28 or step 29 to identify the periodic and triggered location request received in step 1 and then sends the type of event being reported and any location estimate to the external LCS client or AF (via the NEF). The (H-)GMLC may also verify UE privacy requirements before reporting the event and any location to the external LCS client or AF.

47.	The UE continues to monitor for further periodic or trigger events as in step 22 and instigates steps 23-30 each time a trigger event is detected.

Editor's note:	It is FFS how to handle the case where UE moves from 5GS to EPS when there is active location service request. Potential approaches to be considered are: if the target UE is unable to access the 5GCN in the original serving PLMN for a repetition of steps 24-25 but is allowed to and can access E-UTRA connected to EPC for the original serving PLMN, the UE may report the location event using event reporting for the Deferred EPC-MT-LR for Periodic and Triggered Location procedure described in clause 9.1.19 of TS 23.271 [4] by instigating steps 13-24 in clause 9.1.19.1 in TS 23.271 [4]. In this case, the UE includes the (H-)GMLC contact address and LDR reference number when sending the MO-LR Invoke at step 14 for clause 9.1.19.1 in TS 23.271 [4] which enables the serving MME for the UE to report the location event to the (H-)GMLC using steps 19-21 in clause 9.1.19.1 in TS 23.271 [4]. Typically, the E-SMLC used for the procedure in clause 9.1.19.1 of TS 23.271 [4] would be different and separate from the LMF for steps 15-17 here, although a proprietary procedure might be used by a serving PLMN to enable the E-SMLC to access (e.g. read and/or modify) state information for the UE in the LMF (e.g. using the (H-)GMLC contact address and LDR reference number as a key to identify the UE state information), which might assist location of the UE by the E-SMLC and/or by the LMF at a later time.

***** End of CHANGE *****