3GPP TSG-SA2 Meeting #134	S2-1907663

# Sapporo, Japan, June 24 – 28, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1632 | rev | 0 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the SMF context procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 06-17-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2 #133, the solution for supporting context transfer between SMF was agreed. The current text indicates that the target (new) SMF is decided by the OAM upon triggering a context transfer procedure. However, the state transition (to become an idle state) of the UE may take several minutes or even hours and the condition of the pre-determined SMF can be changed during this time period. The current text indicates the solution is only applicable between two different SMF sets. However, this contradicts with the general principle captured in TS 23.501, wherein the context transfer can be used within the same NF set (e.g., for the deployment scenario without a common context storage or the fully supported NF set management). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 4.26.5.3 to clarify the target SMF can be selected dynamically in consideration of the actual network status with the use of the target SMF set ID. Updated 4.26.5.1 to clarify that the SMF context transfer procedure can be used between two SMFs within the same SMF set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of reliable SMFs management becomes inefficient and restrictive. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26.5.1, 4.26.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID or SMF set ID.

2.	From Old SMF to AMF Nsmf_PDUSession_StatusNotify (SMF transfer indication, New SMF ID or SMF set ID)

3.	The AMF waits until UE becomes IDLE or RRC inactive.

Editors note:	It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.

4.	AMF uses New SMF ID or SMF set ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 1 is used. If SMF set ID is provided, the AMF selects a new SMF within the SMF set in consideration of the conditions such as capacity and load status of the SMF.

5.	From New SMF to Old SMF SMF Nsmf_PDUSessionContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	The Old SMF releases the N4 session.

Editors note:	It is FFS if steps 6 and 9 can be optimised or changed to a modification of the N4 session.

7.	From Old SMF to New SMF Nsmf_PDUSessionContextRequest response (SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet.

8.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF sets up a new policy association towards PCF

9.	New SMF establishes N4 session. The parameters from step 7 and, if applis, step 8 are used.

10.	From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

11.	New SMF registers to UDM,

12.	New SMF subscribes to subscription changes for the UE

13.	[Conditional] Old SMF removes its policy association with PCF (triggered by step 7) Any changes to the QoS rules need to be sent to the UE when it becomes active. After Step 13 (Step 7 if there is no policy assiciation) the old SMF can relase any internal resources for the PDU session.

SECOND CHANGE

4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts) and/or between different SMF instances in the same SMF set.