SA WG2 Meeting #134 	S2-1907668

# June 24-28th, 2019 ; Sapporro, Japan		                                    (revision of S2-1907668)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0034 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordination between PCF and ACS (for FN RG) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not described how policies from the PCF and policies from the ACS map in case both are needed (especially for FN-RG). |  |  |  |  |  |  |  |  |
| Summary of change: |  | ACS discovery has been upgraded (aligning the vocabulary, avoiding repetition of BBF specifications, …) An AF can configure the 5GC with the mapping between service multiplexer (e.g. VLAN) used on the Wireline access and PDU sessions parameters (DNN, S-NSSAI). This is required for FN-RG data sessions. The ACS configures RG with the mapping from applications (e.g. internet / IMS Voice / IPTV) to data sessions over the wireline access and thus configures the mapping between applications (e.g. internet / IMS Voice / IPTV) and a service multiplexer (e.g. VLAN) used on the Wireline access. When a W-AGF requests PDU Sessions on behalf of a FN-RG it needs to map the service multiplexer (e.g. VLAN) used on the Wireline access with PDU sessions parameters (DNN, S-NSSAI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wireline service over 5GC may not work if the service multiplexer (e.g. VLAN) used on the Wireline access does not map to proper PDU sessions parameters (DNN, S-NSSAI). For example, a configuration mismatch may lead to “random” usage of PDU sessions parameters (DNN, S-NSSAI) by the W-AGF. In that case it might happen that the traffic from the RG (e.g. IMS signalling to support IMS Voice) cannot reach the target recipient DN (e.g. DN dedicated to IMS). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.1 ; 9.6.2 ; 9.6.3 ; 9.6.X(new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 9.6	Configuration and Management from ACS

### 9.6.1	General

Once the 5G-RG connects to 5GC, the 5G-RG shall establish a PDU session for interaction with the ACS to support the functionalities as described in BBF TR-069 [18] or in BBF TR-369 [19].

Editor's note:	The ACS may also send QoS object to the RG including QoS policy (L2 and L3 marking, see details in BBF TR-198 [30]) to some specific flows. Whether the 5G-RG receives both the QoS object from the ACS and the QoS rule from the 5GC, and the coordination between them are FFS.

### 9.6.2	ACS Discovery

The ACS addressing information may be associated to the RG subscription in the UDM / UDR. In this case the ACS addressing information may be provided to the RG with at least one of the following methods:

-	via DHCP interaction if the RG sends DHCP signalling indicating a request for ACS addressing information. The RG sends a DHCPv4 request including a request for ACS addressing information and receives ACS addressing information from the DHCP server as specified in BBF TR-069 [18] clause 3.1 for ACS Discovery or in BBF TR-369 [19] R-DIS.1 and R-DIS.2.

-	(only for 5G-RG) during the PDU session establishment procedure via PCO (protocol Configuration Option) sent in N1 SM message if the 5G-RG has asked to be provided with ACS addressing information via PCO.

The ACS addressing information (e.g. URL of the ACS) is defined in BBF TR-069 [18] or in BBF TR-369 [19].

If the RG performs ACS discovery via DHCP process and the SMF is not the DHCP server (e.g. in the case of Ethernet PDU session), the ACS URL is provided by the external DHCP server. In this case, the whole process is transparent to the 5GC and the 5GC is not aware of the ACS addressing information. If the RG performs ACS discovery via DHCP process and the SMF is the DHCP server the ACS addressing information is provided by SMF as part of DHCP process.

In case the SMF is to provide ACS addressing information to the RG (via PCO or DHCP), it gets this ACS addressing information from SMF subscription data.

The request of ACS addressing information via PCO or via DHCP are mutually exclusive.

The RG may be pre-configured with an ACS addressing information.

The 5G-RG shall consider the ACS addressing information received with the following descending priority order:

1)	ACS addressing information received during the DHCP process.

2)	(for 5G-RG only) ACS information received during the PDU session establishment procedure from SMF PCO.

3)	The pre-configured ACS addressing information in the RG.

### 9.6.3	ACS Information Configuration by the 3rd party

The ACS information may be configured by a 3rd party AF to the 5GC per subscriber when the SMF is to provide ACS addressing information to the RG.. Subsequently, the ACS discovery via PCO or via DHCP with the DHCP server in the SMF may apply as described in clause 9.6.2.

Figure 9.6.3-1: ACS addressing information configuration procedure

The ACS information configuration procedure enables the 3rd party AF to configure the ACS addressing information  (e.g. URL or IP address) to the 5GC.

1.	The 3rd party AF provides the ACS addressing information  , in the Nnef_ParameterProvision_Update Request to the NEF as in step 1 of TS 23.502 [3] figure 4.15.6.2-1.

2.	As in step 2 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the ACS addressing information  .

3.	As in step 3 and 4 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the ACS addressing information  .

4.	As in step 5 of TS 23.502 [3] figure 4.15.6.2-1.

5.	As in step 6 of TS 23.502 [3] figure 4.15.6.2-1.

6.	As in step 6 of TS 23.502 [3] figure 4.15.6.2-1 in order to update SMF with with ACS addressing information ;

NEXT CHANGE ** all text is NEW**

### 9.6.X	Configuration by a 3rd party of the mapping between Wireline traffic and PDU Sessions

The ACS configures RG with the mapping from applications (e.g. internet / IMS Voice / IPTV) to data sessions over the wireline access and thus configures the mapping between applications (e.g. internet / IMS Voice / IPTV) and a service multiplexer (e.g. VLAN ID) used on the Wireline access.

When a W-AGF requests PDU Sessions on behalf of a FN-RG it needs to map the service multiplexer (e.g. VLAN ID) used on the Wireline access with PDU sessions parameters (DNN, S-NSSAI).

NOTE1: 	The service multiplexers (e.g. VLAN ID) used on the Wireline access can depend on local wireline configurations and thus vary based on the actual access network used by RG(s).

The PCF provides URSP to the W-AGF to guide this mapping; in order to allow the PCF to build URSP that align with the service multiplexer (e.g. VLAN ID) used on the Wireline access, the PCF uses policy data that an AF may have had written in the UDR per the procedure described in this clause.

NOTE2: 	It is out of scope of this document to determine whether the AF in Figure 9.6.X-1 corresponds to the ACS or to another entity such as an OAM controller.

Figure 9.6.X-1: 5GC data session configuration to support Wireline access service

The 5GC data session configuration procedure enables an AF to configure the 5GC with a mapping from service multiplexers (e.g. VLAN ID) used on the Wireline access and PDU sessions parameters (DNN, S-NSSAI).

- The AF provides 5GC data session configuration, in a Nnef_ParameterProvision_Update Request sent to the NEF as in step 1 of TS 23.502 [3] figure 4.15.6.2-1. The information provided by the AF shall include a 5GC subscription identifier and may include a list of 5GC data session configuration information elements defined in Table 9.6.X-1.

The 5GC subscription identifier may correspond to a Line ID based SUPI in case the configuration relates to FN-RG

2.	As in step 2 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the 5GC data session configuration. The usage of NEF is optional as the AF is assumed to be part of the operator’s network.

3.	As in step 3 and 4 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the 5GC data session configuration. The UDM may update Policy Data related with URSP and update Subscription Data / Session Management Subscription data with subscribed (DNN, S-NSSAI)

4.	As in step 5 of TS 23.502 [3] figure 4.15.6.2-1.

5.	As in step 6 and 7 of TS 23.502 [3] figure 4.15.6.2-1. If Step 7 of TS 23.502 [3] applies, it applies only to SMF subscription data.

.

Table 9.6.X-1: 5GC data session configuration information element

| service multiplexer (e.g. VLAN ID) used on the Wireline access | Correspondig DNN | Corresponding S-NSSAI |
| --- | --- | --- |

The possible types of service multiplexers used on the Wireline access are defined by BBF and Cbale Labs specifications. As a minimum it includes the possibility to use a VLAN id as service multiplexers used on the Wireline access

NEXT CHANGE

END OF CHANGES