3GPP TSG-SA2 Meeting #134	S2-1907770

# Sapporo, Japan, June 24 - 28, 2019	(revision of S2-1907423)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1681 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Handling for URLLC KI#7 and V2X Communication Over Uu Reference Point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone (based on CR 1440 rev 4 from Huawei, HiSilicon, Telecom Italia in S2-1907423) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC, eV2XARC |  |  |  |  |  | Date: |  |  | 2019-05-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR 23.786 concluded to take Solution 27 as the basis for normative work. This CR updates CR 1440r4 to also cover Solution XX (S2-1907756) for URLLC KI#7 on automatic resotration of QoS. The URLLC KI 7 updates cover handover situtations which are missing from CR 1440r4 (and handover is obviously needed for V2X). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the QoS flow mapping and the Notification control clauses to introduce the optional Alternative QoS profile(s) to be sent to the RAN. Added the new clause 5.7.1.x to describe what the Alternative QoS profile(s) are and how they are used. - Allowed a best effort Alternative QoS. - Removed the unexplained requirement for 2 alternative QoS profiles. - Added handover functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements for QoS handling in case the QoS parameters of the QoS profile cannot be guaranteed by the NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.5, 5.7.1.x (new), 5.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR overlaps with CR 1440. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Updates compared to S2-1907423 (CR 1440r4) are in green highlights. |  |  |  |  |  |  |  |  |

# * * * First change * * * *

#### 5.7.1.5	QoS Flow mapping

The SMF performs the binding of SDFs to QoS Flows based on the QoS and service requirements (as defined in TS 23.503 [45]). The SMF assigns the QFI for a new QoS Flow and derives its QoS profile, corresponding UPF instructions and QoS Rule(s) from the PCC rules and other information provided by the PCF.

When applicable, the SMF provides the following information to the (R)AN:

-	QFI;

-	QoS profile;

-	optionally, Alternative QoS Profile List as described in clause 5.7.1.x.

The SMF provides the following information to the UPF enabling classification, bandwidth enforcement and marking of User Plane traffic (the details are described in clause 5.8):

-	a DL PDR containing the DL part of the SDF template and an UL PDR containing the UL part of the SDF template;

-	the PDR precedence value (see clause 5.7.1.9) for both PDRs is set to the precedence value of the PCC rule;

-	QoS related information (e.g. MBR for an SDF, GFBR and MFBR for a GBR QoS Flow) as described in clause 5.8.2;

-	the corresponding packet marking information (e.g. the QFI, the transport level packet marking value (e.g. the DSCP value of the outer IP header);

-	for the DL PDR, optionally, the Reflective QoS Indication.

For each SDF, when applicable, the SMF generates an explicitly signalled QoS rule (see clause 5.7.1.4) according to the following principles and provides it to the UE together with an add operation:

-	A unique (for the PDU Session) QoS rule identifier is assigned;

-	The QFI in the QoS rule is set to the QFI of the QoS Flow to which the PCC rule is bound;

-	The Packet Filter Set of the QoS rule is generated from the UL SDF filters and optionally the DL SDF filters of the PCC rule (but only from those SDF filters that have an indication for being signaled to the UE, as defined in TS 23.503 [45]);

-	The QoS rule precedence value is set to the precedence value of the PCC rule for which the QoS rule is generated;

-	for a dynamically assigned QFI, the QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR, Averaging Window, see TS 24.501 [47]) are signalled to UE in addition to the QoS rule(s) associated to the QoS Flow. The QoS Flow level QoS parameters (i.e. GFBR and MFBR) of an existing QoS Flow may be updated based on the MBR and GBR information received in the PCC rule (MBR and GBR per SDF are not provided to UE over N1).

Changes in the binding of SDFs to QoS Flows as well as changes in the PCC rules or other information provided by the PCF can require QoS Flow changes which the SMF has to provide to (R)AN, UPF and/or UE. In case of changes in the explicitly signalled QoS rules associated to a QoS Flow, the SMF provides the explicitly signalled QoS rules and their operation (i.e. add/modify/delete) to the UE.

NOTE 1:	The SMF cannot provide, update or remove pre-configured QoS rules or UE derived QoS rules.

The principle for classification and marking of User Plane traffic and mapping of QoS Flows to AN resources is illustrated in Figure 5.7.1.5-1.

Figure 5.7.1.5-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources

In DL, incoming data packets are classified by the UPF based on the Packet Filter Sets of the DL PDRs in the order of their precedence (without initiating additional N4 signalling). The UPF conveys the classification of the User Plane traffic belonging to a QoS Flow through an N3 (and N9) User Plane marking using a QFI. The AN binds QoS Flows to AN resources (i.e. Data Radio Bearers of in the case of 3GPP RAN). There is no strict 1:1 relation between QoS Flows and AN resources. It is up to the AN to establish the necessary AN resources that QoS Flows can be mapped to, and to release them. The AN shall indicate to the SMF when the AN resources onto which a QoS Flow is mapped are released.

If no matching DL PDR is found, the UPF shall discard the DL data packet.

In UL:

-	For a PDU Session of Type IP or Ethernet, the UE evaluates UL packets against the UL Packet Filters in the Packet Filter Set in the QoS rules based on the precedence value of QoS rules in increasing order until a matching QoS rule (i.e. whose Packet Filter matches the UL packet) is found.

-	If no matching QoS rule is found, the UE shall discard the UL data packet.

-	For a PDU Session of Type Unstructured, the default QoS rule does not contain a Packet Filter Set and allows all UL packets.

NOTE 2:	Only the default QoS rule exist for a PDU Session of Type Unstructured.

The UE uses the QFI in the corresponding matching QoS rule to bind the UL packet to a QoS Flow. The UE then binds QoS Flows to AN resources.

# * * * Second change * * * *

#### 5.7.1.x	Alternative QoS Profile List

The Alternative QoS Profile List can be provided for a GBR QoS Flow with Notification control enabled. The SMF shall provide, in addition to the (desired) QoS profile, the Alternative QoS Profile List to the NG-RAN if the corresponding PCC rule contains the related information (as described in TS 23.503 [45]). The Alternative QoS Profile List contains at least one element. The elements are  in an ordered list with decreasing priority. Each element of the Alternative QoS Profile List includes the following parameters:

-	5QI,

and optionally

-	UL/DL GFBR,

-	UL/DL MFBR.

If the NG-RAN has received and supports the Alternative QoS Profile List for a GBR QoS Flow the following applies for the NG-RAN behaviour for notification control in addition to what is described in clause 5.7.2.4.

-	In case the QoS profile cannot be guaranteed by the NG-RAN, the NG-RAN shall, before sending a notification that the GFBR can no longer be guaranteed towards the SMF, check the elements in the Alternative QoS Profile List in order of priority and determine if it can guarantee any of them. If the NG-RAN can guarantee the QoS parameters of one of them, then the NG-RAN shall indicate a reference to that element in the notification that the GFBR can no longer be guaranteed to the SMF. If the QoS which the NG-RAN can still guarantee does not match to any element of the Alternative QoS Profile List, then the NG-RAN should provide the values for the UL/DL GFBR, the PDB and the PER parameters it can guarantee in the notification (that the GFBR can no longer be guaranteed) to the SMF.

-	Whenever the QoS profile is changed the NG-RAN shall compare the new QoS profile with the first element of the Alternative QoS Profile List.

-	As long as the two are different, the NG-RAN shall be checking whether the QoS described by the first element in the Alternative QoS Profile List can be guaranteed again;

-	If the two are equal, the NG-RAN shall not perform the check.

-	If the check of whether the QoS described by the first element in the Alternative QoS Profile List can be guaranteed is active and successful, then the NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again, and indicate a reference to the first element of the Alternative QoS Profile List.

- as described in clause 5.7.2.4, the NG-RAN continues to attempt to restore the QoS received in the (desired) QoS profile.

NOTE:	It is assumed that some hysteresis is applied before determining that the QoS can be guaranteed again. Consequently, frequent signalling of GFBR can be guaranteed again followed by GFBR can no longer be guaranteed is not expected. Details are up to NG-RAN implementation.

When the source NG-RAN node attempts handover, it indicates the (desired) QoS Profile and the Alternative QoS Profile List to the target NG-RAN node. The target NG-RAN node attempts to admit the desired QoS profile even if the Alternative QoS proile is currently in use. The Target NG-RAN node informs the source NG-RAN node whether the desired QoS profile or one of the Alternative QoS Profiles can be “permanently” admitted (or whether the flow is completely rejected).

If the Target NG-RAN node cannot admit the Desired QoS Profile, the Source NG-RAN node may attempt handover with other Target RAN nodes. Ultimately, the Source NG-RAN node selects one Target RAN node and cancels the handover requests on any other Target RAN nodes.

When the UE completes the handover, the New RAN Node provides information so that the Core Network knows whether the Desired Qos Profile or which specific Altenative QoS Profile is in use (or if the flow has been released).

If an Alternative QoS profile is in use on the New RAN node, it attempts to restore the Flow to its desired QoS profile.

# * * * Third change * * * *

#### 5.7.2.4	Notification control

The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR can no longer be guaranteed, NG-RAN shall send a notification towards SMF and keep the QoS Flow (i.e. while the NG-RAN is not delivering the requested GFBR for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to guarantee the GFBR again. If the NG-RAN has received and supports the Alternative QoS Profile List for this GBR QoS Flow additional behaviour is described in clause 5.7.1.x.

NOTE 1:	NG-RAN can decide that the GFBR can no longer be guaranteed based on, e.g. measurements like queuing delay or system load.

Upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed, the SMF may forward the notification to the PCF, see TS 23.503 [45]. 5GC may initiate N2 signalling to modify or remove the QoS Flow.

When the NG-RAN determines that the GFBR can be guaranteed again for a QoS Flow (for which a notification that the GFBR can no longer be guaranteed has been sent), the NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed whenever necessary. If the NG-RAN has received and supports the Alternative QoS Profile List for this GBR QoS Flow additional behaviour is described in clause 5.7.1.x.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the GFBR can be guaranteed again and therefore a frequent signalling of GFBR can be guaranteed again followed by GFBR can no longer be guaranteed is not expected.

NOTE 3:	If the QoS Flow is modified, the NG-RAN restarts the check whether the GFBR can no longer be guaranteed according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification that the GFBR for a QoS Flow can no longer be guaranteed. The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. The accepted QoS Flows are included in the N2 Path Switch Request or N2 Handover Request Acknowledge message from the NG-RAN to the AMF. Clause 5.7.1.x. provides additional information about the handling of the Aletrnatiev QoS Profile List. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that GFBR can be guaranteed again for this QoS Flow. After the handover is successfully completed, the Target NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed for such a QoS Flow whenever necessary. If the SMF has previously notified the PCF that the GFBR can no longer be guaranteed and the SMF does not receive an explicit notification that the GFBR can no longer be guaranteed for that QoS Flow from the Target NG-RAN within a configured time, the SMF shall notify the PCF that the GFBR can be guaranteed again.

# * * * End of changes * * * *