3GPP TSG SA2 Meeting #134	S2-1907790

# Sapporo, Japan, June 24th - 28th , 2019               (merged S2-1907118 and S2-1906986)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0011 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Functions and entities - PCC clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 18-06-2019 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description of the Network Functions and entities in the PCC clause is empty, including an Editor´s note- |  |  |  |  |  |  |  |  |
| Summary of change: |  | # A description of the session and non session management functionality and support by the NFs for wireline non-3GPP access is included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description of the impacts in the PCC network functions to support wireline non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2, 9.2.x (new), 9.2.y (new), 9.2.z (new), 9.2.w (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 9.2	Network Functions and entities

### 9.2.x	General

This clause specifies the delta related to Network Function and entities defined in TS 23.503 [4] clause 6.2 for 5G-RG and FN-RG.

The functional description of the NEF, NWDAF, UDR and CHF applies as described in TS 23.503 [4].

### 9.2.x	Policy Control Function (PCF)

The PCF provides session management policy control for single access PDU sessions over non 3GPP wireline and multiaccess PDU sessions over both 3GPP and non 3GPP wireline access.

The session management related functionality defined in TS 23.503 [4], clause 6.2.1. applies for 5G-RG and FN-RG, with the following modifications for W-5GAN:

- Determination of Maximum Packet Loss Rate for UL/DL does not apply.

- QoS Notification Control does not apply.

- The Authorized Session-TMBR is provided to the SMF for a single PDU session over wireline.

NOTE: No requirements to support MPS or Mission Critical Services over wireline non 3GPP access are defined in this Release.

The non-session management related functionality defined in TS 23.503 [4], clause 6.2.2 applies for 5G-RG and FN-RG, with the following modifications for W-5GAN:

-	the UE-AMBR control by the Visited Network does not apply.

-	the Service Area Restrictions for a FN-BRG does not apply.

-	the 5G-RG and FN-RG replaces the UE.

-	the PCF provides Access and mobility related policy control as described in clause 9.5.1.

-	the PCF provides UE access selection and PDU session selection

-	the PCF provides the UE access selection and PDU Session selection related policy control as defined in 9.5.2.

The policy control subscription data defined in 23.503 applies for 5G-RG and FN-RG connected via W-5GAN access, except for the definition of MPS data for a 5G-RG or FN-RG that is not applicable in this Release.

The policy control subscription data defined in 23.503 applies for a 5G-RG and FN-RG connected via W-5GAN, except for the definition of MPS data for a 5G-RG or FN-RG that is not applicable in this Release.

The V-PCF anf H-PCF functionality does not apply for session and non-session policy control for 5G-RG and FN-RG users in this Release.

### 9.2.y	Session Management Function (SMF)

The SMF enforces policy decisions related to service data flow detection, authorized QoS, charging, gating, traffic usage reporting, packet routing and forwarding and traffic steering for single access PDU session over W-5GAN and multiaccess PDU sessions over W-5GAN and 3GPP as defined in TS 23.503 [4] in clause 6.2.2 with the following modifications for W-5GAN:

- Reporting RAN/NAS Release Cause over wireline is not supported.

- The Maximum Packet Loss Rate for UL and DL is not forwarded to the wireline non-3GPP access.

- The SMF receives the Authorized-Session TMBR in the PDU Session related information. The SMF ensures that the Authorized-Session TMBR for a PDU Session is enforced for bandwidth policing at the UPF(s).

### 9.2.z	Application Function (AF)

The AF requests for policy control functionality described in TS 23.503 [4]. Clause 6.2.3 applies with the following clarification for W-5GAN:

- Indication that the QoS targets can no longer (or can again) be fulfilled is not supported.

NOTE: No requirements to support MPS or Mission Critical Services over wireline non 3GPP access are defined in this release.

### 9.2.w	Access and Mobility Management Function (AMF)

The policy control related functionality defined in TS 23.503 [4]. applies, with the clarification that the UE-AMBR control by the visited network is only applicable for a 5G-RG registered over 3GPP access.

End of changes