3GPP TSG-SA WG2 Meeting #134	S2-1907865

# Sapporo, Japan, 24 - 28 June 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1493 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CN identifier coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-06-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is beneficial for deployments to avoid the need to coordinate the CN identifiers between AMFs and MMEs To avoid that then NG-RAN/eNB needs to be aware of which GUAMI/GUMMEI values are mapped and which are native during configuration of the NG-RAN/gNB and the AMFs/MMEs. Also, there should be no need to coordinate GUMMEI values originating from 3G as mapped GUMMEIs from the GUMMEIs mapped from 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If MME needs to send both served GUMMEIs mapped from 3G and mapped from 5G in release 16, it sends them to eNB by separating the GUMMEIs mapped from 5G from other mapped GUMMEIs. If the AMF needs to send to NG-RAN both native and mapped from EPS served GUAMIs in release 16, it sends them by separating the natives from those mapped from EPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to avoid address/identifier coordination between AMFs and MMEs and between AMFs and SGSNs . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.X (new), 4.11.5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

##### 4.11.0a.X	Network Configuration

To avoid the need for identifier coordination between MMEs, AMFs and SGSNs, the MME provides to the eNB its served GUMMEIs by separating the values between native MME GUMMEI values, values mapped from AMF, and values mapped from SGSN.

*** NEXT CHANGE ***

##### 4.11.5.X	Network Configuration

To avoid the need for identifier coordination between AMFs and MMEs, the AMF provides to the NG-RAN its served GUAMIs by separating the values between native AMF values and the values mapped from MME.

*** END OF CHANGES ***