3GPP TSG-SA WG2 Meeting #134	S2-1907922

# Sapporo, Japan, 24 June – 28 June 2019 	(revision of S2-197894)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0015 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of URSP policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document addresses the applicability of URSP policy to FN-RG and the UE policy distribution for both 5G-RG and FN-RG. For UE policy applicability to FN-RG the following principles are adopted: - ANDSP is not applicable to device connected via W-5GAN - The URSP policy for FN-RG are not stored in subscription information - Optionally the URSP may be stored by W-AGF for FN-RG that might be connected. How is done is outside the scope of this TS. - If the URSP for the FN-RG is present in W-AGF (e.g. pre-configured or recived from PCF), then the W-AGF shall use as defined for a UE with URSP. - If the URSP for the FN-RG is not present in W-AGF, than the W-AGF acts based on local configuration as defined for a UE without URSP. - It is assumed that the FN-RG configuration (provided via TR-069/TR-369), the URSP rules and the local configuration in the W-AGF are consistent with each other. If the W-AGF detects contrasting requirements based on URSP, local configuration, or requests from the FN-RG, then the URSP rules take precedence since they are considered the most updated and aligned to the current 5G system conditions. UE Policy procedures defined in TS 23.503 clause 6.1.2.2 are applicable as follow: - roaming is not applicable in this release It is proposed to delete the “Editor's note: The applicability and potential enhancement in other part of URSP for 5G-RG is FFS.” Since so far there is no enhancement identified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the EN in clause 9.5.2.1 Add specification in clause 9.5.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unspecified the usage of USRP policy for FN-RG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.2.1, 9.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

If the PCF provides the URSP policy to the 5G-RG, the PCF should neither include NSWO indication nor any ANDSP policies. The 5G-RG shall ignore any NSWO indication or any ANDSP policies if received from the 5GC. The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in TS 23.503 clause 6.1.2.2 are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

#### 9.5.2.2	FN-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6 for 5G-RG.

If the PCF provides the URSP rules related to FN-RG to the W-AGF, the PCF should not include NSWO indication . The PCF should not provide ANDSP policies. The W-AGF shall ignore any NSWO indication or any ANDSP policies if received from the 5GC.

The W-AGF shall use the URSP policy as specified in TS 23.503 [4] with the following modifications:

-	Traffic descriptor

-	the Application Descriptor is not applicable

-	the DNN is not applicable

-	The Connection Capabilities Descriptor is not applicable

NOTE 1: The FN-RG initiates the W-5GAN session with the W-AGF, for example PPPoE, and consequently the W-AGF does not receive any indication of the application used for that session (e.g. wheter it used for web browsing or for any specific application) and any DNN indication from the application, hence the policy including the Application Descriptors and/or DNN will never match the traffic

If the PCF sends UE policy (e.g. URSP), the W-AGF shall store it for the duration that FN-RG is registered. When the FN-RG is deregistered, the UE policy can be removed. Whether it is done immediately, or after a certain period (e.g. for quick recovery from disconnection or fault), or stored permanently it is left to implementation and is out of the scope of this TS.

If the URSP for the FN-RG are present in W-AGF (e.g. pre-configured or received from PCF) the W-AGF shall use them as as defined for a UE with URSP.

If the URSP for the FN-RG are not present in W-AGF, the W-AGF acts based on local configuration, as defined for a UE without URSP.

It is assumed that the FN-RG configuration (provided via TR-069/TR-369), the URSP rules and the local configuration in the W-AGF are consistent with each other. If the W-AGF detects conflicting requirements based on URSP, local configuration, or requests from the FN-RG, then the URSP rules take precedence since they are considered the most updated and aligned to the current 5G system conditions.

UE Policy procedures defined in TS 23.503 clause 6.1.2.2 are applicable with the following modification:

-	Roaming is not applicable to W-5GAN access in this release of specification.

**** End of First Change ****