3GPP TSG-SA2 Meeting #134	S2-1908047

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1393 | rev | 4 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Context Transfer Support via indirect Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Deutsche Telekom, Telecom Italia, Sprint, AT&T, Verizon, Telstra, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#132, Service Context Transfer procedures (S2-1904843) were approved in TS23.502. Indirect transfer shall support Service Context Transfer with any NF/NF service. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document introduces indirect service context transfer via a context handle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service Context Transfer is not fully specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26, 5.2.8.2.10, 5.2.8.2.11, 5.2.X(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial Rev To add the new service in UDSF service Rev 1-2 Use of CTSF for context data transfer Rev 3 1) Additions to sections 4.26, 5.2.8.2.10, 5.2.8.2.11: - to align with CTSF, i.e. description of indirect context transfer - Adressed potential race conditions (validity time) 2) Chagnes to endorsed text in section 5.2.X in yellow: - clarified support for different Service Context Types - added validity time - typos Rev 4 Limited scope to indirect transfer without mention of CTSF Text covering S2-1907542, S2-1907543: Role of O&AM also covering bulk transfer |  |  |  |  |  |  |  |  |

* * * * First change * * * *

## 4.26	Network Function/NF Service Context Transfer Procedures

### 4.26.1	General

Network Function/NF Service Context Transfer Procedures allow transfer of Service Context of a NF/NF Service from a Source NF/NF Service Instance to the Target NF/NF Service Instance, e.g. before the Source NF/NF Service can gracefully close its NF/NF Service. If SMF sets are deployed this applies from an SMF instance within an SMF set to an SMF instance of another SMF set. Context transfer can be performed directly or indirectly via a handle of the context (e.g. an URI pointing to the context) defined by Source NF/NF Service Instance. Indirect transfer does not define a new data model for the Service Context but rather a different means of retrieving the same data. It also assumes that only one target NF/NF Service can get access to a certain context and that target NF/NF Service retrieves data by simply accessing the context handle.

NOTE: Whether Source NF/NF Service and the context handle are co-located is implementation-dependent.

Source NF / OA&M system determines when to trigger a context transfer (e.g. SM Context transfer) and how many context transfers to perform. For direct and indirect transfer, OA&M / Target NF/NF Service may throttle the context transfer(s), e.g. when the context handle(s) are retrieved, when the target NF(s) start processing the context transfer(s).

### 4.26.2	NF/NF Service Context Transfer Push Procedure

Figure 4.26.2-1: NF/NF Service Context Push procedure (direct transfer)

1.	When triggered, the Source NF/NF Service acting as NF Service Consumer its Context (e.g. UE Context or SM context) to the Target NF/NF Service acting as NF Service producer. This may trigger several other procedures that ensure all necessary NF/NF Services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the response indicating the result of the operation (successful or not successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumer of the Source NF/NF Service, which e.g. can be shut down gracefully. The response contains the new location of the transferred context.

NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

Compared to direct transfer, if the Context is transferred indirectly, the Service Push Service Context Request contains a handle pointing to the context (Step 1), which Target NF/NF Service uses to retrieve the context (Steps 2, 3).

Figure 4.26.2-2: NF/NF Service Context Push procedure (indirect transfer)

### 4.26.3	NF/NF Service Context Transfer Pull procedure

Figure 4.26.3-1: NF/NF Service Context Pull procedure (direct transfer)

1.	When triggered, the Target NF/NF Service as an NF Service Consumer requests a Context (e.g. UE context or SM context) from the Source NF as a NF Service Producer. This may trigger several other procedures that ensure all necessary NF/NF services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the Context and if the operation was a successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumer of the Source NF/NF Service, which e.g. can be shut down gracefully.

NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

Compared to direct transfer, if the Context is transferred indirectly, the Service Pull Service Context Response contains a handle pointing to the context (Step 2), as well as any other information necessary so that Target NF/NF Service can retrieve it (Steps 3, 4).

Figure 4.26.3-2: NF/NF Service Context Pull procedure (indirect transfer)

### 4.26.4	Context Transfer due to decommissioning

In the case of decommissioning, the Old NF may inform NRF that it is about to be decommissioned. NRF will in this case not include the NF profile of the Old NF in any discovery result from the NF/NF service discovery procedure. The NRF will also notify any consumers subscribed to changes to this resource.

### 4.26.5	SMF Service Context Transfer procedures

#### 4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

#### 4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_StatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	From Old SMF to AMF Nsmf_PDUSession_StatusNotify (SMF transfer indication, New SMF ID

3.	The AMF waits until UE becomes IDLE or RRC inactive.

Editors note:	It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.

4.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 1 is used.

5.	From New SMF to Old SMF SMF Nsmf_PDUSessionContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	The Old SMF releases the N4 session.

Editors note:	It is FFS if steps 6 and 9 can be optimised or changed to a modification of the N4 session.

7.	From Old SMF to New SMF Nsmf_PDUSessionContextRequest response (SM Context or SM Context handle). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet.

8.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF sets up a new policy association towards PCF

9.	New SMF establishes N4 session. The parameters from step 7 and, if applis, step 8 are used.

10.	From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

11.	New SMF registers to UDM,

12.	New SMF subscribes to subscription changes for the UE

13.	[Conditional] Old SMF removes its policy association with PCF (triggered by step 7) Any changes to the QoS rules need to be sent to the UE when it becomes active. After Step 13 (Step 7 if there is no policy assiciation) the old SMF can relase any internal resources for the PDU session.

* * * * Next change * * * *

##### 5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO), or during mobility procedure with I-SMF changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, PDU Session ID (include PDU Session ID when available).

Output, Required: One of the following:

-	SM Context Container

-	SM Context handle.

Output, Optional:Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requrested, e.g. PDN Connection Context ,5G SM Context or both. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context that may be transferred between I-SMF(s) or between V-SMF(s)

| Field |  | Description |  |
| --- | --- | --- | --- |
| SUPI |  | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |  |
| Trace reference |  | Identifies a record or a collection of records for a particular trace. |  |
| Trace type |  | Indicates the type of trace |  |
| OMC identity |  | Identifies the OMC that shall receive the trace record(s). |  |
| S-NSSAI |  | The requested S-NSSAI for the serving PLMN to the PDU Session. |  |
| HPLMN S-NSSAI |  | Requested S-NSSAI for the HPLMN to the Home-Routed PDU Session |  |
| Network Slice Instance id |  | The network Slice Instance information for the PDU Session |  |
| DNN |  | The associated DNN for the PDU Session. |  |
| AMF Information |  | The associated AMF instance identifier and GUAMI. |  |
| Access Type |  | The current access type for this PDU Session. |  |
| RAT Type |  | RAT Type used by the UE. |  |
| PDU Session ID |  | The identifier of the PDU Session. |  |
| H-SMF Information |  | The associated H-SMF identifier and H-SMF address for the HR PDU Session.( applies only for a V-SMF) |  |
|  |  |  | For each QoS Flow in the PDU Session: |
| 5G QoS Identifier (5QI) |  | Identifier for the authorized QoS parameters for the service data flow. |  |
| ARP |  | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |  |
| GFBR |  | Guaranteed Flow Bit Rate (GFBR) - UL and DL |  |
| MFBR |  | Maximum Flow Bit Rate (MFBR) - UL and DL |  |
| Priority Level |  | Indicates a priority in scheduling resources among QoS Flows. |  |
| Averaging Window |  | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |  |
| Maximum Data Burst Volume |  | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |  |
| Reflective QoS Control |  | Indicates to apply reflective QoS for the SDF in the TFT. |  |
| QoS Notification Control (QNC) |  | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |  |
| Maximum Packet Loss Rate |  | Maximum Packet Loss Rate - UL and DL. |  |
|  |  |  | Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |
| EPS Bearer Id |  | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN |  |
| Mapped EPS Bearer QoS |  | ARP, GBR, MBR, QCI. |  |
| PGW-U tunnel Information |  | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |  |
| TFT |  | Traffic Flow Template |  |

Editor's note:	This table is known to be incomplete and to require updates. Some items listed in the table may be removed. This table requires also updates to cover all kind of SM contexts not limited to I-SMF and V-SMF.

* * * * Next change * * * *

##### 5.2.8.2.11	Nsmf_PDUSession_ContextPush service operation

Service operation name: Nsmf_PDUSession_ContextPushRequest

Description: This service operation is used by the SMF as Service Consumer to push one SM Context to a another SMF as NF Service Producer. It may be triggered by OAM.

Input, Required: One of the following:

-	SM Context of identified PDU session

-	Handle of SM Context of identified PDU session.

The SM context includes SM context in I-SMF(or V-SMF) and SM context in SMF (or H-SMF) separately.

See Table 5.2.8.2.10-1 for single SM Context stored in I-SMF or V-SMF that may be transferred to another SMF instance.

Editor's note:	The SM context stored in SMF(or H-SMF) is to be defined.

Output, Required: Result Indication.

Output, Optional: Cause.

See clause 4.26.2 for an example usage of this service operation.