3GPP TSG-SA WG2 Meeting #134	S2-1908090

# Sapporo, Japan,  June 24 - June 28, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0018 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Resource Change from OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | “NF Load Analytics” defined in Clause 6.5 requires as input data the “NF resource usage”. The problem is that changes in resource usage might also be a consequence of changes in the type of resources being used for NFs. The past data collected about NF based on resource type A might not be suitable to be used for deriving analytics when NF is now related to resource type B. NWDAF needs to be aware of this type of changes in order to generate the NF Load Analytics ID with a proper indication of the degree of confidence. A Note is provided to give a requirement to SA5 to provide appropriate OAM services in this regard removing the need for the editor’s note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | a) Add a sentence in Clause 6.2.3.1 enabling NWDAF to collect resource or changes in resource model information. b) Remove FFS in Clause 6.5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF will not have the information to determine the accuracy of the generate analytics when information about resources are relevant for the Analytics ID. Delay on data collection requirements from NWDAF to be addressed in SA5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.2.3.1	General

The NWDAF may collect relevant management data from the services in the OAM as configured by the PLMN operator.

‐	NG RAN or 5GC performance measurements as defined in TS 28.552 [8].

‐	5G End to end KPIs as defined in TS 28.554 [10].

NWDAF shall use the following services to have access to the information provided by OAM:

-	Generic performance assurance and fault supervision management services as defined in TS 28.532 [6].

‐	PM (Performance Management) services as defined in TS 28.550 [7].

‐	FS (Fault Supervision) services defined in the TS 28.545 [9].

NWDAF may collect information related to resources used by NFs (e.g., CPU load) or changes in resources from OAM

NOTE：Resources from OAM refer to e.g. the respective set of ManagedFunctions associated with the NF as defined in Clause 6.2.1, TS 28.541.

NWDAF can be configured to invoke the existing OAM services to retrieve the management data.

OAM perform the required configuration in order to provide the information requested by NWDAF subscription and perform the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

Another usage of OAM services is when the target of data collection is related to MDT based retrieval of information which is a per UE mechanism.

* * * * Next Change * * * *

6.5.4	Procedures

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to NF load information, the target for analytics is set to NF id. The NF can request statistics or predictions or both and can provide a time window.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF resource usage following steps captured in clause 6.2.3.2 for data collection from OAM. Steps 2-5 may be skipped when e.g. the NWDAF already has the requested analytics.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6.	The NWDAF retrieves the NF load and NF status information from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them to the NF upon reception of notification of new NF load information from OAM.

* * * * End of Changes * * * *