3GPP TSG-SA2 Meeting #134	S2-1908173

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1393 | rev | 8 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Context Transfer Support via indirect Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Deutsche Telekom, Telecom Italia, Sprint, AT&T, Verizon, Telstra, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#132, Service Context Transfer procedures (S2-1904843) were approved in TS23.502. Indirect transfer shall support Service Context Transfer with any NF/NF service. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document introduces indirect service context transfer via a context handle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service Context Transfer is not fully specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26, 5.2.8.2.10, 5.2.8.2.11, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial Rev To add the new service in UDSF service Rev 1-2 Use of CTSF for context data transfer Rev 3 1) Additions to sections 4.26, 5.2.8.2.10, 5.2.8.2.11: - to align with CTSF, i.e. description of indirect context transfer - Adressed potential race conditions (validity time) 2) Chagnes to endorsed text in section 5.2.X in yellow: - clarified support for different Service Context Types - added validity time - typos Rev 4 Limited scope to indirect transfer without mention of CTSF Text covering S2-1907542, S2-1907543: Role of O&AM also covering bulk transfer Rev 5 Removed conted related to “Bulk context transfer procedure using CTSF” Rev 6 Removed ED, replaced example with reference to TS Rev 7/rev 8 (re-upload) Removed last sentence of first change (endpoint co-location), added ED: “Which NF Service operation for indirect mode is FFS” Editorials |  |  |  |  |  |  |  |  |

* * * * First change * * * *

## 4.26	Network Function/NF Service Context Transfer Procedures

### 4.26.1	General

Network Function/NF Service Context Transfer Procedures allow transfer of Service Context of a NF/NF Service from a Source NF/NF Service Instance to the Target NF/NF Service Instance, e.g. before the Source NF/NF Service can gracefully close its NF/NF Service. If SMF sets are deployed this applies from an SMF instance within an SMF set to an SMF instance of another SMF set.

A request (push procedure, see 4.26.2) or response (pull procedure, see 4.26.3) from a Source NF/NF Service Instance to a Target NF/NF Service Instance contains either

- the context being transferred, e.g. SM context (direct mode) or

- optionally, an endpoint from which Target NF/NF Service Instance can retrieve the context, see TS 29.501 [55] (indirect mode).

It assumes that access to a given context endpoint can be restricted to single Target NF/NF Service Instance.

Editor’s Note: Which NF Service operation for indirect mode is FFS

### 4.26.2	NF/NF Service Context Transfer Push Procedure

Figure 4.26.2-1: NF/NF Service Context Push procedure

1.	When triggered, the Source NF/NF Service acting as NF Service Consumer its Context (e.g. UE Context or SM context) to the Target NF/NF Service acting as NF Service producer. This may trigger several other procedures that ensure all necessary NF/NF Services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the response indicating the result of the operation (successful or not successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumer of the Source NF/NF Service, which e.g. can be shut down gracefully.

NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

### 4.26.3	NF/NF Service Context Transfer Pull procedure

Figure 4.26.3-1: NF/NF Service Context Pull procedure

1.	When triggered, the Target NF/NF Service as an NF Service Consumer requests a Context (e.g. UE context or SM context) from the Source NF as a NF Service Producer. This may trigger several other procedures that ensure all necessary NF/NF services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the Context and if the operation was a successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumer of the Source NF/NF Service, which e.g. can be shut down gracefully.NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

### 4.26.4	Context Transfer due to decommissioning

In the case of decommissioning, the Old NF may inform NRF that it is about to be decommissioned. NRF will in this case not include the NF profile of the Old NF in any discovery result from the NF/NF service discovery procedure. The NRF will also notify any consumers subscribed to changes to this resource.

### 4.26.5	SMF Service Context Transfer procedures

#### 4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

#### 4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_StatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	From Old SMF to AMF Nsmf_PDUSession_StatusNotify (SMF transfer indication, New SMF ID

3.	The AMF waits until UE becomes IDLE or RRC inactive.

Editors note:	It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.

4.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 1 is used.

5.	From New SMF to Old SMF SMF Nsmf_PDUSessionContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	The Old SMF releases the N4 session.

Editors note:	It is FFS if steps 6 and 9 can be optimised or changed to a modification of the N4 session.

7.	From Old SMF to New SMF Nsmf_PDUSessionContextRequest response (SM Context or endpoint where New SMF can retrieve SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet.

8.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF sets up a new policy association towards PCF

9.	New SMF establishes N4 session. The parameters from step 7 and, if applis, step 8 are used.

10.	From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

11.	New SMF registers to UDM,

12.	New SMF subscribes to subscription changes for the UE

13.	[Conditional] Old SMF removes its policy association with PCF (triggered by step 7) Any changes to the QoS rules need to be sent to the UE when it becomes active. After Step 13 (Step 7 if there is no policy assiciation) the old SMF can relase any internal resources for the PDU session.

* * * * Next change * * * *

##### 5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO), or during mobility procedure with I-SMF changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, PDU Session ID (include PDU Session ID when available).

Output, Required: One of the following:

-	SM Context Container

-	Endpoint where SM Context can be retrieved.

Output, Optional:Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requrested, e.g. PDN Connection Context ,5G SM Context or both. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context that may be transferred between I-SMF(s) or between V-SMF(s)

| Field |  | Description |  |
| --- | --- | --- | --- |
| SUPI |  | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |  |
| Trace reference |  | Identifies a record or a collection of records for a particular trace. |  |
| Trace type |  | Indicates the type of trace |  |
| OMC identity |  | Identifies the OMC that shall receive the trace record(s). |  |
| S-NSSAI |  | The requested S-NSSAI for the serving PLMN to the PDU Session. |  |
| HPLMN S-NSSAI |  | Requested S-NSSAI for the HPLMN to the Home-Routed PDU Session |  |
| Network Slice Instance id |  | The network Slice Instance information for the PDU Session |  |
| DNN |  | The associated DNN for the PDU Session. |  |
| AMF Information |  | The associated AMF instance identifier and GUAMI. |  |
| Access Type |  | The current access type for this PDU Session. |  |
| RAT Type |  | RAT Type used by the UE. |  |
| PDU Session ID |  | The identifier of the PDU Session. |  |
| H-SMF Information |  | The associated H-SMF identifier and H-SMF address for the HR PDU Session.( applies only for a V-SMF) |  |
|  |  |  | For each QoS Flow in the PDU Session: |
| 5G QoS Identifier (5QI) |  | Identifier for the authorized QoS parameters for the service data flow. |  |
| ARP |  | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |  |
| GFBR |  | Guaranteed Flow Bit Rate (GFBR) - UL and DL |  |
| MFBR |  | Maximum Flow Bit Rate (MFBR) - UL and DL |  |
| Priority Level |  | Indicates a priority in scheduling resources among QoS Flows. |  |
| Averaging Window |  | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |  |
| Maximum Data Burst Volume |  | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |  |
| Reflective QoS Control |  | Indicates to apply reflective QoS for the SDF in the TFT. |  |
| QoS Notification Control (QNC) |  | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |  |
| Maximum Packet Loss Rate |  | Maximum Packet Loss Rate - UL and DL. |  |
|  |  |  | Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |
| EPS Bearer Id |  | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN |  |
| Mapped EPS Bearer QoS |  | ARP, GBR, MBR, QCI. |  |
| PGW-U tunnel Information |  | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |  |
| TFT |  | Traffic Flow Template |  |

Editor's note:	This table is known to be incomplete and to require updates. Some items listed in the table may be removed. This table requires also updates to cover all kind of SM contexts not limited to I-SMF and V-SMF.

* * * * Next change * * * *

##### 5.2.8.2.11	Nsmf_PDUSession_ContextPush service operation

Service operation name: Nsmf_PDUSession_ContextPushRequest

Description: This service operation is used by the SMF as Service Consumer to push one SM Context to a another SMF as NF Service Producer. It may be triggered by OAM.

Input, Required: One of the following:

-	SM Context of identified PDU session

-	Endpoint where SM Context of identified PDU session can be retrieved.

The SM context includes SM context in I-SMF(or V-SMF) and SM context in SMF (or H-SMF) separately.

See Table 5.2.8.2.10-1 for single SM Context stored in I-SMF or V-SMF that may be transferred to another SMF instance.

Editor's note:	The SM context stored in SMF(or H-SMF) is to be defined.

Output, Required: Result Indication.

Output, Optional: Cause.

See clause 4.26.2 for an example usage of this service operation.

* * * * Next change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[4]	IETF RFC 5998: "An Extension for EAP-Only Authentication in IKEv2".

[5]	IETF RFC 4282: "The Network Access Identifier".

[6]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[7]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[8]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[9]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[10]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[11]	3GPP TS 23.335:"User Data Convergence (UDC); Technical realization and information flows;	Stage 2".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.221: "Architectural requirements ".

[15]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[17]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data and Structured Data; Stage 3".

[20]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System ".

[21]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[22]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[23]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[24]	3GPP TS 23.203: "Policy and charging control architecture".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[27]	OMA-TS-ULP-V2_0_3: "UserPlane Location Protocol".

[28]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[29]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[30]	3GPP TS 36.355: "LTE Positioning Protocol (LPP)".

[31]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[32]	3GPP TS 29.507: "Access and Mobility Policy Control Service; Stage 3".

[33]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[34]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[35]	3GPP TS 23.251: "Network sharing; Architecture and functional description".

[36]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[37]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[38]	3GPP TS 23.380: "IMS Restoration Procedures".

[39]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[40]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[41]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[42]	3GPP TS 32.290: "Services, operations and procedures of charging using Service Based Interface (SBI)".

[43]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[44]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[45]	3GPP TS 32.255: "5G system; 5G data connectivity domain charging; Stage 2".

[46]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[47]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[48]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012): "IEEE Standard for Information technology - Telecommunications and information exchange between systems Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[49]	IETF RFC 2410: "The NULL Encryption Algorithm and its use with IPsec".

[50]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services; Stage 2".

[51]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[52]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[53]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[54]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[55]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".