3GPP TSG-SA WG2 Meeting #134	S2-1908345

# Sapporo, Japan, June 24 - 28, 2019 	(revision of S2-1907382)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1560 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IWK IDLE mode mobility using N26 interface with I-SMF/I-UPF and data fowarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When UE is in CM-IDLE mode with data buffering in EPS/5GS and move to 5GS/EPS with N26 interface, there is no I-SMF/I-UPF related handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Idle mode mobility procedures from EPS to 5GS with data forwarding and I-SMF inserstion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing I-SMF handling in idle mode interworking procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The procedure below refers to a procedure introduced by another CR. The reference to clause 4.11.1.3.3A is introduced by 23.502 CR1590 (originally in S2-1907495). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

##### 4.23.X.3	EPS to 5GS Idle mode mobility using N26 interface with Data forwarding and I-SMF insertion

The considered scenario is that a UE has PDN connections in EPS, during idle mode mobility from EPS to 5GS, I-SMF need to be inserted and data forwarding from SGW to I-UPF is performed.

Figure 4.23.X.3-1: EPS to 5GS Idle mode mobility using N26 interface with data forwarding and I-SMF insertion

1. Steps 1-10 of clause 4.11.1.3.3A are performed.

2.	The AMF determines whether a I-SMF needs to be selected based on UE location and service area of the PGW-C+SMF, if new I-SMF needs to be selected, the AMF selects a new I-SMF as described in clause 4.23.2.

Case: I-SMF insertion.

3.	Step 4 of clause 4.11.1.3.3A is performed with following differences:

-	PGW-C+SMF is replaced by I-SMF.

4.	The new I-SMF selects a new I-UPF: Based on the received information from AMF, e.g. EPS PDN connections, UE location information, the new I-SMF selects a new I-UPF as described in clause 6.3.3 of TS 23.501 [2].

5.	The new I-SMF initiates a N4 Session Establishment to the new I-UPF. In case tunnel endpoint is allocated by the new I-UPF, the new I-UPF provide tunnel endpoints to the new I-SMF, otherwise the new I-SMF allocates the tunnel endpoints and provides them to the new I-UPF.

6a.	The new I-SMF invokes Nsmf_PDUSession_Create Request (new I-UPF DL tunnel information, EPS PDN Connection context, UE location Information) towards the PGW-C+SMF. The I-SMF selects the PGW-C+SMF using the PGW-C+SMF info received from AMF. The PGW-C+SMF finds the corresponding PDU Session based on the PDN Connection Context in the request.

The SM Context ID at I-SMF is to be used by the SMF for further PDU Session operation, e.g. to notify the new I-SMF of PDU session release.

6b.	The PGW-C+SMF initiates N4 Session Modification toward the PDU Session Anchor UPF, providing the new I-UPF DL tunnel information. The PSA begins to send the DL data to the new I-UPF as indicated in the new I-UPF DL tunnel information.

6c.	The PGW-C+SMF responds to the new I-SMF with Nsmf_PDUSession_Create Response including the tunnel info for PDU Session.

7.	The I-SMF initiates N4 Session Modificaiton toward the new I-UPF including the tunnel info provided from SMF.

8.	The new I-SMF sends a Nsmf_PDUSession_CreateSMContext Response (N2 SM information (PDU Session ID, QFI(s), QoS profile(s), S-NSSAI, User Plane Security Enforcement, UE Integrity Protection Maximum Data Rate), Cause)) to the AMF. The N2 SM information, including the UL Tunnel infor for of the new I-UPF, formulated by I-SMF based on the Buffered DL data waiting indciaiton in step 3 above.

If the PDU Session has been assigned any EPS bearer ID, the new I-SMF also includes the mapping between EPS bearer ID(s) and QFI(s) into the N2 SM information to be sent to the NG-RAN.

9. 	Step 12-16 of clause 4.11.1.3.3A are performed.

Case: I-SMF insertion.

10.	Step 17a-17c of clause 4.11.1.3.3A are performed with following differences:

-	PGW_C+SMF and UPF are replaced by I-SMF and I-UPF.

-	The I-SMF and I-UPF shall also allocate the data forwarding tunnel for EPS if Buffered DL Data Waitting Indicaiton is received in step 3 above.

11. 	Step 18-20 of the clause 4.11.1.3.3A are performed.

***** End of Changes *****