3GPP TSG-SA WG2 Meeting #134	S2-1908349

# Sapporo, Japan, June 24 - 28, 2019 	(revision of S2-1908344)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1559 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IWK IDLE mode mobility using N26 interface with I-SMF/I-UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no description for how to handle I-SMF/I-UPF for the case with UE in CM-IDLE mode and then moving to 5GS/EPS with N26 interface |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Idle mode mobility procedures from 5GS to EPS with I-SMF removal Introduce Idle mode mobility procedures from EPS to 5GS with I-SMF inserstion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing I-SMF handling in idle mode interworking procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.X (new), 4.23.X.1 (new), 4.23.X.2 (new), 4.23.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change *****

### 4.23.X 	N26 based Interworking Procedures with I-SMF

#### 4.23.X.1	General

This clause describes the N26 based Interworking Procedures with I-SMF insertion and removal.

##### 4.23.X.2	5GS to EPS Idle mode mobility using N26 interface with I-SMF removal

The considered scenario is that a UE has PDU sessions with I-SMF, I-UPF available. The I-SMF is removed after idle mode mobility to EPS.

Figure 4.23.X.2-1: 5GS to EPS Idle mode mobility using N26 interface with I-SMF removal

1.	Step 1-4 from clause 4.11.1.3.2.

2.	Step 5 from clause 4.11.1.3.2 with the following differences:

-	PGW-C+SMF is replaced by I-SMF.

-	PGW-U+UPF is replaced by I-UPF

4. 	Step 6-19 from clause 4.11.3.2.

5. 	After the timer, started in step 4 in AMF, expires, the AMF sends Nsmf_PDUSession_ReleaseSMContext Request (I-SMF only indication) to the old I-SMF for the release of resources in old I-SMF. The I-SMF only indication indicates to old I-SMF not to invoke resource release in SMF.

NOTE: 	The PGW-C+SMF locally releases the N16a related resources when it is notified by SGW about the mobility to EPC.

6. 	The I-SMF initiates PDU session release in I-UPF.

7. 	The old I-SMF responds to AMF with Nsmf_PDUSession_ReleaseSMContext response and remove the PDU session.

**** First Change *****

##### 4.23.X.3	EPS to 5GS Idle mode mobility using N26 interface with I-SMF insertion

The considered scenario is that a UE has PDN connections in EPS, during idle mode mobility from EPS to 5GS, I-SMF need to be inserted.

Figure 4.23.X.3-1: EPS to 5GS Idle mode mobility using N26 interface with I-SMF insertion

1. 	Steps 1-11 from clause 4.11.1.3.3.

2. 	Step 16 from clause 4.2.2.2.2.

3. 	The AMF determines whether a I-SMF needs to be selected based on UE location and service area of the PGW-C+SMF received from MME, if a I-SMF needs to be selected, the AMF selects a new I-SMF as described in clause 4.23.2.

Case: I-SMF insertion.

4. 	Step 4 from clause 4.2.2.2.2 with addition in step 14 of clause 4.11.1.3.3, and the following enhancement:

The AMF initiates communication with the new I-SMF with Nsmf_PDUSession_CreateSMContext.

5. 	The new I-SMF selects a new I-UPF: Based on the received information from AMF, e.g. EPS PDN connections, UE location information, the new I-SMF selects a new I-UPF as described in clause 6.3.3 of TS 23.501 [2].

6. 	The new I-SMF initiates a N4 Session Establishment to the new I-UPF. In case tunnel endpoint is allocated by the new I-UPF, the new I-UPF provide tunnel endpoints to the new I-SMF, otherwise the new I-SMF allocates the tunnel endpoints and provides them to the new I-UPF.

7a.	The new I-SMF invokes Nsmf_PDUSession_Create Request (new I-UPF DL tunnel information, EPS PDN Connection context, UE location Information) towards the PGW-C+SMF. The I-SMF selects the PGW-C+SMF using the PGW-C+SMF info received from AMF. The PGW-C+SMF finds the corresponding PDU Session based on the PDN Connection Context in the request.

The SM Context ID at I-SMF is to be used by the SMF for further PDU Session operation, e.g. to notify the new I-SMF of PDU session release.

7b.	The PGW-C+SMF initiates N4 Session Modification toward the PDU Session Anchor UPF, providing the new I-UPF DL tunnel information. The PSA begins to send the DL data to the new I-UPF as indicated in the new I-UPF DL tunnel information.

7c.	The PGW-C+SMF responds to the new I-SMF with Nsmf_PDUSession_Create Response including the tunnel info for PDU Session.

8.	The I-SMF initiates N4 Session Modificaiton toward the new I-UPF including the tunnel info provided from PGW-C+SMF.

9.	The new I-SMF sends a Nsmf_PDUSession_CreateSMContext Response (PDU Session ID, S-NSSAI, N2 SM information (PDU Session ID, QFI(s), QoS profile(s), N1 SM Container, Cause)) to the AMF.

10. Step 18-20 from clause 4.2.2.2.2.

11. Step 15-18 from clause 4.11.1.3.3.

#### 4.23.X.4	Procedures for EPS bearer ID allocation

The EPS Bearer ID allocation procedure, EPS Bearer ID transfer procedure and EPS Bearer ID revocation procedure are perfomed as described for the home-routed roaming case in clause 4.11.1.4, with the following differences:

-	H-SMF is replaced by SMF

-	V-SMF is replaced by I-SMF

**** End of Changes *****