3GPP TSG-SA WG2 Meeting #134 	S2-1908426

Sapporo, Japan, 24 – 28 June, 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1657 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UP optimization with MO-EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIOT |  |  |  |  |  | Date: |  |  | 2019-06-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The paper address the outstanding issue: “Support of EDT for 5GS UP optimisation”. This contribution provides the procedure for sending MO EDT from RRC IDLE with Suspend that includes handing of AS RAI to indicate whether the UE can be suspended immediately. In the case the UE is suspended immediately the AMF needs to be informed so that it can manage the Periodic Registration Timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure for handling MO EDT from RRC Idle with Suspend, including handling of AS RAI and notifications to the AMF to handle the Periodic Registration Timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDT for User Plane CIoT 5GS optimisation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

4.8.2.4	Connection Resume in CM-IDLE with Suspend and MO EDT procedure

The Connection Resume with Early Data Transmission procedure is used by the UE to optimise sending user data in a single uplink packet and single uplink followed by single downlink packet cases for a UE in CM-IDLE with Suspend.

Figure 4.8.2.4-1: Connection Resume in RRC Idle with Suspend with EDT

1.	UE to NG-eNB: RRC message (Resume ID, AS RAI) with UL EDT.

The UE initiates the transition from RRC IDLE with Suspend state to RRC Connected state, see TS 36.300 [9].

2.	The NG-eNB deciphers the EDT UL data received from the UE and forwards it to the UPF using the N3 UL TEID in the AS context.

The NG-eNB determines if an immediate return to RRC IDLE with Suspend is possible, based on the AS RAI indicating no further Uplink and Downlink Data transmissions are expected.

2b.	[Conditional] NG-eNB to UE: RRC message (no DL EDT).

If no further Uplink and Downlink Data transmissions was determined in step 2, the NG-eNB may release the RRC Connection with Suspend immediately.

If a single Downlink data transmission is determined in step 2, the NG-eNB does not send any RRC message in this step, but waits for DL data to arrive in step 6.

3.	The NG-eNB sends N2 Resume Request to AMF including Resume cause and N2 SM. If no further Uplink and Downlink Data transmissions was determined in step 2, NG-eNB also indicates in the N2 Resume Request the immediate return to RRC IDLE with Suspend.

4.	The AMF interacts with SMF to establish the N3 tunnel. If NG-eNB indicated the immediate return to RRC IDLE with Suspend in the step 3, then the AMF does not perform this step.

5.	If NG-eNB indicated the immediate return to RRC IDLE with Suspend then the AMF sends an N2 Resume Response to NG-eNB with acknowledgment of suspend, the AMF restarts the Periodic Registration Timer when the Strictly Periodic Registration Timer Indication has not been provided, and the procedure stops here. Otherwise the AMF sends an N2 Resume Response to NG-eNB after the N3 Connectivity has been established. If the AMF knows of mobile terminating data or signalling pending, the AMF may include the Extended Connected Time value to the RAN.

6.	 DL data arrives as a response to the UL data in step 2.

7-9. If single Downlink data transmission was determined in step 2, the NG-eNB releases the N3 connectivity by sending an N2 Suspend Request to AMF including Suspend cause and N2 SM. Steps 2-4 of Connection Suspend procedure in clause 4.8.1.2 is executed.

10.	[Conditional] NG-eNB to UE: RRC message (with DL EDT).

The NG-eNB ciphers received DL data.

If single Downlink data transmission was determined in step 2, the NG-eNB releases the RRC Connection with Suspend including the DL EDT data. Otherwise, when RAN receives the Extended Connected Time value in step 5 or multiple packet transmission is determined in step 2, then NG-eNB sends an RRC Resume message to the UE and the UE moves to CM-CONNECTED and RRC Connected.

*** END CHANGES ***