SA WG2 Meeting #135 	S2-1908770

# October 14-18th, 2019 ; Split, HR		                               			  (revision of S2-1908770)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1669 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of ETSUN in case of Non 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ETSUN relies on « When the SMF cannot control the UPF terminating the N3 Interface used by a PDU Session and SSC mode 2/3 procedures are not applied to the PDU Session, an I-SMF is inserted between the SMF and the AMF …. For this purpose the AMF gets from NRF information about the Service Area of SMF(s). During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI “. In other word the AMF compares the UE location (current TA) with the SMF serving area information received from NRF (which basically consists in a collection of TA(s)); in N3GPP case the UE TA does not give any location information as currently there is an unique TA identifying the N3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed solution: - allocate a TA per non 3GPP gateway (e.g. N3IWF) : each non 3GPP gateway is locally configured with its own TAI whose value is provided (as in case of 3GPP access) to AMF at INITIAL UE NGAP/N2 message as part of UE location. This is used by the AMF as the UE TA and the same algorithm than in case of 3GPP access can apply for the determination of I-SMF insertion, change, removal for a PDU Session - document that PDU Session mobility between 3GPP and N3GPP access may lead to I-SMF insertion, change, removal. 4.23.5.1: When the delegated discovery is used, the SMF / I-SMF selection applies as defined in TS 23.501 [2] clause 5.34.3 4.23.5.1: “(i.e. Tracking Area of the UE on the access where the PDU Session establishment has been received)” clarification added to also deal with ATSSS. The selection / insertion of I-SMF applies on a per access basis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ETSUN not supported in case of N3GPP access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1 ; 4.23.X ; 4.23.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 1699 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

FIRST CHANGE

#### 4.2.7.1	N2 Configuration

At power up, restart and when modifications are applied, the 5G-AN node and AMF use non-UE related N2 signalling to exchange configuration data. Full details of this configuration data are specified in TS 38.300 [9], but the following highlights some aspects.

The AMF supplies the 5G-AN node with information about:

a)	the AMF Name and the GUAMI(s) configured on that AMF Name;

b)	the set of TNL associations to be established between the NG-RAN node and the AMF;

c)	weight factor associated with each of the TNL association within the AMF; and

d)	weight factor for each AMF Name within the AMF set; and

e)	(optional) for each GUAMI(s) configured on that AMF the corresponding backup AMF Name.

The weight factors are used for load distribution of the initial N2 messages. The AMF chooses whether or not to use the same TNL association for the initial N2 message and subsequent messages for that UE. TNL associations configured with a weight factor set to zero are not permitted for the initial N2 message, but can be used for subsequent N2 messages.

Deployments that rely solely on 5GC-based load balancing can set the weight factors associated with TNL associations that are permitted for the initial N2 message to the same value.

The 5G-AN supplies the AMF node with information about the Tracking Area(s) it supports.

NEXT CHANGE (2)

### 4.23.X	PDU Session mobility between 3GPP and N3GPP access

At PDU Session mobility between 3GPP and N3GPP access, it may happen that an I-SMF needs to be inserted, changed or removed because the TA associated with the UE location over 3GPP and N3GPP access are not in the same (I-)SMF area.

The Procedure described in clause 4.23.5.1 applies where only I-SMF selection is needed (no SMF selection is needed).

NEXT CHANGE (3)

#### 4.23.5.1	PDU Session establishment procedure

For non roaming or LBO roaming, it includes the following cases:

-	If the service area of the selected SMF includes the location where the UE camps, the UE requested PDU Session Establishment procedure is same as described in clause 4.3.2.2.1.

-	If the service area of the selected SMF does not include the area where the UE camps (i.e. Tracking Area of the UE on the access where the PDU Session establishment has been received), the AMF selects an I-SMF that serves the area where UE camps. The UE requested PDU Session Establishment procedure for Home-routed Roaming defined in clause 4.3.2.2.2 is used to establish the PDU Session. Compared to the procedure defined in clause 4.3.2.2.2, the V-SMF and V-UPF are replaced by I-SMF and I-UPF, and H-SMF and H-UPF are replaced by SMF and UPF(PSA) respectively. Also only the S-NSSAI with the value defined by the serving PLMN is sent to the SMF. The I-SMF provides the DNAI list it supports to SMF as defined in Figure 4.23.9.1-1 step 1.

-	When the delegated discovery is used, the SMF / I-SMF selection applies as defined in TS 23.501 [2] clause 5.34.3.

For the Home-Routed roaming case, the UE requested PDU Session Establishment procedure for Home-routed Roaming in clause 4.3.2.2.2 can be reused without change.

END OF CHANGES