3GPP TSG-SA WG2 Meeting #135	S2-1908822

Split, Croatia, 11-18 Oct 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1654 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on binding ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In binding for subscription and notification procedure, the binding indication of the consumer is missing. The change of the binding indication of the producer is also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of binding indication change in subscription and notification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The binding indication for subscription and notification procedure is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.17.12.4	Binding for subscription and notification

Binding can be created as part of the implicit subscription request. In this case, the service request may include a binding indication of  the NF Service Consumer as in 4.17.12.3, and the response to the service request that creates the implicit subscription may include a binding indication of the NF Service Provider as in 4.17.12.2. For subsequent operations on the created implicit subscription, bindings may be used. The description in 4.17.12.1 applies for Figure 4.17.12.4.-1.

If the binding indication of the NF Service Consumer needs to be updated, the NF service consumer may initiate a new request to the NF service producer using the binding indication of the NF Service Provider. The request includes  an updated Notification Correlation ID and an updated binding indication of the NF Service Consumer.

If the binding indication of the NF Service Provider needs to be updated, the NF Service Provider may initiate a notification to the NF Service Consumer using the binding indication of the NF Service Consumer. The notification includes an updated Subscription Correlation ID and an updated binding indication of the NF Service Provider.

Figure 4.17.12.4-1: Binding for subscription requests

* * * * End of Changes * * * *