3GPP TSG-SA2 Meeting #135	S2-1908957

# 14 - 18 October, 2019, Split Croatia											     (revision of S2-1907219)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0270 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF discovery by AF via BSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-10-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) |  |
| Reason for change: |  | According to the introduction of NF/NF Service Set, when the original NF service/instance is unavailable, the consumer/SCP may reselect another NF serivce instance from the same set. This applies to all control plane NFs. And the binding ID is stored at the NF service consumer. However, when AF as a NF service consumer requests service from PCF, it is unclear whether the AF should support the same function,e.g. store the binding ID. - If SCP is deployed and AF supports to carry binding ID in service request, the SCP can always reselect PCF based on binding ID in case the original one is not available. In this case, the AF is not required to detect PCF failure. - If the SCP is not deployed, the AF has to do the re-selection. However in other case the AF get the PCF information is just from BSF. So why the AF need support two mechanisms for PCF selection? Two mechanism only add complexity to the AF implementation. For the AF not support SBI itnerface, it use the Rx interface to interact with PCF. In this case it is impossible to require the AF to select the PCF based on binding ID as the binding ID is not conveyed at the Rx interface. Hence, it is proposed that if AF need select a new PCF, e.g. PCF failure, the AF can reuse the interaction with BSF, i.e. check with the BSF for another available PCF. And the BSF based on the previous stored binding ID select the available PCF and return it to the AF. This solution has less requirement on AF and can apply to both Rx based AF and serivce based AF, i.e. N5 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Besides the information of PCF instance stored in BSF, it is proposed that binding indication is also stored in BSF. The BSF monitors the availability of PCF/PCF serivces. Upon request for PCF discovery from AF, the BSF selects PCF based on table 6.3.1.0-1 in TS 23.501. The AF always provides the same App Session ID to the PCF For Rx based AF, if the AF need change the PCF, e.g. the failure of the PCF, the AF initiates a new Rx session establishment request including the old Rx session ID towards the BSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCF reliability is not supported for the interaction with AF, especially if Rx based interaction is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2.2, 6.2.3, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

6.1.1.2.2	The Binding Support Function (BSF)

The BSF has the following characteristics:

-	The BSF stores information about the user identity, the DNN, the UE (IP or Ethernet) address(es), the DN information (e.g. S-NSSAI), the selected PCF address and a binding Indication for a certain PDU Session. This information is stored internally in the BSF.

-	The PCF registers, updates and removes the binding information from the BSF using the Nbsf management service operations defined in TS 23.502 [3].

-	The PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.

-	Based on operator's policies and configuration, the PCF determines whether the same PCF shall be selected for the SM Policy associations to the same UE ID, S-NSSAI and DNN combination in the non-roaming or home-routed scenario.

NOTE 1:	This applies to usage monitoring.

-	If the PCF Set is supported, the PCF indicates the binding indication, which is defined in clause 6.3.1 of TS 23.501 [2], to be stored at the BSF. The binding indication is used for the subsequent PCF discovery, e.g. the original PCF is unavailable.

-	The selected PCF (if needed) downloads the user profile from the UDR as described in TS 23.502 [3] 4.16.4 step 2. If usage monitoring is enabled for the user, and based on operator's policies, the PCF checks if the BSF has already existing PCF serving the combination of SUPI, S-NSSAI, DNN.

-	If no such PCF is found the PCF shall register itself to the BSF as described above in this clause.

-	Else if an existing PCF is found for the above combination, the PCF shall return to the SMF the PCF ID of the existing PCF and a redirection indication.

NOTE 2:	The assumption is that for DNN, S-NSSAI combinations where usage monitoring be applied, the same BSF instance or the same BSF SET is selected for all UE PDU sessions to the same DNN, S-NNSAI.

-	For retrieval binding information, any NF, such as NEF or AF, that needs to discover the selected PCF for the tuple (UE address, DNN, S-NSSAI, SUPI, GPSI) (or for a subset of this Tuple) uses the Nbsf management service discovery service operation defined in TS 23.502 [3].

-	The NF may discover the BSF via NRF or based on local configuration. In case of via NRF the BSF registers the NF profile in NRF. The Range(s) of UE IPv4 addresses, Range(s) of UE IPv6 prefixes supported by the BSF may be provided to NRF.

-	When receiving the request to retrieve the serving PCF and the binding indication is stored, the BSF (re-)selects the PCF as described in Table 6.3.1.0-1 of TS 23.501 [2].

The BSF may be deployed standalone or may be collocated with other network functions, such as PCF, UDR, NRF, SMF.

NOTE:	Collocation allows combined implementation.

*** 2nd CHANGE  ***

6.2.3	Application Function (AF)

The AF functional description in clause 6.2.3 of TS 23.203 [4] applies with the following clarifications:

-	the mechanism for an AF to select a PCF associated to a PDU Session is described in clause 6.1.1.2 of the present specification

-	the mechanism for RAN user plane congestion mitigation is not specified in this release and as a consequence will not trigger a re-try interval in the interaction between PCF and the AF.

-	In case of private IP address being used for the end user, the AF may send additional DN information (e.g. DNN). This information is used by the PCF for session binding, and it is also used to help selecting the correct PCF

In addition to the functionality described in clause 6.2.3 of TS 23.203 [4] the AF may contact the PCF via the NEF following the same functional description as when the AF contacts the PCRF via the SCEF described in TS 23.203 [4].

The AF may subscribe in the PCF to receive notifications when the QoS targets can no longer (or can again) be fulfilled for a particular media flow. At the time the PCF gets notified that the GFBR can no longer (or can again) be guaranteed for a PCC rule, the PCF notifies to the AF the affected media and provides the indication that the QoS targets can no longer (or can again) be fulfilled. The AF behaviour is out of the scope of this TS.

NOTE 1:	AF subscription to notifications that QoS targets can no longer (or can again) be fulfilled which results in the setting of the QoS Notification Control in the PCC rule is only applicable to applications being able to adapt to the change of QoS (e.g. rate adaptation) for GBR flows.

To support sponsored data connectivity the AF may provide the PCF with the sponsored data connectivity information, including optionally a usage threshold, as specified in clause 6.2.1.1. The AF may request the PCF to report events related to sponsored data connectivity.

NOTE 2:	Annex N in TS 23.203 [4] describes the scenario for sponsored data connectivity.

If the PCF Set is supported, it should be possible for any PCF within the PCF Set to handle the request from AF. The association between AF and PCF needs to be unique per PCF Set.  At any time the AF may interact with BSF to retrieve the serving PCF, e.g. when the original PCF is not available. When the AF sends the request to the selected PCF the AF always provides the same App Session ID to the PCF regardless whether a new PCF is selected by the BSF. The selected PCF handle the service request based on the received App Session ID, i.e. retrieve the old App Session Context.

*** 3rd Change  ***

Annex C (Normative):
Support for Application Functions supporting Rx interface

To allow the 5G system to interwork with AFs related to existing services, e.g. IMS based services as described in TS 23.228 [5], Mission Critical Push To Talk services as described in TS 23.179 [6], the PCF shall support the corresponding IMS procedures defined in the main body of this TS via Rx interface. This facilitates the migration from EPC to 5GC without requiring these AFs to upgrade to support the Npcf_PolicyAuthorization services in Rel-16.

Figure C-1: Interworking between 5G Policy framework and AFs supporting Rx interface

Session Binding applies for PDU Sessions of IP type only

The functionality described for Multimedia Priority Services (clause 6.11) and Mission Critical service (clause 6.19) applies via Rx interface.

In order to support IMS Emergency services over Rx interface, in addition to the functional description in clause 6.10, the following applies: The PCF shall provide the IMEI and the subscriber identifiers (IMSI, MSISDN) (if available), received from the SMF at PDU Session establishment, if so requested by the P-CSCF. The PCF derives the IMEI from the PEI, the IMSI from the SUPI and the MSISDN from the GPSI.

NOTE 1:	TS 23.501 [2] defines both 5G identifiers, SUPI, PEI and GPSI and then how they are allocated to allow interworking with functional entities not supporting 5G identifies such as P-CSCF.

Any AF using Rx, such as P-CSCF, the BSF determines the selected PCF address according to the information included in the incoming Rx requests and the information stored at the BSF. The BSF is able to proxy or redirect Rx requests targeting an IP address of a UE to the selected PCF.

If the PCF Set is supported, when the AF wants to change the serving PCF, e.g. the original PCF is unavailable, the AF initiates a new Rx session establishment request including the old Rx session ID towards the BSF. The BSF determines a PCF as defined in clause 6.1.1.2.2. The selected PCF handles the Rx session establishment request based on the old Rx session ID, i.e. retrieve the old Rx Session Context.

The following event reporting is supported over Rx interface:

Table C-1: Events relevant for reporting from the PCF

| Event | Description | Availability for Rx Session |
| --- | --- | --- |
| PLMN Identifier Notification | The PLMN identifier where the UE is currently located. | Yes |
| Change of Access Type | The Access Type and, if applicable, the RAT Type of the PDU Session has changed. | Yes |
| Signalling path status | The status of the resources related to the signalling traffic of the AF session. | Yes |
| Access Network Charging Correlation Information | The Access Network Charging Correlation Information of the resources allocated for the AF session. | Yes |
| Access Network Information Notification | The user location and/or timezone when the PDU Session has changed in relation to the AF session. | Yes |
| Reporting Usage for Sponsored Data Connectivity | The usage threshold provided by the AF has been reached; or the AF session is terminated. | Yes |
| Resource allocation status | The status of the resources related to the AF session (established/released). | Yes |
| QoS targets can no longer (or can again) be fulfilled | The QoS targets can no longer (or can again) be fulfilled by the network for (a part of) the AF session. | No |
| Out of credit | Credit is no longer available. | Yes |

*** END CHANGES  ***