3GPP TSG SA WG2 Meeting #135	S2-1909058

# Split, HR, October 14-18, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1725 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE indicating that initial Registration is from GERAN/UTRAN to 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per clause 5.17.2.4 of TS 23.501, upon mobility from GERAN/UTRAN to 5GS, the UE shall do initial Registration. Per 23.502, the AMF does not do Nudm_UECM_Registration if a valid context exists. 4.2.2.2.2 General Registration … 14a-c. If the AMF has changed since the last Registration procedure, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this Registration procedure to add a 3GPP access), the new AMF registers with the UDM using Nudm_UECM_Registration for the access to be registered (and subscribes to be notified when the UDM deregisters this AMF). As a result, the UDM will not be able to trigger cancellation to SGSN via HSS/HLR at initial Registration when UE moves from GERAN/UTRAN to 5GS, which may lead to terminating voice call failure if the HLR/SGSN is not upgraded for T-ADS data retrieval. During initial Registration, when the AMF has valid context, to ensure that the AMF do registration in UDM only if UE moves from GERAN/UTRAN, it’s proposed that the UE includes a new indication. DP S2-1909053 may be referenced for more discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE includes a new indication in initial Registration to indicate that UE moves GERAN/UTRAN to 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminating voice call may fail as the UE may be considered in 2G/3G when in fact in AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11a (new), 4.11a.1 (new), 4.11a.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501CR1744, TS23.502CR1723 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | If there is a strong motiviation to have the solution from Rel-15, it can be UE implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

## 4.11a	Mobility from GERAN/UTRAN to 5GC

### 4.11a.1	General

This clause captures impact to 5GC procedure when UE moves from GERAN/UTRAN to 5GS.

#### 4.11a.2	Registration procedure

The following impacts are applicable to clause 4.2.2.2 (Registration procedure):

-	Step 1

When the UE performs initial registration upon mobility from GERAN/UTRAN to 5GS as described in clause 5.17.2.4 of TS 23.501 [2], the UE should indicate to the AMF that the UE is from GERAN/UTRAN.

-	Step 14a-c

If the UE indicates the initial registration is from GERAN/UTRAN to 5GS, the AMF shall perform the Nudm_UECM_Registration service operation even if the AMF has a valid context.

*************** End of changes ***************