SA WG2 Meeting #S2-135	S2-1909153

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1473 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting delegated discovery and selection and the use of IDs in binding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification it is stated that IDs of NF instance, NF service set and NF set may be included in a response to an initial request by the NF producer, as the binding indication. The question is why these ids shall be provided by the NF producer, when it is enough to provide the binding indication. In case of direct communication between NFs and indirect communication without delegated discovery, the NF consumer knows what it selected in the first place, it has all the necessary information from the NF profile from the initial selected NF and thus getting the IDs together with the binding indication does not help the NF consumer in anyway. If the NF consumer doesn’t have the NF profiles for the NF set, the NF consumer needs to do a discovery of the NF set to get the NF profiles of the NFs in the NF set, this regardless if the NF set ID was received in the response or if parsed from the NF profile. For NF service set ID and NF instance ID, it already has this information in the originally selected NF’s NF profile. In case of delegated discovery, the SCP must in the initial request understand which service that the NF consumer wants. The SCP selects an NF instance and NF service instance of wanted NF service. At response the NF producer may convey a binding indication related to a created resource. The binding indication is passed to the NF consumer. If there is a subsequent request, the SCP does not know which NF was selected in the first place (unless it has data stored per UE/context). This means that for any subsequent request the SCP need to give some information to the NF consumer which NF was selected. Either the SCP alters the actual URI of the resource or it sends some specific information. In which format this indication takes is a stage 3 issue, so it is proposed that the SCP sends an indication of selected NF instance and resource |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing that NF service producers sends IDs of NF, NF service set or NF set. Adding that SCP inform about selected NF instance and resource identity in a first response for which binding information is included |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delegated discovery and selection may not work. Unnecessary information in the signalling flow. Undefined binding with delegated discovery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 6.3.1.0	Principles for Binding, Selection and Reselection

Binding can be used to indicate suitable target NF producer instance(s) for NF service instance selection, reselection and routing of subsequent requests associated with a specific context. This allows the NF producer to indicate that the NF consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set depending on local policies and other criteria (e.g. at what point it is in the middle of a certain procedure, considering performance aspects etc).

The NF service producer may provide a binding indication to the NF service consumer as part of the Direct or Indirect Communication procedures, to be used in subsequent related service requests. The level of binding indication provided by the NF service producer to the NF consumer indicates if the producer is either bound to NF service instance, NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. If the resource is created in the NF Service Producer, the NF Service Producer provides resource information which includes a resource identity In case of delegated discovery and selection, this is used by the SCP to create information to the NF consumer of selected NF and resource identity (e.g. a URL). If the binding indication is present in the response from the NF service producer, the SCP pass this on to the NF service consumer. In case of indirect communication, the The NF service consumer shall include the received binding indication to the SCP. If also delegated discover and selection is used both binding indication aswell as information regarding selected NF and resource identity shall be sent to the SCP by the NF service consumer. The SCP will use binding information and potentially the information of selected NF and resource identity when selelecting a target service instance in a NF service producer instance.

NOTE 1: In which format SCP will will inform NF service consumer about selected NF service producer and resource identity is for stage 3 to decide.

Table 6.3.1.0-1 defines the selection and reselection behaviour of NF services consumers and SCPs depending on the binding indication provided by an NF service producer. The detailed procedures refer to clause 4.17.11 of TS 23.502 [3]

Table 6.3.1.0-1: Binding, selection and reselection

| Binding indication | The NF Consumer/SCP selects | The NF Consumer/SCP can reselect e.g. when selected producer is not available |  |
| --- | --- | --- | --- |
| NF Service Instance | The indicated NF Service Instance | An equivalent NF Service instance: - within the NF Service Set (if applicable) - within the NF instance - within the NF Set (if applicable) |  |
| NF Service Set | Any NF Service instance within the indicated NF Service Set | Any NF Service instance within an equivalent NF Service Set within the NF Set (if applicable) (Note 2) |  |
| NF Instance | Any equivalent NF Service instance within the NF instance. | Any equivalent NF Service instance within a different NF instance within the NF Set (if applicable) |  |
| NF Set | Any equivalent NF Service instance within the indicated NF Set | Any equivalent NF Service instance within the NF Set |  |
| NOTE 1: if binding indication is not available, the NF Consumer/SCP routes the service request to the target based on routing information available. NOTE 2: NF Service Sets in different NFs are considered equivalent if they include same type and variant (e.g. identical NF Service Set ID) of NF Services. |  |  |  |

The requester NF or SCP may subscribe to receive notifications from the NRF of a newly updated NF profile of an NF (e.g. NF service instances taken in or out of service), or newly registered de-registered NF instances. The NF/NF service status subscribe/notify procedure is defined in TS 23.502 [3], clauses 4.17.7 and 4.17.8.

For NF and NF service discovery across PLMNs, the NRF in the local PLMN interacts with the NRF in the remote PLMN to retrieve the NF profile(s) of the NF instance(s) in the remote PLMN that matches the discovery criteria. The NRF in the local PLMN reaches the NRF in the remote PLMN by forming a target PLMN specific query using the PLMN ID provided by the requester NF. The NF/NF service discovery procedure across PLMNs is specified in clause 4.17.5 of TS 23.502 [3].

NOTE2:	See TS 29.510 [58] for details on using the target PLMN ID specific query to reach the NRF in the remote PLMN.

For topology hiding, see clause 6.2.17.

*** END CHANGES ***