# 3GPP TSG-SA2 Meeting #135	                                                                        S2-1909300

# 14 - 18 October, 2019, Split Croatia

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1604 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for N19 Tunnel establishment and release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.7.1/Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been clear the conception of N19 and SMF support for establish N19 tunnel in 23.501. There is no procedure of N19 tunnel establishment and release in 23.502. It also lack when the SMF determines to establish and release N19 tunnels in 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal about the procedures of N19 tunnels establishment and release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear when the SMF determines to establish and release N19 tunnels between UPFs in 5G VN group. It lack of procedure of N19 tunnel establishment and release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X, 4.3.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

##### 4.3.X	N19 Tunnel Establishment

SMF determines to establish the N19 tunnels between a new UPF, and other UPFs(i.e.UPF1 in figure 4.3.X-1) which have been involved with the 5G VN group, during the PDU session establishment procedure.

Figure 4.3.X-1 Procedure for N19 Tunnel Establishment

1. Step 1-9 of UE requestes PDU Session Establishment in clause 4.3.2.2.1. And the UPF in this PDU session establishment is a new UPF for the 5G VN group.

2. SMF determines to establish N19 tunnels between the UPF and UPF1

3a.The SMF sends an N4 Session Establishment Request to the UPF with N19 CN tunnel info of UPF1, if N19 CN tunnel info is allocated by SMF.

3b.UPF sends a N4 Session Establishment Response to SMF, with the N19 CN tunnel info of UPF, if CN tunnel info is allocated by the UPF

4a. The SMF sends a N4 Session Modification Request to UPF1 with N19 CN tunnel info of UPF and N19 CN tunnel of UPF1 if N19 CN tunnel info is allocated by the SMF.

4b.UPF1 sends a N4 Session Modification Response to SMF with N19 CN tunnel info of UPF1 if N19 CN tunnel info is allocated by UPF1.

5a. [Conditional] If N19 CN tunnel info is allocated by UPF1, SMF sends a N4 Session Modification Request to UPF with N19 CN tunnel info of UPF1.

5b.[Conditional] UPF sends a N4 Session Modification Response to SMF.

<<< End of change1 >>>

##### 4.3.Y	N19 Tunnel Release

N19 tunnel(s) can be released during or after the procedure of PDU Session release. SMF determines to release N19 tunnel between a UPF and other UPF(s) (i.e.UPF1 in figure 4.3.Y-1) , when there is no more PDU sessions of the 5G VN group exists in the UPF.

Figure 4.3.Y-1 Procedure for N19 Tunnel Release

1. Step 1 of UE or network requested PDU Session Release in clause 4.3.4.2-1.

2. SMF determines to release N19 tunnel. If this PDU session is the last one of 5G VN group in the UPF, SMF will determine to release the N19 tunnel between the UPF and UPF1 in the 5G VN group.

3a. The SMF sends an N4 Session Release Request (N4 Session ID, N19 CN tunnel Info) message to the UPF. The UPF shall drop any remaining packets of the PDU Session and release N19 CN tunnel info resource and contexts associated with the N4 Session.

3b.	The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response message to the SMF.

4a.The SMF sends a N4 Session Modification Request to UPF1. The UPF1 shall drop any remaining packets of the PDU Session and release N19 CN tunnel info resource and contexts associated with the N4 Session.

4b. The UPF acknowledges the N4 Session Modification Request.

<<< End of changes >>>