3GPP TSG-SA WG2 Meeting #135	S2-1909370

Split, HR, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1798 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NG-RAN sharing options for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.501 states that 5G MOCN supports RAN sharing by multiple SNPNs, and RAN shared by one or multiple SNPNs and one or multiple PLMNs. For other combination of sharing SA2 asked for RAN WG feedback and RAN2 replied in S2-1908685 and RAN3 in S2-1908695 basically stating that all options are possible with the following clarification from RAN3: " logical cell (i.e. defined by a cell identity) would be associated with a single access mode (i.e. a logical cell would either be part of a public or a private network, and in the latter case, support PNI-NPN, or SNPN). In this case, a physical cell may host several logical cells, and therefore may support different network types (and therefore sharing of the physical resource). " With such principle all the sharing combinations SA2 asked for can be supported and it is therefore proposed to move forward with supporting all combinations. The support for Network Sharing with multiple cell-ID broadcast is described in TS 38.300 and TS 38.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for following NG-RAN sharing options are added - RAN sharing between a PNI-NPN (with CAG) and an SNPN - RAN sharing between a PLMN and a PNI-NPN with CAG And existing NG-RAN sharing option for sharing between a PLMN and an SNPN is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All applicable NG-RAN sharing options will not be supported for NPN type of deployments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

## 5.18	Network Sharing

### 5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements.

In this Release of the specification, only the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. 5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID (i.e. with same or different country code (MCC) some of which is specified in TS 23.122 [17] and different network codes (MNC)). 5G MOCN support NG-RAN Network Sharing with multiple cell-ID broadcast as described in TS 38.300 [27], in which case a physical cell can be separated into multiple logical cells.

5G MOCN, with or without the use of NG-RAN sharing with multiple cell-ID broadcast, also supports the following sharing scenarios involving non-public networks:

-	NG-RAN is shared by multiple SNPNs (each identified by PLMN ID and NID);

-	NG-RAN is shared by one or multiple SNPNs and one or multiple PLMNs, with the SNPNs and PLMNs being part of separate logical cells;

-	NG-RAN is shared by one or more PNI-NPNs (with CAG) and one or more SNPNs, with the PNI-NPNs and the SNPNs being part of separate logical cells; and

-	NG-RAN is shared by one or multiple PLMNs and one or multiple PNI-NPNs (with CAG), with the PNI-NPNs and the PLMNs being part of separate logical cells.

NOTE 1:	Different PLMN IDs (or combinations of PLMN ID and NID) can also point to the same 5GC.

NOTE 2:	There is no standardized mechanism to avoid paging collisions if the same 5G-S-TMSI is allocated to different UEs by different PLMNs or SNPNs of the shared network, as the risk of paging collision is assumed to be very low. If such risk is to be eliminated then PLMNs and SNPNs of the shared network needs to coordinate the value space of the 5G-S-TMSI to differentiate the PLMNs and SNPNs of the shared network.

Figure 5.18.1-1: A 5G Multi-Operator Core Network (5G MOCN) in which multiple CNs are 
connected to the same NG-RAN

*** END OF CHANGES ***