3GPP TSG-WG SA2 Meeting #135 	S2-1909455

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1822 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaiton of UPF selection for 5GLAN communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UPF selection is for 5GLAN communication is supported by SMF. Existing UPF selection criteria and implementation assumes that SMF looks up new UPF regardless existing UPF for the same DNN. This implementation is should be enhanced in case of 5G LAN PDU sessions. It is assumes that SMF maintains a 5GLAN DNN level context, in which all the relevant PDU sessions are correlated. When a new 5GLAN PDU session is established, SMF should look up the 5GLAN DNN context and determine whether UPF selected for existing PDU sessions can be reused by the new PDU session as well. Only if SMF determines the existing UPF cannot serve the new PDU session, it selects a new UPF following UPF selection feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When a new 5GLAN PDU session is established, SMF should look up the 5GLAN DNN context and determine whether UPF selected for existing PDU sessions can be reused by the new PDU session as well. Only if SMF determines the existing UPF cannot serve the new PDU session, it selects a new UPF following UPF selection feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPF selection implementation for 5GLAN communication is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.29.4	User Plane handling

User Plane management as defined for 5GS in clause 5.8 is applicable to 5G-LAN-type services with the following clarifications:

-	There are three types of traffic forwarding methods for 5G VN communication:

-	N6-based, where the UL/DL traffic for the 5G VN communication is forwarded to/from the DN;

-	N19-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via N19. N19 is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VN group.

-	The SMF handles the user plane paths of the 5G VN group, including:

-	Establishing N19 tunnels between PSA UPFs to support N19-based traffic forwarding.

-	For Ethernet PDU Session, the SMF may instruct the UPF(s) classify frames based on VLAN tags, and to add and remove VLAN tags, on frames received and sent on N6, as described in clause 5.6.10.2.

NOTE:	For handling VLAN tags to traffic on N6, TSP ID could also be used as described in clause 6.2.2.6 of TS 23.503 [45].

Further description on User Plane management for 5G VN groups is available in clause 5.8.2.13.

When N6-based traffic forwarding is expected, the AF can influence the traffic routing for all the members of a 5G VN group, by providing DNAI(s) together with a 5G VN external group ID, as described in clause 5.6.7.

When receiving a new PDU session establishment request for a 5G VN group, SMF may look up the 5G VN-level PDU session context to select an appropriate UPF to serve this PDU session. To avoid unnecessary N19 tunnels between UPFs, SMF may check previously selected UPFs for 5G VN communication according to 5G VN-level PDU session context, and decide whether one of previously selected UPF could serve the requested PDU session. If there is no appropriate UPF, the SMF selects a new UPF to serve the requested PDU session. It benefits the efficient traffic routing path for the communication between two or more UEs.

# * * * * End of changes * * * *