3GPP TSG-SA WG Meeting # 135	S2-1909489

# Split, Croatia, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1814 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | removing requirement that TAC+SV is used to identify UE model in manufacturer assigned ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The assumption manufacturer assigned ID is assigned per TAC+SVN is leading to a huge number of IDs. There can be massive database size savings if this assumption is lisfted, as documented in S2-19xxxxx |  |  |  |  |  |  |  |  |
| Summary of change: |  | the IDs are assigned per UE Vendor, the transition to manufacturer assigned ID is defined per TAC and not per PLMN assinged ID. The Nucmf_UECapabilityManagement_Notify service operation definition is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS would be very demanding in terms of size of database and signaling interactions required. The cost of supporting it would be high as the RACS caches in CN and RAN will have to be bigger and the signalling interactions to populate them and resolve IDs when caches misses occorr would be very frequent.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.2.18.3	Nucmf_UECapabilityManagement Service

##### 5.2.18.3.1	Nucmf_UECapabilityManagement Resolve service operation

Service Operation name: Nucmf_UECapabilityManagement_Resolve

Description: Consumer NF gets the UE radio access capability corresponding to a specific UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned).

Inputs, Required: UE Radio Capability ID.

Inputs, Optional: None.

Outputs, Required: UE radio access capability.

Outputs, Optional: None.

##### 5.2.18.3.2	Nucmf_UECapabilityManagement_Assign service operation

Service or service operation name: Nucmf_UECapabilityManagement_Assign

Description: The NF consumer sends the UE radio access capability in the UCMF and obtains a PLMN-assigned UE Radio Capability ID in return.

Inputs, Required: UE radio access capability, Type Allocation Code .

Inputs, Optional: None.

Outputs, Required: UE Radio Capability ID.

Outputs, Optional: None.

The AMF sends the UE radio access capability other than NB-IoT radio access capability to the UCMF.

##### 5.2.18.3.3	Nucmf_UECapabilityManagement_Subscribe service operation

Service operation name: Nucmf_UECapabilityManagement_Subscribe

Description: The NF consumer subscribes for updates to UCMF dictionary entries. The UCMF shall check the requested consumer is authorized to subscribe to requested updates.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.18.3.4	Nucmf_UECapabilityManagement_Unsubscribe service operation

Service operation name: Nucmf_UECapabilityManagement_Unsubscribe

Description: The NF consumer unsubscribes from updates to UCMF dictionary entries.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.18.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about changes in UCMF  to subscribed consumer NF.

Inputs, Required: Notification Type ("creation", "deletion", "Added TAC in Manufacturer Assigned list", "Removed TAC from Manufacturer Assigned list").

Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability. If Notification Type is set to "deletion": One or more UE Radio Capability IDs. If Notification Type is set to "Added TAC in Manufacturer Assigned list" or "Removed TAC from Manufacturer Assigned list", one or more TACs,

Outputs, Required: None.

Outputs, Optional: None.

End of changes