3GPP TSG-SA WG2 Meeting #135	S2-1909505

# Split, Croatia, October 14–18, 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0050 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF Selection for 5G-RG with Hybrid access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Firstly, this contribution proposes the referred architecture of Hybrid Access for 5G-RG connected via both NG-RAN and W-5GAN. The referred architecture is Figure 4.2.8.4-1 as defined in TS23.501. Then this contribution considers the UPF selection by the SMF when 5G-RG establishes a MA PDU Session via NG-RAN and W-5GAN accesses. Current mechanism enables a single PDU Session established by a 5G-RG via W-5GAN or a MA PDU Session established via W-5GAN firstly to consider the W-AGF identifiers when the SMF performing UPF selection. For a MA PDU Session via NG-RAN access firstly, it is better to take the W-AGF identifies into account to enable the selected UPF have direct connectivity/collocated with W-AGF. The advantages is: - Save the User Plane data transmission, reduce at least 1-hop. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution firstly clarifies the referred architecture of 5G-RG connected to 5GC via NG-RAN and W-5GAN. Then clarifies the UPF selection for a MA PDU Session via NG-RAN firstly use case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MA PDU Session established via NG-RAN firstly has limited chance to select a UPF which has direct connetion with W-AGF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.12.2 	Hybrid Access with Multi-Access PDU Session connectivity over NG-RAN and W-5GAN

This clause applies to the case where multi-access PDU Session connectivity via NG-RAN and W-5GAN is supported in the 5G-RG and network. The Hybrid Access architecture of 5G-RG is defined in TS23.501 [2] in Figure 4.2.8.4-1. This scenario uses the ATSSS solution described in TS 23.501 [2] , clause 5.33  with the following difference:

-	UE is replaced by 5G-RG.

-	Non-3GPP access(es) is specifically refered to wireline access.

-	In case of a MA PDU Session established by a 5G-RG via NG-RAN firstly, the SMF may also be able to take the W-AGF Identifies into account and first select a UPF which has direct connection to a W-AGF as described in TS23.501 [2], in clause 6.3.3.

**** End of First Change ****