SA WG2 Meeting #S2-135	S2-1909601

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1857 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the 5G VN broadcast solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current description sends out broadcast packets on all outgoing interfaces, while in practice packets sent to the broadcast address are flooded on Ethernet entworks along the spanning tree, which includes active interfaces but not necessarily all interfaces. Furthermore, flooding needs to be used not only for packets to the broadcast address, but also unicast packets to unknown destinations, as well as packets to multicast destinations by default. See S2-1909600 for detailed analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that flooding needs to be used rather than broadcasting on all interfaces; extends the solution to unknown unicast and multicast as well to be a general packet replication solution. Clarifies that looping back to the incoming interface is avoided by using an interface identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution as described may lead to forwarding loops that risk the stability of the network, and also to lack of packet forwarding causing hosts to be unreachable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.3, 5.8.2.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

##### 5.8.2.13.3	Support for user plane traffic replication in a 5G VN

The mechanism defined in this section allows data packets to be sent to multiple endhosts in a 5G VN. This may apply to broadcast traffic, multicast traffic or in the case of Ethernet networks unicast traffic to unknown destinations.

NOTE 1:	Broadcast traffic is used in networks for traffic that is intended to reach all endhosts. In Ethernet networks broadcast packets are flooded along the links of the spanning tree that is set up and maintained in the network. Besides the packets that are sent to the broadcast address, in Ethernet networks unicast packets that are sent to an unknown destination address are also flooded. The default behaviour for multicast packets is also flooding in Ethernet networks, even though it is possible to limit multicast groups to specific interfaces only. In IP networks, typically multicast is used rather than broadcast, but the mechanisms defined in this section are also available for IP based 5G VNs depending on operator configuration. Multicast forwarding in IP networks requires the corresponding forwarding rules for the given multicast address to be explicitly configured.

NOTE 2:	This release of the specification does not further specify how the SMF determines which interface to replicate packets for a given destination address.

When the UPF receives a packet in a 5G VN group, it may be necessary to send it out on multiple interfaces of the UPFs. This can apply to broadcast traffic, multicast traffic, and in the case of Ethernet networks unicast traffic to unkown destinations. In case of traffic that is to be flooded, the UPF shall distribute it to all other interfaces of the UPF that are considered to be active for flooding except the interface from which the packet was received. The interfaces of the UPF are:

-	The PDU Sessions of the  5G VN group members  connected to this UPF via local switch, and

-	The N19 tunnels connected to this UPF via the local switch, and

-	The interfaces to the DN on N6.

To enable  user plane replication in a 5G VN group in a UPF, the following applies:

-	For each outgoing interface, the SMF provides a PDR that detect the packets that are to be forwarded out of the packets that are sent via "internal interface". The SMF can provide such PDRs on each 5G VN group member’s N4 Session, and the SMF can provide such PDRs for each N19 or N6 interface which are considered to be part of the spanning tree on the group-level N4 Session. No such PDR is provided for interfaces that where packets are not sent out. When UPF receives  packets sent via "internal interface", it matches the packets against all PDRs installed at the "internal interface".

NOTE 3:	In Ethernet networks, the PDRs used for flooding would typically use the lowest precedence with a match-all filter in Ethernet networks. This would allow for other PDRs with higher precedence to handle unicast packets to known destinations. The remaining unicast or multicast packets that do not have a corresponding higher precedence PDR, as well as the broadcast packets, would then be handled by the PDRs with the lowest precedence having a match-all filter. However, other configurations are also possible depending on operator configuration. For IP networks, flooding would apply only under special circumstances, whereas multicast could apply to a a set of explicitly configured multicast groups.

-	Packet replication is achieved by the “carry on” indication in the PDR. A matching PDR with the “carry on” indication shall instruct the UPF to duplicate the packet and perform further processing (using associated FAR, URR, QER) on the copy instead of the original packet. The UPF continues the lookup of the other PDRs in all N4 sessions with equal or lower precedence using the original packet.

-	To avoid sending out the packet on the same interface where it was received, the PDR includes an “excluded interface” indication, where the given interface is indicated to be excluded. Packets received by the UPF

are forwarded to the UPF internal interface together with an identifier of the incoming interface. The incoming interface uniquely identifies the incoming interface at the UPF among all the N3, N9, N6, N19 interfaces. The incoming interface identifier is set by the SMF so that it is unique among all the interfaces of a given UPF, and a FAR adds the incoming interface identifier to the packet when it is sent to the internal interface.

-	For each 5G VN group member' N4 Session (i.e. N4 session corresponding to PDU Session) where packets are to be replicated, the SMF provides the following N4 rules.

-	in order to detect the traffic that is to be flooded, a PDR containing Source Interface set to "5G VN internal", Destination Address and precedence based on operator configuration, the Excluded interface information set to the interface identifier of the given PDU Session of this 5G VN group member as set by the SMF, and the Carry on indication to continue matching against other PDRs; and

-	in order to forward the traffic, a FAR containing Outer Header Creation indicating the PDU Session tunnel information, and Destination Interface set "access side".

-	The SMF configures the group-level N4 Session for processing packets received from a N19 tunnel with the following N4 rules for each N19 tunnel.

-	in order to detect all the incoming traffic on the given N19 tunnel, a PDR containing Source Interface set to "core side", and CN Tunnel Information set to N19 tunnel header (i.e., N19 GTP-U TEID); and

-	in order to forward the traffic, a FAR containing Destination Interface set to "5G VN internal", Outer Header Creation with the interface identifier of the given N19 tunnel as set by the SMF.

-	For each N19 tunnel where packets are to be replicated, the SMF provides for the group-level N4 Session the following N4 rules that enable the processing of replicated packets towards the other UPFs.

-	in order to detect the traffic, a PDR containing Source Interface set to "5G VN internal", Destination Address and precedence set based on operator configuration, the Excluded interface information set to the interface identifier of the given N19 tunnel, and the Carry on indication to continue matching against other PDRs; and

-	in order to forward the traffic on the N19 tunnel, a FAR with Outer Header Creation indicating the N19 tunnel information, Destination Interface set to "core side".

-	The SMF configures the group-level N4 Session for processing packets received from N6 with the following N4 rules.

-	in order to detect all the incoming traffic from a given N6 interface, a PDR containing Source Interface set to "core side"; and

-	in order to forward the traffic, a FAR containing Destination Interface set to "5G VN internal", Outer Header Creation with the  interface identifier of the N6 interface as set by the SMF.

-	For an N6 interface where packets are to be replicated, the SMF provides for the group-level N4 Session the following N4 rules that enable the processing of replicated packets towards N6.

-	in order to detect the traffic, a PDR containing Source Interface set to "5G VN internal", Destination Address and precedence set based on operator configuration, the Excluded interface information set to the interface identifier of the given N6 interface, and the Carry on indication to continue matching against other PDRs; and

-	in order to forward the traffic to N6, a FAR containing Destination Interface set to "core side".

* * * * Next Change * * * *

##### 5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN" , "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application ID, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application ID |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN or the local DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Packet replication and detection carry on information | Excluded interface NOTE 7 | Contains the identifier of the excluded interface. If the interface identifier information on the packet matches the excluded interface information in the PDR, , the PDR does not match. |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e., lookup of the other PDRs equal or lower precedence using the original packet, and use a copy of the packet for further processing after detection. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that the I-SMF is to locally determine CN tunnel info in order to build the N4 PDR sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in case of support for traffic replication as described in clause 5.8.2.13.3. NOTE 7: Needed to prevent UPF from sending the replicated packets back to the incoming interface. |  |  |  |

* * * * End of Change * * * *