SA WG2 Meeting #135	S2-1909720

# October 14 – 18, 2019, Split, Croatia 	(revision of S2-19yyyyy)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1881 | rev | -- | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE identifier for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Deutsche Telekom, Charter, Orange, Vodafone, NTT DOCOMO,China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current text seems to exclude support of IMSI as SUPI for UE(s) camping in SNPN. This would preclude a huge market for SNPN thus it is proposed to allow both IMSI and network specific identifier to be supported as possible identifiers for SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for IMSI as SUPI also for UE(s) camping in SNPN, correct wrong reference (28.2.2 does not exist). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Would preclude support for IMSI as SUPI for UE(s) camping in SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.2, 5.30.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

### 5.9.2	Subscription Permanent Identifier

A globally unique 5G Subscription Permanent Identifier (SUPI) shall be allocated to each subscriber in the 5G System and provisioned in the UDM/UDR. The SUPI is used only inside 3GPP system, and its privacy is specified in TS 33.501 [29].

The SUPI may contain:

-	an IMSI as defined in TS 23.003 [19], or

-	a network-specific identifier, used for SNPN.

-	a Line Id and an operator identifier of the operator administrating the Line ID value, used for supporting FN-BRGs, as further described in TS 23.316 [84], or

-	a HFC_Identifier and an operator identifier of the operator administrating the HFC_Identifier value, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [84].

A SUPI containing a network-specific identifier shall take the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19].

When UE needs to indicate its SUPI to the network (e.g. as part of the Registration procedure), the UE provides the SUPI in concealed form as defined in TS 23.003 [19].

In order to enable roaming scenarios, the SUPI shall contain the address of the home network (e.g. the MCC and MNC in the case of an IMSI based SUPI).

For interworking with the EPC, the SUPI allocated to the 3GPP UE shall always be based on an IMSI to enable the UE to present an IMSI to the EPC.

The usage of SUPI for W-5GAN is further specified TS 23.316 [84].

2nd change

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with subscriber identifier (SUPI) and credentials for each subscribed SNPN identified by the combination of PLMN ID and NID.

A subscriber of an SNPN is identified by a SUPI containing a network-specific identifier or an IMSI as defined in TS 23.003 [19] that takes the form of a Network Access Identifier (NAI). If the subscriber is identified by an IMSI, the username part of the NAI contains the IMSI. The realm part of the NAI may include the NID of the SNPN.

NOTE 1:	If the realm part includes a PLMN ID (e.g. MCC=999), then it should also include NID in order for the SUPI to be uniquely identified within the network.

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are not supported in SNPN access mode.

NOTE 2:	Voice support with emergency services in SNPN access mode is not specified in this release.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Annex D.4 provides more details.

NOTE 3:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

End of changes